DELEGATED APPLICATION REPORT 				Target Date:	07/06/2018

Application No: 180369/HOU

Address: 98 St Georges Road, Reading

Proposal: Ground and first floor rear extensions and conversion of two bedroom house into a three bedroom house.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site consists of a two storey mid-terrace property with a two storey rear gable and a single storey addition to the rear of the gable.  The surrounding area is predominantly residential.

Proposal:

The proposal involves single storey extensions to the side of the existing rear gable and to the rear of the existing gable along with a first floor extension to the rear of the existing gable.

The proposed extension to the side of the gable measures 3.8m in depth and 1.7m in width.  The proposed extension to the rear of the gable measures 3.8m in depth and 2.5m in width.  Both extensions have a flat roof and measure 2.7m in height.  The proposed extensions extend along the boundaries with both neighbouring properties but do not extend any deeper than existing extensions at 96 and 100 St Georges Road.

The proposed first floor extension has been amended in order to miss the 45 degree line from the neighbouring window at 96 St Georges Road.  The proposed first floor extension extends the width of the rear gable and measures 1.1m in depth along the boundary with 100 St Georges Road.  The extension has a splayed design in order to miss the 45 degree line from the neighbouring window at 96 St Georges Road.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 26th February 2018:

Drawing No: 02 – Existing Floor Plans

Drawing No: 03 – Existing Elevations and Sections

Drawing No: 04 – Proposed Floor Plans (FP1)

Drawing No: 05 – Proposed Elevations and Section

Design and Access Statement

The agent submitted the following plan, received on 12th April 2018:

Drawing No: 01 Rev A – Location And Block Plan

The agent submitted the following amended drawings, received on 1st June 2018:

Drawing No: 04 Rev A – Proposed Floor Plans (FP1)

Drawing No: 05 Rev A – Proposed Elevations and Section

Relevant Planning History:

180368/HOU - Single storey rear/side extensions.  Permitted 05/06/2018.

Consultation Responses:

Transport raised no objection subject to an informative.

Properties at 96 and 100 St Georges Road and 83 and 85 Shaftesbury Road were consulted.  No comments have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 30th May 2018 that the proposal as originally submitted was not acceptable as it encroached on a line taken at 45 degrees from the closest habitable window at 96 St Georges Road and as such the proposal was considered harmful to the residents of this neighbouring property in terms of loss of light and outlook.

The agent submitted amended plans on the 1st June 2018 which changed the design so that the proposal did not encroach on the 45 degree line from the neighbouring window.  However Officers felt that by overcoming the 45 degree issue created a design issue as the proposed first floor extension now had an angled design.  The agent commented that Reading Borough Council approved a similar application with the same 45 degree design issue at 38 Silverthorne Drive (application 141707) and that this application formed a precedent for this kind of design.

Officers looked at the previous application at 38 Silverthorne Drive and this property was a detached property on a much larger plot and therefore the properties were very different and a comparison could not be made between the two.  The agent was advised that the application would be refused on the amended plans.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

- To assess this, the following has been considered – would the proposal:

detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; YES.

Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document (2012) states that an extension to a house should respect the character of the house in terms of scale, location, materials and design, and respect the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line.  Policy CS7 (Design and the Public Realm) of the Core Strategy also states that all development must be of high design quality that maintains and enhances the character and appearance of the area.  The Councils SPG also states that ‘for terraced housing, where rear extensions can have a significant and detrimental impact on neighbouring properties and the general appearance of the terrace the acceptability of a rear extension, in terms of length and height, will be derived from the particular circumstances of the application site.’

The application site is a mid-terrace property and the properties within the row are characterised by two storey rear gables all of the same depth and design.  There is no objection to the ground floor extensions (which have been previously permitted under application 180368/HOU).

It is acknowledged that the proposed first floor extension is small in terms of its proposed depth however the angled design has been proposed to miss a line taken at 45 degrees from the neighbouring window at 96 St Georges Road as detailed above.  The resulting angled design is considered poor design which detracts from the property’s original character and is uncharacteristic with other the properties within the row.

The proposed development is therefore contrary to Core Strategy Policy CS7 (Design and the Public Realm), Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the amended first floor has an angled design so that it does not encroach on a line taken at 45 degrees from the middle of the closest neighbouring windows.  Avoiding a line taken at 45 degrees is a basic way to protect light reaching main rooms in neighbouring properties and to safeguard their outlook and complies with the Council’s policies and guidance.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; YES, see comments above.

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered harmful in terms of visual dominance or overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

REFUSE

Informatives relating to:

- Refused plans

- Positive and proactive