DELEGATED APPLICATION REPORT - Application No.: 180600    Target Expiry: 12/6/18 (ext)

Address: 17 Beechwood Avenue

Proposal: Single storey rear extension

Case officer: Alison Amoah

Site Description and Surrounding Area:

The property is a semi-detached house in a cul-de-sac.  The boundary to no. 11 (detached for the application site) is slightly splayed.

The adjoining property (no.18) has an existing very high single storey extension (4.3m high) 4m deep, and there is a fence and significant planting along this shared boundary.

There is a good sized rear garden, which wraps round to the side of the application house.  There is a well-established tree in the corner of the garden behind the proposed extension.

Proposal: A single storey rear extension up to the boundary with no.18 of 5.6m deep and 3.8m wide.  It would have a mono-pitched roof, 3.8m high at the boundary reducing to 2.3m, with side facing bifold doors, three rooflights and materials to match existing.

Plans considered: As received 10th April 2018:

Existing and Proposed Plan & Elevations – Drawing no: 17BA.MP01

Planning Officer note:  Due to the location of the well-established tree, in close proximity to the proposed extension, further detail was requested with regard to how the tree would be protected.

Additional plan received 25/5/18:

Tree Protection Zone – Drawing no: 17BA.SP101

Amended received 6th June 2018:

Tree Protection Zone – Drawing no: 17BA.SP101 A

Relevant Planning History:

90/00034 (900121) - Erection of single storey rear extension – Approved 15/2/90

07/00043/OUT (070294) - To erect a detached 2-bed dwelling following demolition of existing garage – Refused 21/3/07

07/00860/OUT (071122) - Outline application to erect a detached 2 bedroom dwelling following demolition of existing garage (layout, scale and means of access only) (re-submission of planning application 07/00043/OUT) – Refused 11/09/07

Consultation Responses: No: 18 Beechwood Avenue and Nos: 38 & 39 Wendover Way were consulted. No responses were received.  A site notice was displayed.

Natural Environment: These details [Tree Protection Plan] indicate (using the scale bar) that the ground beams (spanning between piles) will be 0.75-1m below ground level.  This will effectively be a trench between the piles of that depth.  In addition to this excavation, there is a concrete slab covering the whole extension area below ground level, meaning excavation over the whole area, which appears to be approx. 100mm deep.  This would add up to a lot of excavation within the portion of RPA that overlaps the footprint.  Excavation is only normally accepted within RPAs if confined to pile positions (which should be hand dug to 600mm, cleanly cutting any roots found) and minimal depth for beams (which should ideally be above ground but due to matching levels normally end up partly in the ground).  However, it would appear when comparing your photos to the plan that the extension is outside the canopy area which means it must be within the existing patio area.  This area will therefore have already been affected by the laying of hard surfacing.  This, together with the fact that the tree has evidently been pruned to contain its size and that its roots have the reminder of the garden, which is likely to be a more favourable rooting environment than under the patio, the proposal is acceptable.

The plan should be referred to in the approved documents list and conditions added [as included in the recommendation below]

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (2012)

DM9: House Extensions and Ancillary Accommodation;

DM4: Safeguarding Amenity

Supplementary Planning Guidance:  A Design Guide to House Extensions (2003)

Appraisal Checklist:

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- Detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building;  No

- Reduce the amount of usable amenity space for the property to an unacceptable degree;   No

- Require the loss of trees in the garden or put significant trees at risk; No – A tree protection plan was submitted, which includes implementing mini-piling with shallow ground beams, clay board and a suspended ground floor slab with tree protection measures for the full duration of the works.  The Natural Environment Officer has confirmed the approach is acceptable subject to conditions, as included within the recommendation below.

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- Significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; No

- Invade the privacy of neighbouring properties through overlooking from windows or balconies; No

- Detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; No

- Detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; No

DM4 states: Development will not cause a significant detrimental impact to the living environment of existing or new residential properties, in terms of:

 Privacy and overlooking; NO

 Access to sunlight and daylight; NO

 Visual dominance and overbearing effects of a development; NO

 Noise and disturbance; NO

 Artificial lighting; NO

 Vibration; NO

 Dust and fumes; NO

 Smell; or NO

 Crime and safety. NO

RECOMMENDATION

APPROVE subject to conditions and informatives.

Proposed Conditions:

- Standard Time limit TL1

- The development hereby permitted shall be carried out in accordance with the following approved plans:

Existing and Proposed Plan & Elevations – Drawing no: 17BA.MP01, received 10th April 2018

Tree Protection Zone – Drawing no: 17BA.SP101 A, received 6th June 2018

- Tree protection, as shown on Proposed Tree Protection Zone drawing 17BA.SP101 A shall be erected prior to commencement and retained until completion

Reason: to ensure that appropriate protection is given to trees of amenity value within and adjacent to the site in accordance with Policy CS38.

- During all excavations works on site, any roots discovered shall be cleanly cut with sharp tools and an impermeable membrane put in place where concrete is required adjacent to such roots.

Reason: In order to protect trees intended to be retained on site, in the interests of the visual amenities of the locality in accordance with Policies CS38, Policy CS7 & Policy DM18.

- Matching materials

Informatives:

Terms and conditions

Building regulations

Encroachment

Complaints re noise during construction

Positive and proactive