DELEGATED APPLICATION REPORT 				Target Date:	25/06/2018

Application No: 180716/HOU

Address: 119 Berkeley Avenue, Reading

Proposal: Demolition of existing outbuildings to side elevation and new single and two storey side extension to existing dwelling house

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site consists of a two storey semi-detached property with single storey side and rear extensions.  The property is close to the junction with Berkeley Avenue and Bath Road and adjacent to a railway line and an electricity sub-station.  The surrounding area is predominantly residential.

Proposal:

The proposal involves the demolition of the single storey side extensions and the construction of a two storey and single storey side/rear extension.  The proposed two storey side extension measures 3.4m in width and is set back from the existing front elevation of the property by 0.6m although it does have a bay window that extends up to the existing front elevation of the property.  The proposed two storey extension extends 5.3m in depth and is set down from the existing roof by 0.9m.  The hipped roof design matches the existing property.

The proposed single storey extension is to the rear of the proposed two storey extension and attaches to the existing single storey rear extension.  The proposed single storey rear extension has a flat roof and is the same depth and height as the existing rear extension (3.5m in depth and 2.6m in height).

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 28th April 2018:

Drawing No: 1164/1 – Existing Ground Floor Plan

Drawing No: 1164/2 – Existing First Floor Plan

Drawing No: 1164/3 – Existing Front Elevation

Drawing No: 1164/4 – Existing Side and Rear Elevations

Drawing No: 1164/5 – Proposed Ground Floor Plan

Drawing No: 1164/6 – Proposed First Floor Plan

Drawing No: 1164/7 – Proposed Front Elevation

Drawing No: 1164/8 – Proposed Side Elevation

Drawing No: 1164/9 – Proposed Rear Elevation

Drawing No: 1164/10 – Site Plan

Drawing No: 1164/11 – Location Plan

Relevant Planning History:

89/00065/FUL (Civica Ref: 891072) – Erection of two storey side and single storey rear extension.  Permitted 24/02/1989.

Consultation Responses:

Transport advised the demolition of the garage will result in the loss of a parking space, however the proposed block plan illustrates off road parking for 2 vehicles, which complies with the Councils current standards. Plans indicate that the existing opening is to be widened to facilitate the proposed parking layout.  In principle this is acceptable however it is not clear if the dropped kerb is also to be widened.  In principle this would be acceptable as long as it was widened away from the junction.

Transport requested amended plans however during the Officers site visit the applicant did advise they would be extending the dropped kerb at a later date and was aware that they needed consent from the Council’s Highway section.  An informative is considered sufficient in this instance.

Ecology raised no objection.

Natural Environment Trees raised no objection.

Property at 117 Berkeley Avenue was consulted.  No comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

CS38 (Trees, Hedges and Woodlands)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal meets general policy and guidance.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, due to the property’s corner location adjacent to an electricity substation the proposal is not harmful in terms of visual dominance or overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

Informatives relating to:

- Positive and proactive

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment

6.	Highways