DELEGATED APPLICATION REPORT 				Target Date:	15/05/2018

Application No: 180479/HOU

Address: 3 Fontwell Drive, Reading

Proposal: Single storey front extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey detached property with a part cat slide roof with dormer window to the front along with a staggered front elevation.  The property has been extended in the past and has a two storey side extension.  The surrounding area is predominantly residential of similar style properties.

Proposal:

The proposal involves a single storey extension to the front of the existing two storey side extension.  The extension would be set back from the front elevation of the property by a maximum of 1.7m and measures 2.2m in depth.  The proposal has a mono pitch roof and measures 2.5m in height to eaves level and 4m at its highest point.

The proposal would be 1.5m from the boundary with the neighbouring property at 5 Fontwell Drive and would not encroach on a line taken at 45 degrees from the middle of the closest front window at this neighbouring property.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 15th March 2018:

Location Plan

Existing Elevations

Proposed Elevations

The agent submitted the following drawings, received on 20th March 2018:

As Existing Plan

Site Plan

Front Extension Option

Relevant Planning History:

96/00048/FUL (Civica Ref: 960179) - Single storey rear extension plus new bedroom over existing garage to side.  Permitted 26/02/1996.

Consultation Responses:

Transport raised no objection subject to informatives and a condition requiring the parking to be provided in accordance with the approved plans.  The submitted plans show 3 parking spaces and the removal of part of the front lawn.  As two parking spaces meet the Council’s requirements it is not considered necessary to include this condition as it would be preferable to retain the front lawn.

Natural Environment Trees advised there is a significant protected Lime at the rear of the property, however as a fairly small, single storey extension at the front of the property there would be no obvious risk to the tree from this proposal if permitted.  In view of this, no tree and landscape conditions are considered necessary subject to an informative.

Properties at 1, 5 and 6 Fontwell Drive, 12 Lingholm Close and 42 Hogarth Avenue were consulted and no comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS38 (Trees, Hedges and Woodlands)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered harmful in terms of visual dominance or overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

Informatives relating to:

- Positive and proactive

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment

6.	Lime tree at rear

7.	Highways

8.	Do not damage the verge