DELEGATED APPLICATION REPORT

Target Date:	27/07/2018				Extended Target Date: 26/10/2018

Application No: 180920/FUL

Address: 1 - 3 Osborne Road, Reading

Proposal: Two storey side and rear extensions to 1 and 3 Osborne Road

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site relates to 1 and 3 Osborne Road which are two storey semi-detached properties.  Both have detached garages to the rear and relatively large rear gardens.  Both properties have hardstanding parking areas on the front. The properties are at a higher level than the neighbouring properties on Craig Avenue.  There is a development of flats to the rear.  The surrounding area is predominantly residential.

Proposal:

The proposal is for a two storey side and rear extension to 1 and 3 Osborne Road.  The proposed side extensions measure 0.9m in width and extend the full depth of both properties.  The proposed two storey rear extensions extend to the rear of the side extensions and extend 4.5m in depth.  To reduce the bulk of the proposal at roof level the roof design was amended so that the roof to the proposed side extensions becomes a continuation of the main roofs.  The roof to the proposed rear extensions are hipped and set down from the main roof by 0.5m.

The neighbouring property at 63 Craig Avenue is adjacent to 1 Osborne Road and is a bungalow.  This property is at a lower level than 1 Osborne Road and occupies an angled corner position.  There is a distance of approximately 12m between the side elevation of 1 Osborne Road and the rear of 63 Craig Avenue.

The neighbouring property at 5 Osborne Road is similar in design to 1 and 3 Osborne Road but is at a higher level.  This property has two first floor side windows, a larger window to the landing and a smaller (secondary) window to the bathroom.  A gap of 1m will remain between the proposed side extension and the boundary with this neighbouring property.

The materials consist of brick and render and tiles to match the existing properties.

Plans Considered:

The agent submitted the following drawings, received on 1st June 2018:

Location Plan

Drawing No: PL-01 – Proposed Block Plan

Drawing No: PL02 – Existing Ground Plans

Drawing No: PL03 – Existing First Floor

Drawing No: PL04 – Existing Elevations

Drawing No: PL05 – Proposed Ground

Drawing No: PL06 – Proposed First Floor

Drawing No: PL07 – Proposed Elevations

The agent submitted the following amended drawings, received on 9th August 2018:

Drawing No: PL06 Rev 1 – Proposed First Floor

Drawing No: PL07 Rev 1 – Proposed Elevations

Relevant Planning History:

180475/FUL - Two storey side and rear extensions to 1 and 3 Osborne Road.  Withdrawn 14/05/2018.

Consultation Responses:

Transport raised no objections subject to conditions and informatives.

Ecology raised no objection.

Properties at 2, 4, 5 and 6 Osborne Road, 63 Craig Avenue and 63-86 (all) Rowe Close.  One letter of support has been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 13th July 2017 that in principle there was no objection to the proposal however it was felt that the proposed side extensions should follow the guidelines for two storey side extensions in the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions and should be set down from the main roof and set back from the existing front elevation.  The agent argued the case for not setting the two storey side extensions back from the existing front elevation of the properties but did amend the roof design to reduce the bulk at roof level. Although the proposed side extensions are only marginally set back this was considered acceptable due to their narrow width.

The applicants have agreed to enter into a S106 Legal Agreement to ensure that the development is built at the same time.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal meets general policy and guidance and although only marginally set back from the existing front elevations of the property this was considered acceptable due to their narrow width.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the proposed rear extension at 3 Osborne Road does not encroach on a line taken at 45 degrees from the middle of the closest habitable window at 5 Osborne Road which protects light reaching these mains rooms.  There is also sufficient distance between 63 Craig Avenue and 1 Osborne Road not to significantly reduce the amount of light to this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, there are no first floor side windows proposed and a condition will be imposed to restrict first floor side windows to both properties.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO, as the proposal is to 1 and 3 Osborne Road the properties will be identical in appearance.

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the neighbouring properties will notice the additional height and depth of the proposal they are not considered to impact neighbouring properties in terms of visual dominance and overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

- No first floor side windows

- Vehicle parking in accordance with approved plans

- Vehicle access in accordance with approved plans

- Notification of development commencing

- Notification of development completed

Informatives relating to:

- S106 Legal Agreement

- Terms and conditions

- Building regulations

- Construction and demolition

- Encroachment and Party Wall

- Highways

- Access construction

- Damage to verge

- Positive and proactive