DELEGATED APPLICATION REPORT			                         Target Date: 08/10/18

Application No: 181380/FUL

Address: 6a Hawthorn Gardens

Proposal: Demolition of bungalow and erection of 2 storey 4 bed dwelling

Case Officer: Tom Hughes

Site Description and Surrounding Area

The application site is a detached timber framed Colt bungalow located at the end of a cul-de-sac. The bungalow benefits from a conservatory and a detached garage. Landscaped areas surround the bungalow, with a driveway forward of the garage. The surrounding area is predominantly residential, comprised of dwellings of various styles and design.

Proposal

The proposal is for the demolition of the existing bungalow and the erection of a two storey, four bedroom dwelling. The footprint and ridge height of the existing bungalow would be maintained, with the dwelling oriented in the same direction. The proposed dwelling would extend at a width of 13.2m, projecting to a depth of 7.6m. A centrally located porch would project forward of the principal elevation to a depth of 1.2m, extending at a width of 2.1m. The dwelling would have a pitched roof with a maximum height of 4.9m and a height to eaves of 2.2m. A flat-roof dormer would be located to the southward roof slope, extending at a width of 11.5m at a height of 2.2m, projecting to a depth of 3.4m. The dormer would be set down from the ridgeline by 0.1m. Three roof lights would be located within the northward roof slope. The porch would have a pitched roof with a maximum height of 3m and a height to eaves of 2m.

To the front elevation two windows would be located at the ground floor. The porch would contain the main entrance door and glazing to the front and side elevations. A further door would be located to the eastward side elevation. A ground floor window would be located to the westward side elevation. To the rear elevation three windows and a set of bi-fold doors would be located to the ground floor, with two further windows to the dormer.

The dwelling would have rendered external walls and a slate roof. The dormer would be clad in hanging tiles. Fenestration would be double glazed uPVC units.

The existing garage would be retained, with off-road parking continuing to be located to the existing driveway.

Plans Considered

The agent supplied the following, received 07/08/18:

Design and Access Statement

Drawing No: A3-05 – Site Location and Site Plan

Drawing No: 1 of 1 – Site Survey

Drawing No: A1-01 – Existing Ground Floor Plan, Elevations, Section and 3D Visuals

Drawing No: A1-06 – Proposed Floor Plans, Elevations, Sections and 3D Visuals

Subsequently, the agent supplied the following, received on 17/09/18:

Document ref: CE1171-01 – Preliminary bat roost assessment, dated 11/09/18, by Chase Ecology Limited

Subsequently, the agent supplied the following, received on 27/09/18:

Document ref: CE1171-01 – Preliminary bat roost assessment and Emergence survey, dated 23/09/18, by Chase Ecology Limited

Subsequently, the agent supplied the following, received on 15/10/18:

Drawing No: A3-05 Rev. A – Site Location, Site Plan and Proposed Parking Layout

Relevant Planning History

None.

Consultation Responses

Neighbouring owners and occupiers at 5, 5a and 6 Hawthorn Gardens and 134, 136, 138 and 140 Shinfield Road were consulted by letter. A site notice was displayed. Five letters of representation were received with regard to the following:

Five letters of representation were received with regard to the following:

- The construction process and contractor parking. The accessibility of the site for deliveries Officer note: full details to be secured by condition

- The use of the dwelling as an HMO Officer note: the change of use from a Class C3 dwelling house to a Class C4 HMO does not require planning permission. The use of the dwelling as an HMO will be appraised below

- Parking has not been illustrated Officer note: this has been confirmed by way of amended plan

- There is a right of access through to 136 Shinfield Road and concern that this access would be blocked Officer note: not a material planning consideration. It is noted that there is no change proposed to the existing access

- Existing drainage issues would be made worse Officer note: not a material planning consideration. The development would be subject to compliance with the relevant building regulations

- Concern that the existing garage contains asbestos Officer note: not a material planning consideration. The existing garage is to be retained

- The control of noise and dust during construction and demolition Officer note: details to be secured by condition

- The dwelling would overshadow and overlook 138 Shinfield Road and would result in a loss of light and a loss of privacy to this dwelling Officer note: the proposed dwelling would sit on the same footprint as the existing, and the existing ridge height would not be exceeded. Neighbouring dwellings are a significant distance from the proposed development, which is not considered to cause any overshadowing or loss of light to neighbouring dwellings. The proposed dormer window would be located within the roof slope of the dwelling, at a height lower than existing first floor windows of the adjacent dwellings. Any overlooking that would occur would be only to the rear of gardens of dwellings on Shinfield Road, and not to any significant degree. A distance of approximately 25m would be maintained to the rear of the dwellings on Shinfield Road (at a 45 degree angle), which is in accordance with Policy DM4 of the Sites and Detailed Policies Document

- A two storey dwelling is not in keeping with the area Officer note: the adjacent dwellings are of two storeys

- The demolition process would raise asbestos related issues Officer note: not a material planning consideration. The development would be subject to compliance with the relevant building regulations

- Concern for the impact of the proposal on trees close to the boundary Officer note: details to be secured by condition

- Overlooking would occur to 140 Shinfield Road by the dormer Officer note: as above

Natural Environment advised that trees just off-site to the southeast corner may be affected by the proposed development. A plan illustrating the proposed development in the context of the surrounding area, with trees plotted including their trunks, canopies and Root Protection Areas was required.

Transport Development Control advised that the site is located in Zone 3, Secondary Core Area of the Council’s adopted Parking Standards and Design SPD. In accordance with this Standard, the proposed dwelling would be required to provide a minimum of two off-road parking spaces. A plan illustrating the proposed parking layout was required. Transport also required that details of cycle and bin storage be confirmed.

Environmental Protection raised concerns regarding the potential for nuisance associated with the construction and demolition phases, and the possible adverse impact on nearby residents. Accordingly, conditions controlling noise and dust, hours of working and bonfires were recommended.

Ecology advised that the site is surrounded by habitat of good suitability for use by foraging and commuting bats, including large connected gardens with trees, broadleaved woodland 60m northeast and 205m southwest, and grassland fields 125m east of the site. Additionally, it was advised that there are records of bats roosting nearby. Due to the site location and extent of the works, there is a risk that the proposal would affect roosting bats, so a bat survey would need to be provided prior to the determination of the application. This information was relayed to the agent, who subsequently provided a bat survey report. Ecology confirmed that the bat survey report, conducted by Chase Ecology and Conservation Consultants in September 2018, details the results of a preliminary roost assessment and one dusk emergence survey. Ecology advised that the survey report is not strictly in accordance with the Bat Conservation Trust Bat Survey Guidelines which states where a building has low potential of roosting bats a dusk or dawn survey should be undertaken in the optimum period of May to August (inclusive), however it was advised that the findings of the report are likely to be accurate. The report states that three bat droppings were observed close to a gap within the eaves on the north elevation and as such, a dusk emergence survey was required. The report states that no bats emerged the property during the dusk survey and concludes that it is unlikely that the proposal will affect bats. As such, since the works are unlikely to affect bats or protected species, there are no objections to the application on ecological grounds.

Equality Act 2010

In determining this application, the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework

Reading Borough Local Development Framework Core Strategy (2008)

Policy CS1 – Sustainable Construction and Design

Policy CS2 – Waste Minimisation

Policy CS4 – Accessibility and Intensity of Development

Policy CS7 – Design and the Public Realm

Policy CS9 – Infrastructure, Services, Resources and Amenities

Policy CS14 – Provision of Housing

Policy CS15 – Location, Accessibility, Density and Housing Mix

Policy CS24 – Car/Cycle Parking

Policy CS34 – Pollution and Water Resources

Policy CS36 – Biodiversity and Geology

Policy CS38 – Trees, Hedges and Woodlands

Sites and Detailed Policies Document (2012)

Policy SD1 – Presumption in Favour of Sustainable Development

Policy DM1 – Adaption to Climate Change

Policy DM3 – Infrastructure Planning

Policy DM4 – Safeguarding Amenity

Policy DM6 – Affordable Housing

Policy DM10 – Private and Communal Outdoor Space

Policy DM11 – Development of Private Residential Gardens

Policy DM12 – Access, Traffic and Highway-Related Matters

Policy DM17 – Green Network

Policy DM18 – Tree Planting

Supplementary Planning Document – Revised Parking Standards and Design (2011)

Appraisal

The main issues to be considered are:

- Principle of development

- Design and impact on the character of the surrounding area

- Impact on neighbours

- Amenity of future occupiers

- Transport

- Natural Environment

- Environmental matters

- Affordable housing and CIL

a) Principle of development

The principle of the development is acceptable as it replaces an existing dwelling in a poor state of repair with a new dwelling suitable for modern occupation. The provision of housing is in accordance with Policy CS14 of the Core Strategy. The development is acceptable, subject to other policy considerations as set out below.

b) Design and impact on the character of the surrounding area

Policy CS7 of the Core Strategy states that all development must be of a high design quality that maintains and enhances the character and appearance of the area. Policy DM11 of the Sites and Detailed Policies Document requires that residential development makes a positive contribution to the character of the area in respect of the layout and spacing of the development; the form, height and massing of buildings; materials and appearance; landscaping and boundary treatments. The application site is also required to be of an adequate size and dimension to accommodate the development proposed in terms of its setting and spacing around buildings, amenity space, landscaping and space for access and roads.

The proposed development would be facilitated by the demolition of the existing dwelling in its entirety. The resultant dwelling would maintain the existing footprint, enlarging it only by a minimal degree, and oriented in the same direction. The existing dwelling sits comfortably within its plot, with outdoor amenity space to all sides. The extent of the proposed built form is therefore acceptable, relative to the site area.

The existing Colt bungalow is of a distinctive style, dissimilar to the general norm of dwellings in the surrounding area. Despite this, the existing dwelling does not detract from the character and appearance of the area. The proposed dwelling reflects the general form of the existing, with the footprint, orientation and ridge height to be maintained. The consistency of design is noted. Materials to be used on the external faces of the development and the positioning of fenestration have been assessed and are deemed to be acceptable.

Though the use of the proposed dwelling as a House in Multiple Occupation would result in a net loss of accommodation suitable for family occupation, the proposal is not considered to unacceptably dilute the prevailing residential character of the area.

The proposed development represents an upgrade of the existing dwelling, suitable for modern occupation and improving the general appearance of the site. The spacing around the building and the provision of outdoor amenity space is consistent with the existing built form and spaces that characterise the surrounding area. As such, the proposal is in accordance with Policy CS7 of the Core Strategy and Policy DM11 of the Sites and Detailed Polices Document.

c) Impact on neighbours

The scale and positioning of the proposed dwelling generally reflects the existing building, which does not significantly impact on the residential amenities of neighbouring owners and occupiers. The primary difference between the existing and proposed dwellings in terms of impact on residential amenity is the provision of a southward facing box dormer. Two windows would be located to the southward elevation of the dormer, with windows serving first floor bedrooms. Due to the ridge height of the existing bungalow not being exceeded, the relative height of these first floor windows is comfortably lower than the respective first floor windows of the adjacent dwellings. It is not considered that any significant degree of overlooking would occur from these dormer windows, which face towards the end of lengthy rear gardens of dwellings fronting Shinfield Road. Indeed, a distance of approximately 30m (at an angle of 45 degrees) would be maintained from the dormer windows to the rear elevation of 140 Shinfield Road. Policy DM4 of the Sites and Detailed Policies Document requires that a back-to-back distance of 20m between dwellings is usually appropriate to avoid a detrimental effect on privacy.

As noted above, the proposed dwelling would be set comfortably within its plot, with spacious gaps maintained to each of the site boundaries. The height and bulk of the proposed dwelling is not considered to have a detrimental overbearing or visually dominant effect on neighbouring dwellings. As such, the proposal is in accordance with Policy DM4 of the Sites and Detailed Policies Document.

d) Amenity of future occupiers

The proposed layout of internal rooms is considered to be generally acceptable, with appropriate room sizing and heights maintained throughout the dwelling. Noise and disturbance as a result of inappropriate stacking of rooms is not considered to be an issue. The proposal is therefore in accordance with Policy DM4 of the Sites and Detailed Policies Document.

Policy DM10 of the Sites and Detailed Polices Document requires that dwellings be provided with functional, private outdoor space. The existing outdoor space would be retained, which is considered acceptable, and it would not be compromised by the relationship with other surrounding buildings. The proposal is therefore in accordance with Policy DM10 of the Sites and Detailed Policies Document.

e) Transport

As per the consultation response from Transport Development Control, the proposed development would be required to provide off-road parking provision for two vehicles. The existing dwelling benefits from a driveway and a garage. The agent has confirmed that these would be retained, illustrating parking provision for a minimum of two vehicles. This provision therefore accords with the Council’s adopted standard which is acceptable.

Concerns have been raised regarding the blocking of a right of access to 136 Shinfield Road and the impact of the parking of contractor vehicles during the construction and demolition phases on the accessibility of Hawthorn Gardens for neighbouring residents. The right of access to 136 Shinfield Road is a civil matter and does not constitute a material planning consideration. It was noted during the officer’s site visit that the existing garage and boundary treatment already restrict any access. The proposal does not seek to alter the existing garage or boundary treatment. The control of contractor vehicles and the need to ensure that the cul-de-sac remains accessible during the course of the construction and demolition phases is noted. Full details of where the vehicles of site operatives and visitors will be parked and managed, and the location on site for the storage of plant and materials will be secured by condition. This is in the interest of protecting the amenity of neighbouring residents, the character of the area and highway safety, in accordance with Policy DM12 of the Sites and Detailed Policies Document.

f) Natural Environment

As per the consultation response from Natural Environment, there are concerns that the impact of the construction and demolition phases would be detrimental to trees adjacent to the site. In particular, the tree to the southeast corner of the site to the boundary with 138 Shinfield Road is considered to be of particular amenity value. This tree should be protected in accordance with the relevant recommendations of appropriate British Standards. Full details will be secured by condition. Following the submission of bat survey reports, there is no objection to the development on ecological grounds.

g) Environmental matters

As per the consultation response from Environmental Protection, there are concerns that neighbouring residents would suffer as a result of the construction and demolition phase. Accordingly, prior to the commencement of development a Construction Method Statement is required to be submitted to and approved in writing for the control of noise and dust emanating from the site during the demolition and construction phases. Additionally, hours of working and the burning of waste on site would be restricted by condition.

h) Affordable housing and CIL

The Council’s affordable housing policies are crucial in seeking to assist the Council in meeting the requirements of the NPPF. The provision of affordable housing is a major issue in the borough. The borough experiences high levels of need as evidenced by a recent Strategic Housing Market Assessment and other indicators. Residents face major issues relating to affordability as a result of high and increasing house prices compared to average local incomes, and there are significant issues in the future supply of affordable housing to meet identified needs.

Recent appeal decisions in Reading Borough have found in favour of the Council’s approach, whereby the Planning Inspector has agreed with the Council’s position that the ‘local circumstances in Reading justify the need for small sites to make contributions to affordable housing as an exception to national policy’. These include:

- Appeal Ref: APP/E0345/W/16/3162427 – 171 Blagdon Road

- Appeal Ref: APP/E0345/W/16/3153661 – 17 St Barnabas Road

- Appeal Ref: APP/E0345/W/16/3159962 – 8 Thornton Road

- Appeal Ref: APP/X0360/W/16/3154971 – 51 Cressingham Road

In the recent appeal decision dated 13/03/17 (Appeal Ref: APP/E0345/W/16/3162427 – 171 Blagdon Road), the Inspector concluded that ‘…given the circumstances of this proposal and the acute and substantial need for affordable housing in the Borough, and the significance of small sites in achieving the aim of Policy DM6 of the Sites and Detailed Policies Document, outweigh the development plan. Accordingly, the need for the contribution sought by the Council arises from the proposal and satisfies the tests in Regulation 122(2) of the Community Infrastructure Regulations 2010 and paragraph 204 of the Framework… My findings are consistent with the most recent decisions by Inspectors in like cases, in which light the determination of this case in a like manner is a material consideration to which I attach substantial weight. For the reasons above, I conclude that the proposed development should provide a financial contribution to affordable housing, the absence of which would place the proposal in direct conflict with Policy DM6 of the Sites and Detailed Policies Document and the SPD’.

Reading Borough Council has considered its options for implementing its affordable housing policy in light of these decisions. At its meeting of the Strategic Environment Planning and Transport Committee on 13th July 2016, the Council discussed he outcome of the Court of Appeal’s decision on its challenge (the report can be found here: http://www.reading.gov.uk/media/5651/Item09-SEPT-C-Report-on-C-of-Appeal-judgement-05-16/pdf/Item09_SEPT_C_Report_on_C_of_Appeal_judgement_05_16.pdf ).

Having considered its options, the Committee agreed the following as the basis for determining planning applications where Policy DM6 of the SDPD is relevant:

To implement Policy DM6 as currently adopted in the SDPD but excluding proposals that solely involve the conversion of an existing property, where the conversion involves the provision of 10 or less dwelling units (i.e. not HMOs), or the replacement of dwellings by the same number of replacement dwellings where there is no net increase.

On this basis, the proposed development would not require a provision for the delivery of affordable housing, in compliance with Policy DM6 of the Sites and Detailed Policies Document and the Affordable Housing Supplementary Planning Document.

The proposal would be a Community Infrastructure Levy liable development. The applicant has provided the appropriate CIL Additional Information Form. An informative will be attached to the decision notice to advise the applicant of their responsibilities in this respect.

Conclusion

The proposed development has been assessed and found to be acceptable as it accords with the identified policy and guidance. It is recommended that planning permission be granted for the proposed development, subject to conditions.

RECOMMENDATION

GRANT