### Delegated Application Report				Target Date: 07/02/2018

Address:  17 Hemdean Road

Application No: 172231/CLP

Proposal: Two storey rear extension and front porch

Case Officer: Heather Banks

## Introduction & Proposal

The application site is a two storey semi-detached property on the east side of Hemdean Road. The property is set back from the road, with a modest front garden, and has a two storey rear projection which is original to the property. The property and its adjoined neighbour also have two storey front bay windows and recessed arched front door/entrance areas, which are also found on the neighbouring terraced properties.

The applicant proposes a two storey, full width rear extension, projecting 1.35m from the rear of the main house and 3m from the rear of the original two storey rear projection. The extension is to be roofed by two pitched roof gables (one the width of the existing two storey rear projection). The ridge heights of the gables will be 0.9m below the ridgeline of the main house, and the eaves will be the same level as the existing eaves. Bi-fold doors are proposed across the width of the rear of the extension at ground floor level, and at first floor level the more northerly of the two gables will be glazed with full height windows and a Juliet balcony whilst the other gable is to be fitted with a much smaller bathroom window. No information has been provided with regards to the materials to be used in the proposed extension.

The applicant also proposes to construct a front porch. The porch is to project 1.5m from the front of the existing property and is to have a width of 2m. The porch is to have a pitched roof with a maximum height of 3m and an eaves height of 2.4m. Again no information has been provided with regards to the proposed materials.

Plans Submitted:

The agent supplied the following drawings:

Drawing No: EHP/17/1 – Proposed and Existing Plans and Elevations

Received 13th December 2017

Relevant Planning History:

180031/HOU - Proposed front entrance porch and proposed two storey rear extension. Under consideration

Summary of Consultations:

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy & Guidance:

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions of Classes A, B and D of Part 1, of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relate to ‘the enlargement, improvement or other alteration of a dwelling house’, ‘the enlargement of a dwellinghouse consisting of an addition or alteration to its roof’ and ‘the erection or construction of a porch outside any external door of a dwellinghouse’.

Development is not permitted by Class A if:

- A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); COMPLIES

- A.1 (c) the height of the dwellinghouse enlarged, improved or altered would exceed the highest part of the roof of the existing dwellinghouse; COMPLIES.

- A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; COMPLIES

- A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principal elevation of the original dwellinghouse; (ii) fronts a highway and forms a side elevation of the original dwellinghouse; COMPLIES

- A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres in the case of a semi-detached house, or (ii) exceed 4 metres in height; NOT APPLICABLE IN THIS INSTANCE

- A.1 (g) - NOT APPLICABLE IN THIS INSTANCE.

- A.1 (h) - The enlarged part of the dwellinghouse would have more than a single storey and—

(i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres, COMPLIES

(ii) be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse; COMPLIES

- A.1 (i) the enlarged part of the dwellinghouse would be within 2m of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3m; DOES NOT COMPLY. The eaves of the proposed two storey extension would run along the boundary with the neighbouring property 15 Hemdean Road and would be 5.2m from ground level

- A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would (i) exceed 4m in height, (ii) have more than one storey, or (iii) have a width greater than half the width of the original dwellinghouse; DOES NOT COMPLY. The original property has a two storey projection to the rear of the main building. The proposed two storey extension will wrap around this two storey projection of the original house. The side elevations of the original two storey rear element are considered to be side elevations of the original house. As such the proposed extension will extend beyond a wall forming a side elevation of the original dwelling house and will exceed 4m in height (max. height 6.3m), have more than one storey and have a width greater than half the width of the original dwelling house (it would run the full width of the property). As such the proposal does not comply with the stipulations of this paragraph.

- A.1 (k) it would consist of or include – (i) the construction or provision of a veranda, balcony or raised platform, (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse.   COMPLIES

Development is permitted by Class A subject to the following relevant conditions:

- A.3 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; INSUFFICIENT INFORMATION SUBMITTED. No information has been submitted with regards to the materials proposed to be used in the external finishes of the extension. As such, it cannot be confirmed that the materials used in the exterior works will be similar to those of the existing dwellinghouse.

- A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse shall be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed; NOT APPLICABLE IN THIS INSTANCE and

- A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, the roof pitch of the enlarged part shall, so far as practicable, be the same as the roof pitch of the original dwellinghouse. COMPLIES

Development is not permitted by Class B if:

- B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; COMPLIES

- B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; COMPLIES

- B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than—

(ii) 50 cubic metres; COMPLIES. Resulting roof space is approx. 11m3.

- B.1 (e) it would consist of or include—

(i) the construction or provision of a verandah, balcony or raised platform, or

(ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; or COMPLIES

- B.1 (f) the dwellinghouse is on article 2(3) land. COMPLIES

Development is permitted by Class B subject to the following relevant conditions:

- B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse;

INSUFFICIENT INFORMATION SUBMITTED. No information has been submitted with regards to the materials proposed to be used in the external finishes of the extension.

- B.2 (b) the enlargement must be constructed so that—

(i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension—

- (aa) the eaves of the original roof are maintained or reinstated; and

- (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; NOT APPLICABLE IN THIS INSTANCE

(ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; NOT APPLICABLE IN THIS INSTANCE

- B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the

dwellinghouse must be—

(i) obscure-glazed, and

(ii) non-opening unless the parts of the window which can be opened are more than 1.7metres above the floor of the room in which the window is installed. NOT APPLICABLE IN THIS INSTANCE

Development is not permitted by Class D if:

- D.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- D.1 (b) the ground area (measured externally) of the structure would exceed 3 square metres; COMPLIES

- D.1 (c) any part of the structure would be more than 3 metres above ground level; or COMPLIES

- D.1 (d) any part of the structure would be within 2 metres of any boundary of the curtilage of the dwellinghouse with a highway. COMPLIES

Conclusion

A Certificate of Lawfulness for the proposed development is refused for two reasons. Firstly because the proposed extension would run along the boundary with the neighbouring property 15 Hemdean Road and would be 5.2m from ground level, and therefore would not comply with paragraph A.1(i) of Class A. Secondly the proposed two storey extension will wrap around the two storey projection of the original house. As such, the proposed extension will extend beyond a wall forming a side elevation of the original dwelling house, will exceed 4m in height (max. height 6.3m), have more than one storey and have a width greater than half the width of the original dwelling house (it would run the full width of the property). As such the proposal does not comply with the stipulations of this paragraph A.1(j) of Class A. The extension therefore fails to meet the Permitted Development criteria as stated Under Class A.1(i) and A.1(j) of Part 1, Schedule 2, of The Town and Country Planning (General Permitted Development)(England) Order 2015 (as amended).

Recommendation:

REFUSE Certificate

Conditions

As conclusion.

Informatives

The refusal relates only to the following plans and drawings as received by the local planning authority on 13th December 2017:

Drawing No: EHP/17/1 – Proposed and Existing Plans and Elevations

Reason: For the avoidance of doubt and in the interests of proper planning.

Location Plan (not to scale)