DELEGATED APPLICATION REPORT

Target Date:	24/09/2018				Extended Target Date: 25/10/2018

Application No: 181281/HOU

Address: 82 Whitley Wood Lane, Reading

Proposal: Two storey rear extension and single storey rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site consists of a two storey semi-detached property.  The property has been extended in the past and has a two storey side extension and single storey rear extensions.  The property at 80 Whitley Wood Lane sits further back than the application site and has two storey and single storey rear extensions.  The surrounding area is predominantly residential.

Proposal:

The proposal is for the construction of a two storey rear extension to infill beside the existing two storey rear projection.  The proposed two storey extension measures 1.6m in depth and 2.8m in width.  The proposal is set down significantly from the main roof and has a gable end.  The proposed single storey rear extension has been amended and reduced in depth from 7m to 5m.  The proposal incorporates all the other single storey rear elements and has a flat roof with roof lights.  The proposed single storey rear extension has parapet walls measuring 3m in height.

The proposed single storey rear extension extends along the boundary with 84 Whitley Wood Lane but does not extend any deeper than the existing rear elements of this property.  This extension is also 2.8m from the boundary with 80 Whitley Wood Lane.  The proposed two storey rear extension does not extend any wider than the existing side elevation of the property and although it extends along the boundary with 80 Whitley Wood Lane it misses the first and ground floor side windows to this neighbouring property and there remains a gap of approximately 1m between the boundary and this neighbouring property.

The materials will consist of brick and render and tiles to match the existing property.

Plans Considered:

The applicant submitted the following drawings, received on 23rd July 2018:

Location Plan

Proposed two storey rear extension and single storey rear extension

The applicant submitted the following amended drawing, received on 12th October 2018:

Proposed two storey rear extension and single storey rear extension

Relevant Planning History:

90/00127/FUL (Civica Ref: 900493) - Two storey side extension & single storey rear kitchen & front lobby extensions.  Permitted 04/05/1990.

Consultation Responses:

Ecology raised no objection.

Properties at 68C, 80 and 84 Whitley Wood Lane and 10 Dovecote Road were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The applicant was advised in a letter dated 8th October 2018 that the depth of the single storey rear extension was excessive and it was recommended that it be reduced to 5m.  Amended plans were submitted on 12th October 2018.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposed single storey rear extension exceeds the Council’s guidance of 4m the extension does not extend past the rear of the neighbouring rear addition and the depth of 5m is considered acceptable.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the neighbouring properties will notice the additional height and depth of the proposals they are not considered harmful in terms of visual dominance or overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

Informatives relating to:

- Positive and proactive

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment