DELEGATED APPLICATION REPORT 			Target Date: 19/09/2017

Application No: 171234/HOU

Address: 147 Foxhays Road, Reading

Ward: Church

Proposal: Demolition of existing garage, erection of single storey side and rear extension and front porch

Case officer: Ralph Chakadya

Site Description and Surrounding Area:

The application site contains a semi-detached two-storey property on a residential street.  The house is not listed or in a conservation area.

Plans Considered:

The following plans were submitted:

Amended Drawing No. 17-/PL/07 Rev A Location and proposed block plan

Drawing No.1762/PL/01 Existing Floor Plan

Drawing No.1762/PL/02 Existing Roof Plan

Drawing No.1762/PL/03 Existing Elevation Plan

Drawing No.1762/PL/04 Proposed Floor Plan

Drawing No.1762/PL/05 Proposed Roof Plan

Drawing No.1762/PL/06 Proposed Elevation Plan

Following concerns raised by officers, amended plans were submitted illustrating acceptable car parking provision.

Relevant Planning History:

None

Consultation Responses:

No letters were received from consulted neighbours

Non-statutory consultees

RBC Transport – Following submission of amended plans, there is no objection to the proposal subject to conditions and informatives.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008 and altered 2015)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (Oct 2012 and altered 2015)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM10: Private and Communal Outdoor Space

Supplementary Planning Guidance “A Design Guide to House Extensions” (2003)

Supplementary Planning Guidance “Revised Parking Standards and Design” (2011)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO – Whilst the extension is up to the side boundary, due to the angled relationship, the extension would be well set back from the frontage building line of neighbouring number 147.  The side and front sloping roof with low eaves height is well integrated onto the host dwelling.  The front porch is of an acceptable design size and scale. The porch also satisfies the criteria for permitted development and could therefore still be erected without the need for planning permission.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; - NO. Whilst the extensions breaches the 45-degree angle in relation to the rear ground floor window to neighbouring number 149, the extension with lower eaves height of 2.4m and sloping away from the boundary is not considered to result in harmful loss of light or being overbearing to number 149. Neighbouring number 145 has similar rear extensions and therefore would not be adversely affected.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; No

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; No.

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

- DM4: Safeguarding Amenity states that an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties. The proposed development has been found to be acceptable when assessed against the relevant criteria.

- Parking

Following concerns raised by the Council’s Transport Officer, revised plans illustrating acceptable parking provision were submitted. However the front of the property is already entirely laid out with gravel and cars park on it.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

RECOMMENDATION:

GRANT