DELEGATED APPLICATION REPORT - Application No: 170487/FUL

Address: 146 Friar Street Reading

Proposal: Change of use of first and second floor from B1(a) Office to C3 residential, together with a third floor and roof level extension to the building and alterations to fenestration to form 2 x two bedroom flats and 1 x 4 bedroom flat

Case officer: Matt Burns

Site Description and Surrounding Area: The application site is located on the south side of Friar Street and consists of a three storey building occupied by a retail unit on the ground floor with vacant rooms on first and second floor which were last used as consulting rooms in association with the previous use of the ground floor unit as an optician. The site is surrounded by a mixture of town centres uses. The site is located within the Reading Central Area, Central Core, Primary Shopping Area and Office Core Area within an existing designated Active Shopping Frontage. The adjoined building, no. 147, is grade II listed.

Proposal: The existing building is flat roof with a brick front façade. It is proposed to replicate this form by way of the proposed third floor roof extension.  On top of this would be a pitched roof extension.

The proposal involves converting the existing 1st and 2nd floors to 2 x 2 bedroom flats, creating a third 4 bedroom flat split over a new 3rd floor and roof addition.

Alterations to fenestration include the addition of decorative window arches to the four existing second floor front windows and the enlargement of the four existing first floor front windows to reflect the position and proportions of the existing second floor windows (also including addition of decorative window arches). To the rear of the property it is proposed to insert an additional first floor window and replace an existing second floor window with a double set of doors and juliet balcony. The second floor window arrangement is to be reflected to the rear of the proposed third floor extension.  Rooflights are shown front and rear .

The proposal seeks to retain the existing A1 shop use at ground floor level and would utilise the existing separate ground floor entrance door to the upper floors from Friar Street.

Plans Considered:

Drawings ref.

Site Location Plan – 471487, 173570

Block Plan

1110-P.102 – Scheme A proposed first, second and roof plans

1110-P.101 – Scheme A proposed ground floor plan

1110-P.103 – Scheme A proposed fourth floor and roof plans

1110-P.110 – Scheme A proposed front and rear elevations

1110-P.111 – Scheme A proposed sections AA & BB

1110-D.102 – Existing first, second floor and third/roof plans

1110-D.111 – Existing sections AA & BB

1110-D.101 – Existing ground floor plan

1110-D.110 – Existing front and rear elevations

Received by the Local Planning Authority on 27th March 2017

Relevant Planning History:

172172/FUL - Change of use of first and second floor from B1a Office to a one-bedroom flat (C3) – Granted

170774/FUL - Change of use of first and second floor from B1a Office to C3 residential together with a third floor extension to the building and alterations to fenestration to provide 3 x self-contained one bedroom flats – Under Consideration – pending S106

170654/FUL - Change of use of first and second floor from B1a Office to C3 residential to provide two self-contained flats, together with additional first floor window to rear elevation - Granted

150726/FUL - Proposed change of use of first and second floors from Consulting

Rooms to Offices (B1) - Granted

140031/FUL - Change of use from opticians to Class A1 (shops) on the ground floor to include new shop front – Granted

140033/ADV - 1 fascia sign and 1 awning - Granted

13/00246/CLP - Change of use of 1st and 2nd floors from ancillary A1 use to C3 - Refused

97/00297/ADV - Fascia pod sign internally illuminated, double sided projecting sign internally illuminated - Granted

97/00025/FUL- Alterations to existing shop front to include an additional single door – Granted.

Public Consultation:

Notification letters were sent to properties and no.s 145, 147 and 148 Friar Street and a site notice was displayed. No letters of representation have been received.

Consultation Responses:

Transport: No objection, subject to conditions.

Environmental Protection: Recommend conditions regarding noise and air quality assessments/mitigation.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

In this regard, the NPPF states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Core Strategy and Sites and Detailed Policies Document) according to their degree of consistency with the NPPF (the closer the policies in the plan to the policies in the NPPF, the greater the weight that may be given).

Accordingly, the National Planning Policy Framework and the following development plan policies and supplementary planning guidance are relevant:

### Reading Borough LDF Core Strategy (RCS) (Adopted January 2008 – amended 2015)

CS1	Sustainable Construction and Design

CS2	Waste Minimisation

CS4 	Accessibility and the Intensity of Development

CS5	Inclusive Access

CS11	Use of Employment Land for Alternative Uses

CS14	Provision of Housing

CS15	Location, Accessibility, Density and Housing Mix

CS20	Implementation of the Reading Transport Strategy (Local Transport Plan 2006-2011)

CS24	Car/Cycle Parking

CS33	Protection and Enhancement of the Historic Environment

CS34	Pollution and Water Resources

Sites and Detailed Policies Document (SDPD) (2012, altered 2015)

SD1	Presumption in Favour of Sustainable Development

DM1	Adaption to Climate Change

DM4     Safeguarding Amenity

DM5	Housing Mix

DM6     Affordable Housing

DM8	Residential Conversions

DM10	Private and Communal Outdoor Space

DM12	Access, Traffic and Highway-Related Matters

DM19	Air Quality

Reading Central Area Action Plan (RCAAP) (2009, altered 2015)

RC5	Design in the Centre

RC6	Definition of the Centre

RC9	Living in the Centre

Supplementary Planning Documents & other relevant guidance

Affordable Housing SPD (2013)

Revised Parking Standards and Design SPD (2011)

Sustainable Design and Construction SPD (2011)

Residential Conversions SPD (2013)

# Appraisal

Principle

The proposal is for a conversion and additional development on top of an existing building to residential use and as such is entirely on previously developed land within the urban area. This is therefore considered suitable in principle, subject to any other relevant policies relating to the loss of the office use, design and appearance and the amenities of existing and new residential occupants being satisfied.

Loss of office use

The proposal would retain the existing A1 shop use at ground floor level as such would continue to accord with Policies CS26 and CS27 of the Core Strategy 2008, 2015 and Policy RC6 of the Reading Central Area Action Plan 2009 whilst located within an existing active frontage the proposal would also continue to accord with Policy RC10 of the Reading Central Area Action Plan 2009.

The upper floors of the building are currently in office use following grant of planning permission for change of use from consulting room use ancillary to the previous occupation of the ground floor units as an opticians (ref. 150726).

The principle of the loss of office accommodation and change of use to residential was established under planning permission ref. 170654 for change of use of the first and second floors to two self-contained flats.

The site is not located within a designated core employment area. In such locations Policy CS11 of Core Strategy sets a number of criteria against which proposals resulting in a loss of employment land must be considered. In this instance the office floor space is modest in size whilst there are a number of examples of conversion of upper floors from office to residential in the surrounding area. Given this part of the building has only very recently been occupied as B1a office use and the current provision of office accommodation available within the Borough as a whole it is not considered that the loss would result in any significant economic harm in the context of Policy CS11. As such, the loss of the existing use at the site is established.

Design, Character and Setting of Heritage Assets

Policy CS7 seeks to preserve or enhance the character of the area in which a development is located whilst CS33 seeks that development should preserve or enhance heritage assets, including their setting. Policy RC5 seeks to secure appropriate relationships between buildings, spaces and frontages within the centre of Reading.

The existing building has an oversized shop front fascia which is out of keeping with both adjoined buildings. The proposed reduction of the fascia space to accommodate the enlarged first floor front windows is considered to enhance the appearance of the building as a whole with the street-scene.

The proposed extensions to the building would raise the height of the building up to the ridge line of the adjacent Grade II Listed No. 147 Friar Street and above the roof height of the adjacent unlisted WHS building (no. 145 Friar Street).

The proposed two storey addition is considered to represent an unsympathetic addition to the existing modest mid terrace building, particular given the narrowness of the building. The proposed eaves level and roof form is considered to result in an awkward integration with the intricate and detailed form of the eaves to the grade listed building at no. 147, whilst the eaves level would be sited halfway up the mansard roof storey of no. 145, again resulting in awkward transition between roof forms. This is considered to result in an extension that would fail to integrate satisfactorily with the host or adjoining buildings and would not contribute positively to the character and appearance of this part of the street-scene.

The failure of the proposal to integrate with the character and appearance of no. 147 is considered to result in harm to the setting of this grade II listed building. The proposal is also considered to harm the historic fabric of the listed building where it would attach to the currently exposed gable wall at roof level.

The proposal, due to the roof extension, is not considered to accord with Policies CS7, CS33 and RC5.

Amenity of surrounding occupiers

Policies DM4 and RC9 seek to protect the amenity of surrounding occupiers from the impacts of new development. Policy CS34 seeks to protect surrounding occupiers from the impacts of pollution associated with proposed development.

The proposed roof extension would not expand the footprint of the building. The proposal is not considered detrimental to any surrounding occupiers in terms of loss of light or overbearing impact.

Surrounded by commercial properties and with a yard area to the rear it is not considered that the proposed change of use or extension would result in any undue overlooking, loss of privacy or disturbance to nearby occupiers.

The proposal is considered to accord with Policies DM4, RC9 and CS34.

Amenity of Future Occupiers

Policy DM4 also seeks to protect the amenity of future occupiers of development and Policy RC9 also seeks that new residential development within the Town Centre is adequately protected from surrounding noise and disturbance sources.

The flats proposed are considered to be of suitable size with acceptable outlook and daylighting and stacking of rooms between units.

The site is situated within a busy town centre location with a number of surrounding commercial uses with the potential to cause noise and disturbance to future occupiers. Were permission to be granted to introduce sensitive residential units into this location conditions to require noise and air quality assessment and mitigation schemes to be submitted and approved by the Local Planning Authority prior to commencement of development would be used.

The proposal is not considered to be contrary to Policy DM4, RC9 and CS34.

Transport

The site is located within the Reading Central Area and within Reading’s primary shopping area.  The site is in a key central location, being situated on the corner of Friar Street and Queen Victoria and in close proximity to Reading rail station to the north.  This area is well served by rail and bus links and also contains the largest proportion of public car parking spaces.

Friar Street itself is used by buses and has access restrictions in place during certain times of the day. There is a 24 hours restriction in place for Friar Street West Bound (except for buses and taxis) and time restricted access Friar Street East Bound.  Access for loading and unloading (only) via Queen Victoria Street is restricted between the hours of 10am and 6pm.

There is no car parking provision and it is intended that the development will remain car free.  Friar Street and the surrounding road network all have extensive parking restrictions preventing on-street parking. Therefore, the non-provision of parking is acceptable given the development sites town centre location. It should be noted that the development site is located in close proximity to a Residents Parking Permit Area; therefore if this application is approved there should be an assumption that any future occupants of the proposed dwelling will not be issued with a resident parking permit which should be covered by condition and an informative applied. This will ensure that the development does not harm the existing amenities of the neighbouring residential properties.

No Cycle storage has been proposed, given that it is unlikely that cycle parking is currently provided for the site, and the lack of external storage space, it is not considered that cycle parking is required for the proposal.

There is considered to be sufficient space within the units for waste and recycling to be stored (prior to on-street collections), without the need for a dedicated facility at ground floor level in this instance.

There are no transport objections to the proposal given that it is likely to generate fewer trips to the existing use. The proposal is considered to accord with Policies CS24 and DM12.

Sustainability

Policy CS1 of the Core Strategy, supported by the Council's Sustainable Design and Construction SPD seeks the proposal, as a ‘creation of new residential units through Conversion’, to comply with BREEAM Level of ‘Very Good’. If permission were to be granted it would be acceptable to deal with this matter by way of a suitably worded condition.

Affordable Housing

West Berkshire District Council and Reading Borough Council applied for a judicial review of the Secretary of State’s Written Ministerial Statement (WMS) to Parliament in 2014 on changes to national planning policy. Those changes sought to exempt developments of 10 or less dwellings from planning obligations for affordable housing and social infrastructure contributions and to introduce a new measure known as the Vacant Building Credit.

The High Court handed down its judgment on the case on 31st July 2015. The High Court found in favour of the challenge by the local authorities and quashed the amendments to the NPPG. The Secretary of State appealed the judgment and the Court of Appeal then quashed the decision of the High Court.

At its meeting of the Strategic Environment Planning and Transport Committee on 13th July 2016, the Council discussed the outcome of the Court of Appeal’s decision on its challenge (http://www.reading.gov.uk/media/5651/Item09-SEPT-C-Report-on-C-of-Appeal-judgement-05-16/pdf/Item09_SEPT_C_Report_on_C_of_Appeal_judgement_05_16.pdf ).

Having considered its options, the Committee agreed the following as the basis for determining planning applications where Policy DM6 of the SDPD is relevant:

To implement Policy DM6 as currently adopted in the SDPD but excluding proposals that solely involve the conversion of an existing property, where the conversion involves the provision of 10 or less dwelling units (i.e. not HMOs), or the replacement of dwellings by the same number of replacement dwellings where there is no net increase.

Whilst the proposal relates to the conversion of an existing property it incorporates a significant extension to the roof which facilitates provision of the provision of one of the three units proposed in its entirety. On this basis Policy DM6 is applied to the single unit not forming part of the conversion only and as would attract an equivalent contribution for provision of 10% of the housing to be provided off-site elsewhere in the Borough. The applicant has agreed the principle of making a contribution but in the absence of a legal agreement failure to meet policy DM6 requirements would form a reason for refusing planning permission.

Community Infrastructure Levy

The existing office space and first and second floor is in use and therefore the new floor space within the new third floor and roof extension only would be liable for CIL.

RECOMMENDATION:

Refuse Planning Permission:

Reasons

- The proposed two storey addition, by virtue of its massing, eaves level and roof form, would fail to integrate satisfactorily with the host and adjoining buildings (no. 147 and 145), detracting from their character and failing to preserve the character of this part of the street. The failure of the proposal to integrate satisfactorily with no. 147 would result in harm to the setting and historic fabric of this grade II listed building and fail to preserve or enhance is historic character. The proposal is contrary to Policies CS7 and CS33 of the Core Strategy 2008, 2015 and Policy RC5 of the Reading Central Area Action Plan 2009.

- In the absence of a completed legal agreement to secure an acceptable contribution towards the provision of Affordable Housing, the proposal fails to contribute adequately to the housing needs of Reading Borough, contrary to Policy DM6 (Affordable Housing) of the Sites and Detailed Policies Document 2012 (altered 2015) and the Council’s Adopted Affordable Housing Supplementary Planning Document 2013.

Informatives

- Officers have worked positively and proactively.

- Reason for refusal could be met by a UU.

- Refused plans:

1110-P.102 – Scheme A proposed first, second and roof plans

1110-P.101 – Scheme A proposed ground floor plan

1110-P.103 – Scheme A proposed fourth floor and roof plans

1110-P.110 – Scheme A proposed front and rear elevations

1110-P.111 – Scheme A proposed sections AA & BB