# DELEGATED REPORT

App No.: 171068/FUL

Address: Western Elms Lodge", Western Elms Avenue, Reading

Case officer: Ralph Chakadya

INTRODUCTION:

The site is an existing doctors’ surgery at the corner of Oxford Road and Western Elms Avenue. The surgery fronts Western Elms Avenue.

There is existing on site parking to the front of the surgery.

PROPOSAL:

Permission is sought for alterations and first floor extensions with rooms in roofspace to an existing Surgery. The extensions would provide additional consulting rooms on the first floor and training / meeting room within the roofspace.

Access to the site is from Western Elms Avenue and existing parking would be retained. Additional cycle parking is to be provided in the form of 5 additional spaces

Following concerns raised by officers regarding the design and street scene appearance of the proposed extensions, amended plans were submitted illustrating the street scene in relation to the adjoining residential block 2-4 Western Elms Avenue and changes introduction of a pitched roof dormer to improve the design of the extensions. These plans were considered acceptable.

PLANNING HISTORY:

900517 - Temporary permission (12 months) for portakabin to accommodate Doctor

Surgeries. Granted December 1990

900793 – Alteration and extension to doctors’ surgery including demolition of garage, repositioned access and additional car parking. Granted May 1990

CONSULTATIONS:

RBC Transport:

The proposal is for alterations and extensions to the existing Surgery creating additional consulting rooms on the first floor and a training / meeting room on the second floor.

The proposal retains the existing access arrangements along with the current car parking provision which equates to 22 spaces.  It is however acknowledged that not all of these parking spaces are to the correct dimensions but as this is an existing situation and the parking is to be retained solely for staff this is deemed acceptable.

However, the increase in consulting rooms would require an increased parking provision equating to 1.5 spaces per consulting room and 1 space per general practitioner, this would equate to a total 15 spaces.  The applicant has provided a travel plan in order to reduce vehicle trips associated with staff and therefore I am happy that no additional car parking is required for staff.

There is however likely to be an increase in the demand for vehicle parking associated with patients, the transport statement states that ‘Also along Western Elms Avenue, there is adequate on street parking within easy walking distance of the site. There are other areas of unallocated on street parking in the general vicinity of the site. In many areas nearby the parking is restricted to 2-hour parking during the day. These areas are generally suitable for visitors to the surgery or patients’.  Given that people are unwell when they visit the doctors they are more likely to travel by car than alternative modes and therefore this would increase the demand for on street parking.  It is also noted that the opening times of the surgery would be 7am until 9pm and this would be outside of the current parking restriction that allows for two-hour parking between 8am and 6.30pm when there is more likely to be a higher demand for parking associated with the residential.  Therefore to ensure that adequate parking is available along Western Elms Avenue a car parking survey is required for a weekday, Saturday and Sunday for the full duration of the proposed opening hours.

This has been provided and the results have identified that during the week of the survey only an hour period on one day resulted in insufficient parking being available to accommodate the extended surgery.  This would be based on a worst case scenario and given that the vast majority of the week sufficient parking would be available I am happy that the parking provision is acceptable.

Additional cycle parking is to be provided in the form of 5 additional spaces, however the Councils minimum standard associated with the proposed intensification would be 1 space per 2 staff & 1 stand per consulting room, this therefore equates to an additional 9 spaces.  A revised drawing is therefore required illustrating a provision of 12 cycle spaces.  These should be located within a conveniently located, lockable store equipped with Sheffield type stands.  Discussions with the agent have identified that revised plans would be submitted but these have not been forthcoming, I am however happy for this to be dealt with by way of a condition.

In the circumstances there are no transport objections to the proposal subject to the conditions.

Public/local consultation and comments received

A letter of support was received from the NHS North & West Reading Clinical Commissioning Group.

RELEVANT PLANNING POLICY AND GUIDANCE

Relevant Planning Policy

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise. Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Guidance

National Planning Policy Framework (NPPF)

Reading Borough Local Development Framework Core Strategy Document, 2008.

Policy CS7 (Design and the Public Realm)

Policy CS24 (Car/Cycle Parking)

Policy CS31 (additional and Existing Community Facilities)

Policy CS34 (Pollution and Water Resources)

Reading Borough Local Development Framework: Sites and Detailed Policies Document (2012)

Policy DM3 (Infrastructure)

Policy DM4 (Safeguarding Amenity)

Policy DM12 (Access, Traffic and Highway-Related Matters)

Supplementary Planning Guidance/Documents

‘Revised Parking Standards and Design’ (2011)

APPRAISAL

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.

Main considerations

i. The Principle of Development

The principle for the extensions is considered to accord with policy CS31 that seeks to promote ‘new, extended or improved community facilities, particularly where this will involve co-location of facilities on a single site’. The extensions are to an existing facility. Subject to other planning considerations, the principle for the extensions is considered acceptable.

ii. Character and Design

As highlighted within the proposals section above, amended plans illustrating the street scene and improving design of the front elevation were submitted. The existing north wing of the building is forms the original building to the site and defines this corner of Oxford Road and Western Elms Avenue.  The contemporary additional extensions to the south wing of the building would be more in keeping with the adjoining 2-4 Western Elms Avenue building. The existing lower mid-section of the building would retain the visual separation between the original building and the extensions. The additions would introduce built form that is not considered to be detrimental to the host building and the character of the area. As such it accords with policies CS7 and CS31.

iii. Amenity

The extensions would adjoin a flat roof rear element to neighbouring number 319 Oxford Road to the rear and e Additional cycle parking is to be provided in the form of 5 additional spaces the proposal is therefore not considered to have an adverse effect onto the amenities of number 319 Oxford Road in terms of being overbearing. No harm would be caused to neighbouring number 2 Western Elms Avenue as there are no windows to the side elevation facing the proposed extensions.

iv. Access/ Parking

The proposal would result in the increase in consulting rooms that would require an increased parking provision equating to 1.5 spaces per consulting room and 1 space per general practitioner. The Council’s Transport officer has assessed the proposals and noted the applicant has provided a travel plan in order to reduce vehicle trips associated with staff. The travel plan is considered acceptable therefore no additional parking is required for staff.

To ensure adequate parking provision for the visiting patients, the Council’s Transport officer requested a car parking survey for a weekday, Saturday and Sunday for the full duration of the proposed opening hours. The survey results demonstrated that sufficient parking would be available for the patients. Therefore no additional parking is required as a result of the extensions.

As requested by the Council’s Transport officer, a revised drawing illustrating a provision of 13 cycle spaces was submitted. The application is considered to accord with Policy DM12 of the SDPD.

v. Trees and Ecology

No implications.

vi. Other issues:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. The key equalities protected characteristics include age, disability, gender, gender reassignment, marriage and civil partnership, pregnancy and maternity, race, religion or belief, sexual orientation.  There is no indication or evidence (including from consultation on the application) that the protected groups have or will have different needs, experiences, issues and priorities in relation to the particular planning application.

In terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Conclusion

Grant permission