DELEGATED APPLICATION REPORT 				Target Date:	07/06/2018

Application No: 180368/HOU

Address: 98 St Georges Road, Reading

Proposal: Single storey rear/side extensions

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site consists of a two storey mid-terrace property with a two storey rear gable and a single storey addition to the rear of the gable.  The surrounding area is predominantly residential.

Proposal:

The proposal involves single storey extensions to the side of the existing rear gable and to the rear of the existing gable.  The proposed extension to the side of the gable measures 3.8m in depth and 1.7m in width.  The proposed extension to the rear of the gable measures 3.8m in depth and 2.5m in width.  Both extensions have a flat roof and measure 2.7m in height.  The proposed extensions extend along the boundaries with both neighbouring properties but do not extend any deeper than existing extensions at 96 and 100 St Georges Road.

The materials will consist of brick to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 26th February 2018:

Drawing No: 02 – Existing Floor Plans

Drawing No: 03 – Existing Elevations and Sections

Drawing No: 06 – Proposed Floor Plans (2)

Drawing No: 07 – Proposed Elevations and Section (FP2)

The agent submitted the following plan, received on 12th April 2018:

Drawing No: 01 Rev A – Location And Block Plan

Relevant Planning History:

180369/HOU - Ground and first floor rear extensions and conversion of two bedroom house into a three bedroom house.  Pending consideration.

Consultation Responses:

Transport raised no objection subject to an informative.

Properties at 96 and 100 St Georges Road and 83 and 85 Shaftesbury Road were consulted.  No comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered harmful in terms of visual dominance or overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

Informatives relating to:

- Positive and proactive

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment

6.	Highways