DELEGATED APPLICATION REPORT 			Target Date: 03/04/2018

Application No: 180228/HOU

Address: 152 Southcote Lane, Reading

Proposal: Two storey side and single storey front and rear extensions

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The site comprises a two storey end-terrace property.  The property is at a lower level than the road and there is a driveway for two cars.  The side boundary of the application site is the rear boundary to properties along Fawley Road.  The surrounding area is predominantly residential.

Proposal:

The proposal involves a two storey side extension and single storey front and rear extensions.  The proposed two storey side extension measures 3.5m in width and 6.8m in depth.  The proposal is set back from the existing front elevation of the property by 0.7m and set down from the main roof by 0.3m.  The roof design matches the existing pitched roof.  The proposal would be a minimum of 1m from the rear boundaries of 2 and 4 Fawley Road.  No first floor side windows are proposed (secure by condition).

The proposed single storey rear extension measures 3m in depth and extends the full width of the property and to the rear of the proposed two storey side extension.  The proposal has a mono pitched roof and measures 2.5m in height to eaves level and 3.2m in height at its highest point.  The proposal extends along the boundary with 150 Southcote Lane.

The proposed single storey front extension measures 1.5m in depth and 6m in width.  The proposal has a mono pitched roof and measures 2.5m in height to eaves level and a maximum height of 3.8m at its highest point.

The materials consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 5th February 2018:

Drawing No: PL-01 – Location and Block Plan

Drawing No: PL-02 – Existing Plans

Drawing No: PL-03 – Proposed Plans

The agent submitted the following amended drawing, received on 19th March 2018:

Drawing No: PL-03 Rev B – Proposed Plans

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection.

Ecology raised no objection.

Properties at 150 Southcote Lane, 2, 4 and 6 Fawley Road and 4 and 6 Aldworth Close were consulted.  No comments have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 15th March 2018 that in principle the proposal was considered acceptable however it was lacking subservience.  Amended plans were submitted on 19th March 2018 which set the two storey side extension back from the existing front elevation by 0.7m and set the roof down by 0.3m.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM12: Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal is considered a subservient addition to the property.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposals they are not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

- No first floor side windows

Informatives relating to:

- Positive and proactive

- Highways

- Terms and conditions

4.	Building regulations

5.	Construction and demolition

6.	Encroachment