DELEGATED APPLICATION REPORT 				              Target Date: 07/06/18

Application No: 180599/HOU

Address: 1 Polsted Road

Proposal: Part one, part two storey side and rear extension

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site is a two storey end of terraced dwelling with a pitched roof. The dwelling benefits from a loft conversion with a rear box dormer. The site benefits from a relatively wide frontage, which tapers to the rear. The site is within an Article 4 area controlling the use of patterned brickwork. The surrounding area is predominantly residential. The United Reformed Church is located beyond the south-westward site boundary. Other neighbouring dwellings have been variously extended to the rear.

Proposal

The proposal is for a part one, part two storey side and rear extension. The proposal would extend beyond the south-westward elevation at a width of 4.2m. A minimal distance would be maintained along the south-westward site boundary. The proposal would project along the side elevation, following the splay of the site boundary, to a depth 6m beyond the original rear elevation. The proposal would extend across the full width of the original rear elevation at this depth at the ground floor, and at a width of 3.8m at the first floor. Beyond the original side elevation the proposal would have a pitched roof, with a maximum height of 8.2m and a height to eaves of 6m, set down from the existing ridgeline by 1m. Beyond the original side and rear elevations, the two storey element would have a further pitched roof, separated by a short flat roof section. This element would have a maximum height of 8.5m and a height to eaves of 6m. Two roof lights would be located within the rearward roof slope. The single storey rearward element would have a flat roof at a height of 4m. A roof lantern would be located within the section of flat roof, projecting to an additional height of 0.3m. To the front elevation, a window and single door would be located at the ground floor, with a further window at the first floor. To the rear elevation, two sets of bi-fold doors would be located at the ground floor with two further windows at the first floor. Materials have been selected to match those of the existing dwelling.

Plans Considered

The agent supplied the following, received on 10/04/18:

Drawing No: 1 – Site Location, Block Plan, Existing and Proposed Floor Plans and Elevations

Relevant Planning History

940853/FUL – Loft conversion and insertion of window to forward roof slope – Permitted 22/11/94

Consultation Responses

Transport advised that in accordance with the Council’s adopted standard, the dwelling is required to provide two off-road parking spaces. Transport advised that plans should be supplied illustrating off-road parking provision. Officer note: having visited the site, officer’s are satisfied that the existing off-road parking arrangements would not be affected by the proposal

Summary of discussions with applicant/agent

The agent was advised on 25/05/18 that the proposal was unacceptable and subject for refusal. The proposal represents a substantial addition to the original dwelling and considered cumulatively, lacks subservience to it. Of particular concern is the two storey side and rear element which is considered to be of poor design. The pitched roof of this element makes the proposal appear distinctly separate from the host dwelling, more akin to a separate residential unit. Furthermore, the pitched roof would partially obscure the outlook from the rearward windows of the existing dormer, causing significant harm to the residential amenities of current and future occupiers. The depth of the rearward projection is also considered to be excessive, presenting an excessive scale and bulk which detracts from the character of the original dwelling. The height and depth of the rearward element would also appear visually dominant, with associated effects presented to the adjoining dwelling. The Council’s guidance states that rear extensions that exceed a depth of 4m will not normally be granted planning permission. It is not considered that the site is capable of accommodating a rearward extension of a greater depth.

The agent was also advised that the proposal is not without merit, and individual components of it might be acceptable. For example, viewed from the front elevation, the proposed two storey side extension does appear subservient – set back from the principle elevation and set down from the existing ridgeline. A significantly reduced rear extension might also be acceptable.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

Design and impact on the character of the surrounding area

Policy DM9 of the Sites and Detailed Policies Document states that an extension to a house should respect the character of the house in terms of scale, location, materials and design. The extension should also respect the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line. Policy CS7 of the Core Strategy also states that all development must be of a high design quality that maintains and enhances the character and appearance of the area. The Council’s Design Guide to House Extensions states that planning permission will not normally be granted for rear extensions that are longer than 4m, and that they should be located as far away from side boundaries as possible. The Design Guide also states that for terraced housing, rear extensions can have a significant and detrimental impact on neighbouring properties and the general appearance of the terrace. The acceptability of rear extensions will be derived from the particular circumstances of the application site. Two storey side extensions should normally be designed to be smaller in scale than the main house, set down from the existing roof line and set back from the front elevation.

The application site is a two storey end of terraced dwelling with characterful patterned brick work. The dwelling benefits from a loft conversion with the provision of a rear box dormer, though the footprint of the dwelling has not been previously extended. The proposal represents a substantial addition to the original dwelling and considered cumulatively, would not be subservient to it. Of particular concern is the two storey side and rear element which is considered to be of poor design. The pitched roof of this element makes the proposal appear incongruous, distinctly separate from the host dwelling and more akin to a separate residential unit. This is exacerbated by the form and relationship with the existing rear box dormer, and does not contribute to a characteristic roof form. Furthermore, the pitched roof would partially obscure the outlook from the rearward windows of the existing dormer, causing significant harm to the residential amenities of current and future occupiers.

The depth of the rearward projection is also considered to be excessive, presenting a jarring scale and bulk which detracts from the character of the original dwelling and deviates from the general norm of neighbouring dwellings. The Council’s guidance states that rear extensions that exceed a depth of 4m will not normally be granted planning permission. It is not considered that the site is capable of accommodating a rearward extension of a greater depth.

The proposal is not without merit, and individual components of it may be acceptable. The two storey side element, considered individually, does appear subservient. It is set back from the principle elevation and set down from the existing ridgeline. Despite this, when considered cumulatively the proposal does not reflect the scale and proportion of the original dwelling due to its size and poor design and it is not in character with, or subservient to, the original dwelling. The proposal does not complement the existing dwelling, appearing overly dominant, and unbalances the terraced row which detracts from the character of the street scene. The dual-pitched roof would have a jarring effect on the existing dwelling. The proposal is therefore contrary to Policy CS7 of the Core Strategy, Policy DM9 of the Sites and Detailed Policies Document and The Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

Impact on residential amenity

The single storey element of the proposal would project to a depth of 6m, with a flat roof height of 4m, along the boundary with the 3 Polsted Road. It is acknowledged that this neighbouring dwelling does benefit from its own single storey rear extension, up to the boundary with the application site, projecting to a depth of approximately 3.5m. As a result of this, this element of the proposal is considered to cause some overbearing effect and visual dominance to this neighbour. The effect is significantly exacerbated by the scale and bulk of the two storey rearward element of the proposal. Cumulatively, the proposal is considered unneighbourly. As a result of the dual-pitched roof form of the proposal, the outlook from the rearward windows of the existing dormer would be partially obscured. This is considered to cause significant harm to the residential amenities of current and future occupiers. As such, the extension is contrary to Policy DM4 of the Sites and Detailed Policies Document and The Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

RECOMMENDATION

REFUSE

Informatives relating to:

- Refused plans

- Positive and proactive