DELEGATED APPLICATION REPORT 			Target Date: 13/03/2018

Application No: 180087/HOU

Address: 33 Northcourt Avenue, Reading

Proposal: First floor rear extension, additional velux windows, side dormer window and ground floor side extension.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The site comprises a detached chalet style dwelling with a single storey rear extension and dormer windows to the north-west side roof slope.  The property is adjacent to the University of Reading and is one of ten similar style properties within a row along Northcourt Avenue.  The row of chalet style bungalows are slightly unusual in that they are located directly along the south east boundary of each plot and therefore not centrally located within the plot.  The surrounding area is predominantly large, detached residential properties of varying design.

Proposal:

The proposed single storey side extension would be to the side of the existing single storey rear extension and would measure 2.7m in width, 5.4m in depth and 3.2m in height.  The proposal would have a flat roof and would create storage and a bike store.  The proposal would be marginally off the boundary with the neighbouring property at 31 Northcourt Avenue and would extend 4m from the rear of this neighbouring property.

The proposed first floor rear extension would extend directly above the existing single storey rear extension and would measure 7.4m in width and 4m in depth.  The roof height and pitched roof design would match the existing property.  Three additional velux roof windows are proposed and two rear windows.  The proposal would create an additional bedroom with en-suite.

A side dormer window is also proposed to the north-west elevation to match two existing dormer windows within the same roof slope.  The dormer window would be obscure glazed and fixed up to 1.7m in height above internal floor level (secure by condition).

The materials consist of brick to the ground floor, render to the first floor, timber cladding to the dormer and tiles to match the existing property.

For information, two previous applications for similar proposals have been refused due to the impact of the first floor rear extension on the neighbouring property at 35 Northcourt Avenue.  However, since this time 35 Northcourt Avenue has been granted planning permission (application ref: 170746) for a two storey rear extension along with a lawful development certificate for two side dormer windows and four roof lights (application ref: 170692).  These permissions have been constructed and therefore the previous concerns that related to the impact on this neighbouring property have been removed.

Plans Considered:

The agent submitted the following drawings, received on 16th January 2018:

Location Map

Drawing No: 1 – Existing Floor Plan

Drawing No: 2 – Proposed Floor Plan

Drawing No: 3 – Existing Elevations

Drawing No: 4 - Proposed Elevations

Drawing No: 5 – Block Plan

Relevant Planning History:

08/00247/FUL (Civica Ref: 080715) - Single storey rear extension to replace existing conservatory. Replace existing upstairs front window.  Permitted 23/04/2008.

10/00763/FUL (Civica Ref: 100156) - First floor rear extension; addition of two dormer windows, two velux windows and window to primary side elevation on first floor.  Withdrawn 10/06/2010.

10/1192/FUL (Civica Ref: 100772) - First floor rear extension and addition of ground floor window to side elevation (resubmission of 10/00763/FUL).  Refused 01/09/2010.

10/02267/CLP (Civica Ref: 101856) - Certificate of lawfulness for two proposed dormer windows.  Permitted 08/02/2011.

150024/HOU - Single storey side extension and first floor rear extension.  Refused 19/03/2015.

171624/PREAPP - Pre-application advice for first floor rear extension, with additional

velux and dormer window and ground floor side extension.  Observations sent 16/10/2017.

Consultation Responses:

Transport raised no objection.

Ecology raised no objection.

Properties at 31 and 35 Northcourt Avenue and 34 Shinfield Road were consulted.  No comments have been received.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM12: Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposals they are not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

- Side dormer window to be obscure glazed and fixed

Informatives relating to:

- Positive and proactive

- Highways

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment