DELEGATED APPLICATION REPORT

Target Date: 07/05/2018				Extended Target Date: 18/06/2018

Application No: 180360/HOU

Address: 70 Brooksby Road, Tilehurst, Reading

Proposal: Two storey side and rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property with an attached single storey side garage that extends along the boundary with 72 Brooksby Road.  The pair of semi-detached properties are unusual in their design.  The application site has a gabled front elevation whilst the adjoining property at 68 Brooksby Road is set back from No.70 and has a gabled side elevation.  This is characteristic of the semi-detached properties within the road.

Brooksby Road rises upwards and the application site is located towards the top, on a slight bend.  Due to its location the property has a prominent position and is at a higher level than the neighbouring property at 72 Brooksby Road.  The surrounding area is predominantly residential of similar style properties.

Proposal:

The proposal is for the construction of a part two storey part single storey side and rear extension.  There have been numerous amendments to the original scheme however the proposal is now set down from the main roof of the property by 0.5m, set back from the existing front elevation of the property by 1m and set in from the boundary at first floor by 0.5m.  The proposal has a hipped roof and the ground floor rear/side extension measures a maximum of 4m in depth and the first floor a maximum of 3m in depth.

The proposed ground floor side/rear extension extends along the boundary with 72 Brooksby Road and extends 4m deeper to the rear than the existing garage.  The proposed ground floor rear extension also extends along the boundary with the neighbouring property at 68 Brooksby Road although it is no deeper than an existing single storey rear extension.  The proposed first floor rear extension is 2.4m from the boundary with this neighbouring property.

The proposals do not encroach on a line taken at 45 degrees from the closest habitable windows to both neighbouring properties although there is a ground floor side window to 72 Brooksby Road which is currently being used as a bedroom.  72 Brooksby Road is at a lower level and is set further back than the application site and it is also located at a slight angle on the plot which means the property is further away from the side boundary at the rear than it is at the front.

A new storm porch canopy is also proposed above the front entrance door.  Two parking spaces will remain on the frontage although this will require a new retaining wall.

The materials would consist of brick and render and brown tiles to match the existing property.

Plans Considered:

The applicant submitted the following drawings, received on 26th February 2018:

Drawing No: 01 – Existing Elevations

Drawing No: 02 – Existing Plans

Drawing No: 03 – Proposed Elevations

Drawing No: 04 – Proposed Ground Floor Plan

Drawing No: 05 – Proposed First Floor Plan

Drawing No: 06 – Proposed Plans

The applicant submitted the following amended drawings, received on 19th May 2018:

Drawing No: 01 – Existing Elevations

Drawing No: 02 – Existing Plans

Drawing No: 03 – Proposed Elevations

Drawing No: 04 – Proposed Ground Floor Plan

Drawing No: 05 – Proposed First Floor Plan

Drawing No: 06 – Proposed Plans

The applicant submitted the following amended drawings, received on 4th June 2018:

Drawing No: 01 – Existing Elevations

Drawing No: 02 – Existing Plans

Drawing No: 03 – Proposed Elevations

Drawing No: 04 – Proposed Ground Floor Plan

Drawing No: 05 – Proposed First Floor Plan

Drawing No: 06 – Proposed Plans

The applicant submitted the following amended drawings, received on 17th June 2018:

Drawing No: 01 – Existing Elevations

Drawing No: 02 – Existing Plans

Drawing No: 03 – Proposed Elevations

Drawing No: 04 – Proposed Ground Floor Plan

Drawing No: 05 – Proposed First Floor Plan

Drawing No: 06 – Proposed Plans

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection subject to a condition and informatives.

Properties at 68 and 72 Brooksby Road, 5 Oakham Close and 4 Mapledurham View were consulted.  No comments have been received.

Summary of discussions with applicant/agent:

The applicant was advised in an email dated 2nd May 2018 that there was no objection to a two storey side and rear extension however there were concerns with regards to the proposal as submitted.  The applicant was advised that the proposal was not considered a subservient addition to the property as it was not set down from the main roof and the first floor side extension was not set in from the boundary with 72 Brooksby Road.  There were also concerns with regards to the impact on the neighbouring property at 72 Brooksby Road in terms of the proposal being visually dominant and overbearing and loss of light to a side window due to the difference in levels and proximity to boundary.

There have been numerous amendments to the proposal with acceptable amended plans being submitted on the 17th June 2018.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM12: Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal meets general policy and guidance.  It would have been preferable for the first floor side extension to be set in from the boundary by 1m however, 0.5m is considered acceptable in this instance as the proposal has been set back from the existing front elevation by 1m and sufficiently set down from the main roof.  As the neighbouring property is also at a lower level and set further back than the application property the proposal does not create a terrace effect.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; There will be some loss of light to the ground floor side window at 72 Brooksby Road however there is already some loss of light to this window due to the location of the existing garage at the application site along the boundary.  The proposed side extension is not considered to significantly reduce the amount of light to this room to an unacceptable level.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, there are no side windows proposed and a condition will be imposed which restricts side windows to protect the privacy of the neighbouring properties.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, the neighbouring properties will notice the additional height and depth of the proposal close to the boundaries, particularly 72 Brooksby Road which is at a lower level however the two storey element of the proposal does not extend much further than the existing two storey section of this neighbouring property and the proposal is therefore not considered to have a detrimental impact on the residents of these neighbouring properties.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Approved Plans

- Materials

- Vehicle parking

- No side windows

Informatives:

- Positive and Proactive

- Terms and Conditions

- Building Regulations

- Construction and Demolition

- Encroachment

- Highways

- Damage to Verge