DELEGATED APPLICATION REPORT 				Target Date:	06/03/2018

Application No: 180035/HOU

Address: 94 Amblecote Road, Tilehurst, Reading

Proposal: Single storey rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a bungalow with a flat roof rear conservatory.  The property has a long rear back garden and a garage to the rear which is to be demolished.  There are levels at the site with steps down from the bungalow to the garden.  The surrounding area is predominantly residential consisting of bungalows and two storey properties.

Proposal:

The proposal has been amended and the height of the proposed extension reduced from 3m in height to eaves level and 4.3m in height at its highest point to 2.5m in height to eaves level and 3.9m in height at its highest point.  The proposal involves the removal of the existing conservatory and the construction of a 6m depth single storey rear extension.  The proposal has a pitched roof with glazing to the rear elevation and 4 rooflights.  A rooflight is also proposed to the existing roof and a new window along the side elevation of the existing property.

The first 2.5m of the proposed extension would extend along the boundary with 92 Amblecote Road (which is the same as the existing arrangement) and then it steps away from the boundary by 0.3m.

Due to the levels at the site there will be steps down from inside the existing bungalow into the proposed extension which has allowed the height of the extension to be reduced.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 9th January 2018:

Drawing No: 01 – Existing Plans & Elevations

Drawing No: 02 – Proposed Ground Floor Plan

Drawing No: 03 – Proposed Plans & Elevations

Drawing No: 04 – Site & Block Plans

The agent submitted the following amended drawings, received on 14th February 2018:

Drawing No: 02 Rev A – Proposed Ground Floor Plan

Drawing No: 03 Rev A – Proposed Plans & Elevations

Drawing No: 04 Rev A – Site & Block Plans

Relevant Planning History:

None.

Consultation Responses:

Properties at 92 and 96 Amblecote Road and 316 and 318 Tilehurst Road were consulted.  A site visit was made to 92 Amblecote Road to assess the application from this neighbouring property.  This neighbouring property was notified of the amendments on 14th February 2018 and no comments have been received.

Summary of discussions with applicant/agent:

The agent was advised that there were concerns with the height of the proposal along the boundary with 92 Amblecote Road.  As the internal floor level was being lowered amended plans were submitted to reflect this and the height of the extension was reduced which overcame the initial concerns raised.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposal exceeds the Council’s guidance of 4m in depth there is an existing 2.5m depth conservatory and the house and garden are considered capable of taking an extension of a longer depth.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the proposed extension has been reduced from 3m in height to eaves level to 2.5m and as the roof of the extension hips away from the neighbouring property the proposal is not considered to significantly reduce the amount of light to this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, the proposal is a sufficient distance away from the neighbouring property at 96 Amblecote Road and although the residents of 92 Amblecote Road will notice the additional height and depth of the proposal the reduced height and depth of 3.5m from the rear conservatory of this neighbouring property reduces the overall impact in terms of visual dominance and overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

Informatives relating to:

- Positive and proactive

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment