Sustainability Measures, 27 Queen Street, Caversham

Roof Insulation 	Existing roof has no insulation. Main part of roof to both houses to be upgraded to U value of 0.16 W/m2K being 100mm fibreglass insulation between joists and 170mm min. fibreglass insulation at right angles over joists.  Insulation between ceiling joists to be linked to cavity wall insulation (fire-stopping to be used where necessary).

Glazing		All windows to be double glazed.  All UPVC windows to be double glazed with min.16mm double glazed units and to have low energy glass  with En of 0.05 max. to give a max. u-value of 1.6W/m2oC.

Boilers		New boiler to each property  to be condensing type and to be A rated with 90% efficiency.  Details of zone controls, timing controls and boiler control interlocks to be fitted in connection to a space heating system to be made available to householders.

Light bulbs		All light bulbs to be energy efficient LED type.  Fixed energy efficient light fittings to number not less than 3 per 4 fixed light fittings.  Fixed external light fittings to be either A.) lamp capacity does not exceed 150 W and lamp switches off when there is not enough daylight or when not required at night or B.) light fittings have sockets that can only be used with lamps having an efficacy greater than 45 lumens per circuit-Watt.

Bicycle storage	Two secure, covered and lockable storage spaces per dwelling will be provided

Water fittings	 Water rating and usage to comply with the following table :

1.

Sustainability Measures, 27 Queen Street, Caversham

Continued

Water fittings cont.

1.Low flow taps to be used (subject to any minimum flow required to activate boiler for hot water

2.Low volume, dual flush toilets to be used

3.Reduced volume baths to be installed

4.Low flow showers with aerated shower heads to be used

5.Low water, A rated washing machines and dishwashers to be used

2.