DELEGATED APPLICATION REPORT

Target Date:	13/04/2018				Extended Target Date: 27/04/2018

Application No: 180309/HOU

Address: 116 Overdown Road, Tilehurst, Reading

Proposal: Two storey side extension, single storey rear extension to garage and part modification of garage roof.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property.  Due to levels at the site the property is raised up from the road and the rear garden continues to rise upwards.  The property has a detached garage (which is adjoined to the neighbouring garage at No.114) which is at a lower level than the property.  The property is at a higher level than the neighbouring property at 114 Overdown Road.  The surrounding area is predominantly residential.

Proposal:

The proposal has been amended to overcome initial concerns and involves a two storey side extension, a single storey rear extension and a pitched roof to the existing garage.

The proposed two storey side extension measures 3.1m in width and 7.3m in depth.  The proposed extension is set back from the existing front elevation of the property by 0.5m and is set down from the main roof by 0.3m.  The proposal has a hipped roof design to match the existing roof.  Due to the levels at the site the proposal is at a lower level than the existing property however the windows are generally in line with the windows in the existing property.  The proposed two storey side extension would be 2.8m from the boundary with 114 Overdown Road.

The proposed single storey extension is to the rear of the garage and measures 3.1m in depth.  The proposal has a flat roof and measures 2.6m in height.  The proposal is 1m from the boundary with 114 Overdown Road.

Due to the levels on site the existing garage measures 3m in height from the front and 2.2m in height from the rear.  The proposed pitched roof measures 2.5m in height to eaves level and 3.5m at its highest point from the front and 1.5m in height to eaves level and 2.6m at its highest point at the rear.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 16th February 2018:

Drawing No: OR-01 – Location And Block Plan

Drawing No: OR-02 – Existing Plans & Elevations

Drawing No: OR-03 – Proposed Plans And Elevations

The agent submitted the following amended drawings, received on 18th April 2018:

Drawing No: OR-01R1 – Location And Block Plan

Drawing No: OR-02 – Existing Plans & Elevations

Drawing No: OR-03R1 – Proposed Plans And Elevations

Drawing No: OR-04 – Proposed Elevations & Section

The agent submitted the following amended drawing, received on 23rd April 2018:

Drawing No: OR-03R2 – Proposed Plans And Elevations

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection subject to informatives.

Properties at 114 and 118 Overdown Road and 8 Rissington Close were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The applicant was advised in an email dated 4th April 2018 that in principle a two storey side extension was acceptable however there was concerns with regards to the design of the proposal and the impact on the neighbouring property due to the levels at the site.  Amended plans were submitted on 18th April 2018 and a further amended plan (to overcome inaccuracies in the height of the garage roof) on the 23rd April 2018.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal meets general policy and guidance.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered harmful in terms of visual dominance or overbearing effects.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

Informatives relating to:

- Positive and proactive

- Terms and conditions

- Highways

- Building regulations

5.	Construction and demolition

6.	Encroachment

7.	Do not damage the verge