# DELEGATED APPLICATION REPORT

Address: 138 Upper Woodcote Road,

Caversham,

Reading

RG4 7LD

Application No: 180167

Proposal: Single storey rear extension. Two storey side extension. New Pitched and flat roof area.

Applicant: Mr & Mrs P Vallender

Date received (valid from): 26/01/2018

Target Decision date: 23/03/2018

Case officer: Tom French

1. Site description and Surrounding Area

The application site contains a semi-detached two storey dwellinghouse located on the north side of Upper Woodcote Road between Woodcote Way and Little Woodcote Close, opposite Knowle Close. The property has an existing two storey flat roof extension at the rear. The surrounding area is predominantly residential and characterised by large detached and semi-detached properties.

2. The Proposal

A householder application for planning permission is sought for a single storey rear extension and a two storey side extension. The rear extension would be 2.7m deep and 5.9m wide. The rear extension would have a flat roof with a single, centrally located, roof light and would be set back 0.9m from the boundary.

The side extension would adjoin the rear extension and also be built to 0.9m of the boundary. It would protrude from the existing dwelling by 3.1m and have a length of 6.5m. The ridgeline of the roof of the proposed side extension would be set down 0.1m from the existing ridgeline. One window is proposed in the side elevation of the proposed extension and would be at a height of 1.7m.

3. Consultation

Neighbours consultation:

No.s 136a and 140 Upper Woodcote Road and no. 1 Woodcote Way have been notified of the application and a site notice has also been displayed at the site from the 2nd February 2018.

No responses have been received.

Ecology:

RBC Ecology Consultations sent the following consultation:

The application site comprises a semi-detached house where it is proposed to construct a two-storey side and single-storey rear extension. The bat survey report (Eaves Ecology, Ref: 17-016-IBA, October 2017), has been undertaken to an appropriate standard and concludes that the risk of the works adversely affecting roosting bats is minimal. This is because the building has very few features potentially suitable for use by roosting bats and no bats or signs of bats were observed during the survey. As such, there are no objections to this application on ecological grounds.

Transport:

RBC Transport sent the following consultation:

The proposal is acceptable in Transport terms in that it will retain the parking provision on the driveway which complies with the requirements within the Council’s adopted Parking Standards and Design SPD.

There are no objections to this proposal subject to the conditions and informatives stated below.

4. Plans and Documents Considered

The Council, in reaching its recommendation, has taken into consideration the following plans & documents:

Drawing No.:

- 3453/200 A – Block & Location Plan

- 3453/201 D – Proposed Floor Plans

- 3453/202 D – Proposed Elevations

- 3453/01 – As Existing Floor Plans

- 3453/02 – As Existing Elevations & Section

As received: 29/01/2018

5. Relevant Planning History

28/04/2017 – 170682 – HOU – Alterations and extensions to dwelling to include 2 storey side extension and single storey rear extension with further first floor rear extension and new pitched roof. Removal of first floor bay window to front and new pitched roof added to ground floor bay. – Application Withdrawn

04/08/2017 – 171346 – HOU – 2 storey side extension and single storey rear extension. New Pitched roof to ground and first floor rear extension. Removal of first floor bay window and new pitched roof added to ground floor bay. Resubmission of 170682. – Application Refused

10/11/2017 – 172010 – PRE – Two storey side extension with single storey rear extension, alterations to rear facing bay and new porch to side – Observations Sent

# 6. Equality Act 2010:

# In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

7. Relevant Planning Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  The Development Plan for Reading relevant to the application site comprises the Reading Local Development Framework ‘Core Strategy’ 2008 (Altered 2015) and ‘Sites and Detailed Policies Document’ 2012 (Altered 2015).

The ‘National Planning Policy framework’ (‘’NPPF’’) 2012 states clearly that its content is to be a material consideration in the determination of applications.  The ‘NPPF’ states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Core Strategy and Sites and Detailed Policies Document) according to their degree of consistency with the ‘NPPF’ (the closer the policies in the plan to the policies in the ‘NPPF’, the greater the weight that may be given). Accordingly, the ‘NPPF’ and the following development plan policies and supplementary planning guidance are relevant:

# 8. Relevant Policies

The application has been assessed against the following policies:

National

National Planning Policy Framework;

National Planning Policy Guidance;

RBC Local Development Framework

Core Strategy 2008 (Altered 2015)

CS7	Design and the Public Realm

Sites and Detailed Policies Document 2012 (Altered 2015)

DM4 (Safeguarding Amenity)

DM9 (House Extensions and Ancillary Accommodation)

DM10 (Amenity Space)

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

9. Appraisal

Design, impact on the host dwelling, character of the area and street scene

In terms of design and scale the proposal is considered to be in keeping with the street scene along Upper Woodcote Road.

The side element of the proposed extension would be set back 3m from the existing front elevation and the roof would be set down from the existing ridgeline. Further, the proposal would extend less than half the width of the original dwelling and be made with material to match existing. As such, it would respect the character and design of the host dwelling.

The proposed rear extension would protrude 3.0m from the existing rear elevation. With a flat roof 3m in height it is considered to be in keeping with the character of the existing extension in design, materials used and proportion.

The host dwelling would retain amenity space to the rear elevation which is considered adequate for a dwelling of this size and relative to the character of amenity spaces to surrounding dwellings.

The proposal would accord with Policies CS7, DM9 and DM10.

Impact on neighbouring amenity

The proposed rear element would be single storey and would be built 3.8m from the boundary with no.140 Upper Woodcote Road. Given the distance from the boundary with no.140 and the single storey nature of the extension, the proposal would not result in any significant material loss of amenity at no.140 in terms of loss of light or overbearing impact.

In relation to No.136a the extension would be offset a distance of 0.9m from the boundary and the roof is fully hipped to help minimise the impact. Whilst visible to the occupiers of this property, given the above, it would not result in any significant overbearing impact. Whilst the proposals would just breach a 45 degree angle to the first floor front dormer, given that this window has a good southerly outlook, this infringement is not considered to be harmful.

No first floor windows are proposed on the flank elevation facing this property and it is noted that there are no first floor windows on the flank elevation of No.136a. As such it is considered that the proposal would have no adverse effects in terms of privacy or overlooking and is in accord with Policies DM4 and DM9.

Recommendation

Grant planning permission, subject to the following conditions:

Conditions

- The development to which this permission relates must be begun not later than the expiration of three years beginning with the date on which the permission was granted.

Reason: to prevent an accumulation of unimplemented planning permissions, and in accordance with Section 91 of the Town & Country Planning Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

- The materials to be used in the construction of the external surfaces of this development shall match those of the existing building.

Reason: To ensure that the new development appears in keeping with the existing building, in the interests of the character of the area, in accordance Policy DM9 of the Sites and Detailed Policies Document 2012 (altered 2015)

- The development hereby permitted shall be carried out in accordance with the following approved plans:

Drawing No.:

- 3453/200 A – Block & Location Plan

- 3453/201 D – Proposed Floor Plans

- 3453/202 D – Proposed Elevations

As received: 29/01/2018

Reason:  because permission is granted in respect of the development as detailed in the approved plans only

- The roof area of the extension hereby permitted shall not be used as a balcony, roof garden or similar amenity area without the grant of further specific planning permission from the Local Planning Authority.

Reason: To safeguard the amenities of the adjoining premises from overlooking and loss of privacy in accordance with Policy DM 4.

- The extension hereby permitted shall not be occupied until all vehicle parking space(s) have been provided in accordance with the local plan. The space(s) shall be kept available for parking at all times thereafter.

Reason: To ensure that the development is provided with adequate parking facilities to meet the needs of future occupiers and reduce the likelihood of roadside parking which could be a danger to other road users to accord with Policy CS24 and Policy DM12

Informatives

- Your attention is drawn to the terms and conditions of this permission. Any development which is carried out but which differs materially from the approved plans and details, or does not comply with any condition(s) or planning obligation(s) attached to the permission may result in the Council taking action to remedy the breach of planning control. If you are in any doubt please contact the Council.

- The applicant’s attention is drawn to Building Regulations requirements. Please contact Building Control on 0118 937 2449 for advice.

- The construction (and demolition) of the development will be subject to environmental health law and the applicant is referred to:

http://beta.reading.gov.uk/environmentalprotection

- The decision to grant planning permission follows the positive and proactive consideration of the issues raised by this planning application and issuing the decision within the statutory time period.

- i) The attention of the applicant is drawn to Section 59 of the Highways Act 1980, which enables the Highway Authority to recover expenses due to damage casued by extraordinary traffic.

ii) Any works affecting the Highway shall be in accordance with Reading Borough Council’s document ‘Guidance Notes for Activities on the Public Highway within the Borough of Reading’. The applicant should note that compliance with this document is mandatory and licences to work on the Highway will only be issued if the requirements contained within it are met. A copy can be obtained from the Council’s website.

- You are advised to avoid the use of the grass verges for development related activity, e.g. Storage of materials, due to the resulting potential harm to important street trees.