DELEGATED APPLICATION REPORT 				Target Date:	09/04/2018

Application No: 180241/HOU

Address: 2 Littlecote Drive, Reading

Proposal: Two storey side and rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property with a detached garage to the rear.  The property is located in a cul-de-sac position and there are examples of other side and rear extensions at neighbouring properties.  The application site is at a lower level than the neighbouring property at 3 Littlecote Drive.  The surrounding area is predominantly residential with similar style properties.

Proposal:

The proposal involves the demolition of the existing garage and the construction of a two storey side and rear extension.

The proposed side extension measures 1.7m in width and extends 10.3m in depth.  The proposed side extension is set back from the existing front elevation of the property by 0.7m and has a cat slide roof that is 1.6m lower than the main roof.  The proposal is 0.7m from the boundary with 3 Littlecote Drive.  3 Littlecote Drive is at a higher level than the application property and has an existing single storey side and rear extension.  A first floor side window is proposed and this will be conditioned to be obscure glazed and fixed up to 1.7m in height above internal floor level.  The proposed side extension wraps around the rear of the existing property and extends 3.5m in depth.  The proposed rear extension is 3.5m from the boundary with 1 Littlecote Drive.

The materials consist of brick and plain tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 7th February 2018:

Drawing No: PL-01 - Location Plan

Drawing No: PL-02 – Existing And Proposed Block Plans

Drawing No: PL-03 - Existing And Proposed Floor Plans

Drawing No: PL-04 – Existing Elevations

Drawing No: PL-05 – Proposed Elevation

The agent submitted the following information, received on 16th February 2018:

Bat Survey Report, by ethos Environmental Planning, dated February 2018

The agent submitted the following amended drawing, received on 22nd March 2018:

Drawing No: PL-02 Rev A – Existing And Proposed Block Plans

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection subject to a condition and informatives.

Ecology raised no objection following the submission of a Bat Survey.

Properties at 1 and 3 Littlecote Drive and 80 Berkeley Avenue were consulted and no comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposed cat slide roof does not match the roof design of the existing property the proposal is subservient to the main property and meets general policy and guidance.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, although the orientation of the properties means there may be some loss of direct light to the side of 3 Littlecote Drive, the application site is at a lower level than this neighbouring property and the proposed roof height is significantly lower than the main roof.  The proposal also does not encroach on a line taken at 45 degrees from the middle of the closest habitable windows at both 1 and 3 Littlecote Drive and the proposal is therefore not considered to significantly reduce the amount of light to neighbouring properties.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, the first floor window to the side elevation of the extension will be conditioned to be obscure glazed and fixed up to 1.7m in height above internal floor level.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the neighbouring properties at 1 and 3 Littlecote Drive will notice the additional height and depth of the proposal the proposal meets general policy and guidance and as such the proposal is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials (to match)

- Approved plans

- First floor side window obscure glazed and fixed

- Vehicle parking

Informatives relating to:

- Positive and proactive

- Highways

- Terms and conditions

- Building Regulations

- Construction and demolition

- Encroachment and Party Wall

- Do not damage the verge