DELEGATED APPLICATION REPORT 			Target Date: 30/03/2018

Application No: 180216/HOU

Address: 167 Whitley Wood Road, Reading

Proposal: Single storey rear to side extension with pitched roof

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The site comprises a two storey semi-detached property.  The neighbouring property at 165 Whitley Wood Road has a single storey rear extension and the adjacent property at 169 Whitley Wood Road has a single storey side extension.  The surrounding area is predominantly residential.

Proposal:

The proposal involves a single storey side and rear extension.  The proposed side extension is set back from the existing front elevation of the property by 3.7m and measures 2m in width.  The proposed side extension wraps around the rear of the property and extends 4m in depth. The proposal has a hipped roof with a flat top and measures 2.5m in height to eaves level and 3.4m in height at its highest point.  The proposal extends along the boundary with the neighbouring property at 165 Whitley Wood Road and is 1m from the boundary with 169 Whitley Wood Road.

The materials consist of pebble dash render and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 2nd February 2018:

Drawing No: SLP01 – Site Location Plan

Drawing No: E01 – Existing Plans

Drawing No: E02 – Existing Elevations

Drawing No: E03 – Existing Elevations

Drawing No: BP01 – Block Plan

Drawing No: P01_B – Proposed Plans

Drawing No: P02_B – Proposed Plans

Drawing No: P03_B – Proposed Elevations

Drawing No: P04_B – Proposed Extension

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection subject to a condition for a plan to be submitted showing vehicle parking spaces.  Officer Note: the front of the property has hardstanding for 2 cars and therefore this condition is not applicable.

Properties at 165 and 169 Whitley Wood Road and 1 Woodside Way were consulted.  No comments have been received.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM12: Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposals they are not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

Informatives relating to:

- Positive and proactive

- Highways

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment

6.	Do not damage to verge