Delegated Application Report 				          Target Date: 27/03/18

Address: 3 Park Crescent

Application No: 180172/CLP

Proposal: Loft conversion with side box dormers and a flue

Case Officer: Tom Hughes

Introduction and Proposal

The application site is a bungalow with a pitched roof. The dwelling is of a style and design consistent with those in the surrounding area.

The proposal is for a loft conversion with the provision of side box dormers and a flue to the roof of the previously approved single storey rear extension. The dormers would be located to each sideward roof slope, extending to a length of 8.4m, with a maximum height of 1.9m, with a maximum projection of 2.9m. Each dormer would be set down from the existing ridgeline by 0.4m, and set back from the existing eaves by 0.45m. Each dormer would have a volume of 23.14 cubic metres, a total additional volume of 46.28 cubic metres. Each dormer would have three windows, to the sideward elevations, obscure-glazed and non-opening to 1.7m above the floor level of the room in which they are located. Materials have been selected to be of a similar appearance to those of the existing dwelling.

To the south-eastward corner of the previously approved single storey rear extension would be located a flue for a log burner. The flue would have a height of 1.9m, approximately 1m below the ridgeline of the existing roof.

Plans Submitted

The agent supplied the following, received on 29th January 2018:

Drawing No: DN68/250 – Site Location and Block Plan

Drawing No: DN68/251 – Proposed Floor Plans

Drawing No: DN68/252 – Proposed Elevations and Section

Relevant Planning History

172223/HPA – Single storey rear extension – Prior Approval Notification – Approval 19/01/18

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2017

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed loft conversion with side box dormers and a flue must be assessed to ensure that they comply with the conditions of Class B and Class G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relate to ‘additions etc to the roof of a dwellinghouse’ and ‘chimneys, flues etc on a dwellinghouse’.

The relevant conditions of the GPDO for a loft conversion with side box dormers and a flue, which is not listed or in a conservation area, are as follows:

Loft conversion with side box dormers

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, n/a or (ii) 50 cubic metres in any other case; complies, the cubic content of the resulting roof space would exceed the cubic content of the original roof space by 46.28 cubic metres

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; complies, subject to Class G or

B.1 (f) the dwellinghouse is on article 2(3) land. complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards, and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

Flue

G.1 Development is not permitted by Class G if:

G.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P, or Q of Part 3 of this Schedule (changes of use); complies

G.1 (b) the height of the chimney, flue or soil and vent pipe would exceed the highest part of the roof by 1 metre or more; complies or

G.1 (c) in the case of a dwellinghouse on article 2(3) land, the chimney, flue or soil and vent pipe would be installed on a wall or roof slope which (i) fronts a highway, complies and (ii) forms either the principal elevation or a side elevation of the dwellinghouse. Complies

The rear extension shown on the plans complies with Class A and is therefore also PD.

Recommendation

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class A, Class B and Class G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on 29th January 2018:

Drawing No: DN68/250 – Site Location and Block Plan

Drawing No: DN68/251 – Proposed Floor Plans

Drawing No: DN68/252 – Proposed Elevations and Section

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed loft conversion with side box dormers and a flue. Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition

Bats