DELEGATED APPLICATION REPORT 					    Target Date: 11/10/18

Application No: 181397/HOU

Address: 270 Wensley Road

Proposal: Part one, part two storey side and rear extension

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site is a two storey semi-detached dwelling with a pitched roof. The dwelling benefits from a single storey side extension with a steep mono-pitch roof. Photovoltaic panels are located to the rearward roof slope. The surrounding area is predominantly residential, comprised of terraced and semi-detached dwellings of a similar style and design. The adjoining dwelling at 268 Wensley Road has not been extended to the rear. The adjacent dwelling at 272 Wensley Road benefits from a two storey side and rear extension.

Proposal

The proposal is for a part one, part two storey side and rear extension. The footprint of the existing extension would be retained. At the ground floor the proposal would project to a depth of 3.26m beyond the original rear elevation, extending across the full width of the dwelling. At the first floor, the proposal would extend above the existing side extension and across the full width of the rear of the dwelling, at the same depth as the ground floor element. The first floor element would be set back from the principal elevation by 1m.

The element of the proposal beyond the original side elevation would have a pitched roof with a maximum height of 6.2m and a height to eaves of 4.9m. This element would be set down from the original ridgeline by 0.2m. Forward of the two storey element, the single storey element would have a mono-pitch roof with a maximum height of 3.1m and a height to eaves of 2.6m. The element of the proposal projecting beyond the original rear elevation would have a mono-pitch roof, adjoining the rearward slope of the existing roof. This element would have a maximum height of 5.6m and a height to eaves of 5m. Three roof lights would be located to the rearward roof slope of the original dwelling.

To the front elevation, windows would be located at the ground and first floors. To the rear elevation, two large windows and a set of double doors would be located at the ground floor with five windows located at the first floor. Materials have been selected to match those of the existing dwelling.

Plans Considered

The agent supplied the following, received on 09/08/18:

Drawing No: WR 001 – Site Location

Drawing No: WR 009 – Proposed Site Plan

Drawing No: WR 01 – Existing and Proposed Ground Floor Plans

Drawing No: WR 02 – Existing and Proposed First Floor Plans

Drawing No: WR 07 – Existing and Proposed Roof Plans

Drawing No: WR 03 – Existing and Proposed Front Elevations

Drawing No: WR 04 – Existing and Proposed Rear Elevations

Drawing No: WR 06 – Existing and Proposed Side Elevations

Relevant Planning History

None.

Consultation Responses

Neighbouring owners and occupiers at 268 and 272 Wensley Road were consulted by letter. One letter of representation was received with regard to the following:

- The proposal would result in a loss of light to the side of 272 Wensley Road

- The proposal would extend to the boundary with 272 Wensley Road

Transport Development Control used historic photographs to identify the site and noted a low boundary wall the site frontage. Associated concerns were raised with regard to the widening of the access to facilitate the parking of three vehicles to the site frontage. Transport Development Control required revised plans illustrating the dimensions of the proposed access and any alterations to the site frontage. Officer note: at the time of the officers site visit, part of the previous front boundary treatment had been removed, to facilitate dual access to hardstanding to the site frontage. Accordingly, the existing site could accommodate the parking of three vehicles, with no changes proposed as part of this application

Ecology advised that the site is surrounded by habitat of excellent suitability for use by foraging and commuting bats. Rough grassland, a stream and trees are located to the rear of the property, a Local Wildlife Site (Fobney and Kennet Valley Meadows) is located 115m south of the site and broadleaved woodland is located 185m to the north. Considering the location of the house, the fact that the proposal would affect the roof and the extent of the works, there is a risk that the proposal may affect roosting bats and a bat survey would need to be provided before the application is determined. If a bat survey is not provided, the application would need to be refused on the grounds of insufficient evidence being provided to determine the likely impacts of the proposal on bats (which are a protected species and a material consideration in the planning process).

Summary of discussions with applicant/agent

The agent was advised on 05/10/18 of a number of concerns held with the proposal, which was subject for refusal.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 - Design and the Public Realm

CS36 – Biodiversity and Geology

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

Design and impact on the character of the surrounding area

Policy DM9 of the Sites and Detailed Policies Document states that an extension to a house should respect the character of the house in terms of scale, location, materials and design. The extension should also respect the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line. Policy CS7 of the Core Strategy also states that all development must be of a high design quality that maintains and enhances the character and appearance of the area. The Council’s Design Guide to House Extensions states that planning permission will not normally be granted for rear extensions that are longer than 4m, and that they should be located as far away from side boundaries as possible. The acceptability of rear extensions will be derived from the particular circumstances of the application site. Two storey side extensions should normally be designed to be smaller in scale than the main house, set down from the existing roof line and set back from the front elevation.

The proposed extension represents a substantial enlargement to the original dwelling, Due to its unrelenting width, scale and bulk the proposal lacks subservience to the original dwelling. The set back and set down of the first floor element beyond the original side elevation is minimal, and the depth of the proposal along the boundary beyond the original side and rear elevations obscures the form of the original dwelling. It is difficult to comprehend the proposed roof design from the plans supplied with the application, but the shallow mono-pitch roof form proposed to the rear is not considered to be a sympathetic addition. Due to its width and proximity to the side boundary, there is also a terracing/gapping issue that would occur. As such, the proposal is contrary to Policy CS7 of the Core Strategy, Policy DM9 of the Sites and Detailed Policies Document and the Council’s Design Guide to House Extensions.

Impact on neighbours

Due to its height and depth directly along the boundary, the proposal is considered excessive, appearing unneighbourly, visually dominant and presenting an overbearing effect to the residents of the neighbouring dwellings, particularly 268 Wensley Road. The proposal would also encroach on a line taken at 45 degrees from the midpoint of the nearest habitable window of the adjoining dwelling at 268 Wensley Road, which would reduce the amount of natural light currently achieved by this neighbour. As such, the proposal is contrary to Policy DM4 of the Sites and Detailed Policies Document and the Council’s Design Guide to House Extensions.

Bats

As per the consultation response from Ecology, there is a risk that the proposal may affect roosting bats. All species of bats receive protection under UK law and it is a criminal offence under the Wildlife and Countryside Act 1981 (as amended) and the Conservation of Habitats and Species Regulations 2010 (The Habitat Regulations), to deliberately or recklessly destroy or damage their roosts, or to disturb, kill or injure them without first having obtained the relevant licence for derogation from the regulations from the Statutory Nature Conservation Organisation (the SNCO – Natural England in England).

Paragraph 99 of the government Circular 06/05: Biodiversity and Geological Conservation – Statutory Obligations and Their Impact Within The Planning System states that:

“It is essential that the presence or otherwise of protected species, and the extent that they may be affected by the proposed development, is established before planning permission is granted, otherwise all relevant material considerations may not have been addressed in making the decision. The need to ensure ecological surveys are carried out should therefore only be left to coverage under planning conditions in exceptional circumstances with the result that the surveys are carried out after planning permission has been granted.”

Policy CS36 of the Core Strategy requires that developments retain, protect and incorporate features of biodiversity or geological interest (including protected species and their habitats) found within the application site into their schemes. In this case because neither the presence or otherwise of protected species, nor the extent to which they may be affected has been established, the application is not in accordance with the above planning policy in relation to biodiversity.  This is a further reason to refuse planning permission.

RECOMMENDATION

REFUSE

Informatives relating to:

- Refused plans