Delegated Application Report 				          Target Date: 03/10/17

Address: 42 Water Road

Application No:

Proposal: Rear box dormer and alteration to fenestration

Case Officer: Tom Hughes

Introduction and Proposal

The application site is a two storey mid-terraced property with a pitched roof. A two storey gable projects to the rear. The surrounding area is predominantly residential.

The proposal is for a loft conversion with the provision of a rear box dormer and an alteration to the existing fenestration. The volume of the enlarged part of the roof would exceed the volume of the existing roof by 20.1 cubic metres. The dormer would extend at a width of 3.8m, and would project to a maximum depth of 4.1m. The dormer would have a maximum height of 2.7m, set down from the existing ridgeline by 0.2m and set back from the existing eaves by 0.2m. A window and a Juliet balcony would be located to the rear elevation. Materials have been selected to match those of the existing property.

The existing window to the first floor side elevation of the gable end would be removed. A further first floor window would be inserted to this elevation. The window would be obscure-glazed and non-opening to 1.7m above the floor level of the room in which it is located.

Plans Submitted

The agent supplied the following, received on 8th August 2017:

Drawing No: 17.42WR.SITE.P01 – Site Location and Block Plan

Drawing No: 17.42WR.P01 – Existing Floor Plans, Rear Elevation and Section

Drawing No: 17.42WR.P02 – Proposed Floor Plans, Rear Elevation and Section

The agent provided confirmation of the materials to be used and of the proposed first floor window in an email dated 4th September 2017.

Relevant Planning History

None.

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2017

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed rear box dormer and alteration to fenestration must be assessed to ensure that it complies with the conditions of Class B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘additions etc to the roof of a dwellinghouse’.

The relevant conditions of the GPDO for a rear box dormer and alteration to fenestration, which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, complies, the cubic content of the resulting roof space would exceed the cubic content of the original roof space by 20.1 cubic metres or (ii) 50 cubic metres in any other case; n/a

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; complies or

B.1 (f) the dwellinghouse is on article 2(3) land. complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards, and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

Recommendation

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed rear box dormer and alteration to fenestration.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on 8th August 2017:

Drawing No: 17.42WR.P02 – Proposed Floor Plans, Rear Elevation and Section

Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition

Bats