Delegated Application Report 				          Target Date: 07/03/18

Address: 28 St Peters Road

Application No: 172331/CLP

Proposal: Single storey side extension

Case Officer: Tom Hughes

Introduction and Proposal

The application site is a two storey semi-detached property with a hipped roof. A garage adjoins the north-westward side elevation. A single storey projection extends to the rear. The plot splays slightly to the rear. The site is located within an Article 4 area controlling the change of use from a C3 dwelling house to a C4 house in multiple occupation. During the course of the application the agent supplied information to show that the property was in C4 use prior to the introduction of the Article 4 area (designated 16/05/13), and continues to be so.

The proposal is for a single storey side extension. The existing garage would be demolished to facilitate the proposal. The proposal would extend at a width of 2.4m, flush to the principal elevation. The proposal would project to a total depth of 9.6m, flush to the rear elevation of the single storey rear projection. The proposal would follow the splay of the north-westward site boundary, with a maximum width of 3m at the rear. The proposal would have a flat roof with a maximum height of 2.9m. Windows would be located to the front and rear elevations. Materials have been selected to match those of the existing dwelling.

Plans Submitted

The agent supplied the following, received on 29/12/17:

Drawing No: StP:002 – Site Location

Drawing No: StP:003 – Existing and Proposed Floor Plans

Drawing No: StP:004 – Existing Elevations

Drawing No: StP:005 – Proposed Elevations

Subsequently, the agent supplied the following, received on 10/01/18:

Drawing No: StP:001 – Block Plan

The agent was advised on 23/02/18 that the proposed development would constitute permitted development. However, as the supplied plans include the label ‘house in multiple occupation’ the Local Planning Authority would need to be certain that this is in fact the current lawful use of the property. There was no current planning record of the change of use from C3 to C4, so the lawful use of the property is as a C3 dwellinghouse. The Local Planning Authority need to be certain that the issue of a Lawful Development Certificate for a proposed development would not also imply the change of use. The agent was advised to remove the label so that the application could be determined.

Subsequent discussion with the agent followed, who advised that evidence could be provided to show that the property had been is use as a C4 house in multiple occupation, prior to the designation of the Article 4 area.

Subsequently, the agent supplied the following, received on 13/03/18:

Tenancy Agreement, dated 11/03/11 – period 01/07/11 to 30/06/12

Tenancy Agreement, dated 28/01/12 – period 01/07/12 to 30/06/13

Tenancy Agreement, dated 26/02/13 – period 01/07/13 to 30/06/14

Tenancy Agreement, dated 28/05/14 – period 01/07/14 to 30/06/15

Tenancy Agreement, dated 06/12/14 – period 01/07/15 to 30/06/16

Tenancy Agreement, dated 08/12/15 – period 01/07/16 to 30/06/17

Tenancy Agreement, dated 28/11/16 – period 01/07/17 to 30/06/18

Relevant Planning History

071314/FUL – Two storey side extension and single storey rear extension – Permitted 02/08/07

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

Section 10 of the Planning and Compensation Act 1991 established a procedure that enables anyone who wishes to do so, to apply to the local planning authority to determine whether a proposed use or operation, or an existing operational development or an existing use of land, or any other matter constituting a failure to comply with any condition or limitation subject to which planning permission has been granted, is lawful, and is so, be granted a certificate to that effect.

A Lawful Development Certificate is a legal document stating the lawfulness of past, present or future development. If granted by the local planning authority, the certificate means that enforcement action cannot be carried out to the development referred to in the certificate. However, the certificate will not protect from enforcement action by the planning authority if the specified use or development is then changed ‘materially’ without a planning application for it.

The certificate is not planning permission. The planning merits of use, operation or activity in the application are not relevant. The issue of a certificate depends entirely on factual evidence about the history and planning status of the building or other land and the interpretation of any relevant planning law or judicial authority. The responsibility is on the applicant to provide evidence to support the application.

The NPPG states that ‘The applicant is responsible for providing sufficient information to support an application. In the case of applications for existing use (section 191 of the Town and Country Planning Act 1990), if a local planning authority has no evidence itself, nor any from others, to contradict or otherwise make the applicants version of events less than probable, there is no good reason to refuse the application, provided the applicants evidence alone is sufficiently precise and unambiguous to justify the grant of a certificate on the balance of probability’.

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2017

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The agent has supplied evidence dating to 2011, showing the use of the property as a house in multiple occupation - Use Class C4. Officers are satisfied that the evidence provided is sufficient to show that the property has been occupied by no more than 4 tenants at any one time since July 2011, and continues to be so. Having regard to the foregoing, it is considered on the balance of probability, the property known as 28 St Peters Road, Reading, RG6 1PA has been in use as a small house in multiple occupation (Use Class C4) prior to the designation of the Article 4 area on 16/05/13.

The proposed must be assessed to ensure that it complies with the conditions of Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to the ‘enlargement, improvement or other alteration of a dwellinghouse’.

The relevant conditions of the GPDO for a single storey side extension, which is not listed or in a conservation area, are as follows:

A.1 Development is not permitted by Class A if:

A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); complies

A.1 (c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse; complies

A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; complies

A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principle elevation of the original dwellinghouse; complies or (ii) fronts a highway and forms a side elevation of the original dwellinghouse; complies

A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, complies or (ii) exceed 4 metres in height; complies

A.1 (g) until 30th May 2019, for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, complies or (ii) exceed 4 metres in height; complies

A.1 (h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres, complies or (ii) be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse; complies

A.1 (i) the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3 metres; complies

A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would – (i) exceed 4 metres in height, complies (ii) have more than a single storey, complies or (iii) have a width greater than half the width of the original dwellinghouse; complies

A.1 (ja) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (e) to (j); complies

A.1 (k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform, complies (ii) the installation, alteration or replacement of a microwave antenna, complies (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, complies or (iv) an alteration to any part of the roof of the dwellinghouse. complies

A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if:

A.2 (a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles; complies

A.2 (b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse; complies

A.2 (c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse; complies

A.2 (d) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (b) and (c). complies

Conditions

Development is permitted by Class A subject to the following conditions:

A.3 (a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the dwellinghouse; complies

A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed; complies and

A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse. complies

Recommendation

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed development would be lawful for planning purposes on the basis of the drawings received on 29/12/17:

Drawing No: StP:003 – Existing and Proposed Floor Plans

Drawing No: StP:005 – Proposed Elevations

Drawing No: StP:001 – Block Plan (received 10/01/18)

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed single storey side extension. Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition