DELEGATED APPLICATION REPORT 				Target Date: 10/10/2017

Application No: 171239/LBC

Address: Woodlands Farm, 59 Luscombe Close

Proposal: Removal of existing modern block work internal wall separating kitchen and dining room and replacement with new oak beams and post

Case officer: Heather Banks

Site Description and Surrounding Area:

The application property is a Grade II listed farmhouse building located on the southern side of Luscombe Close. The historic core of the property is a mid 17th Century rebuilding of an older farmhouse. The kitchen and dining room where the proposed works are to take place form the ground floor of this historic core of the building, including a large fireplace within the dining room. The exposed ceiling beams are also a feature of the property. The exposed beams of the kitchen run in a perpendicular direction to those of the dining room. The gable and room over the porch to the east of the central gable are a large 19th Century extension to the original farmhouse. The second front facing gable to the west of the central core is a much more recent extension, granted permission in 1993.

Proposal:

This application seeks listed building consent for the removal of the internal wall separating the kitchen and the dining room to create a more open plan living area. There is to be a new 150mm deep oak beam over the opening with a centrally located oak post for support. It is proposed that a width of 600mm of the existing wall is to be retained, projecting from each side wall to help to define the two separate spaces of the original property.

It is considered that the existing dividing wall is constructed of modern brick work, given some minor exploratory work and that a secondary staircase along this wall was removed and the door between these two rooms was relocated under the 1993 works to the property.

Documents Considered:

The agent supplied the following documents:

Drawing No: AD/01 rev. D – Existing and proposed details.

Received 6th October 2017

Relevant Planning History:

05-00261-FUL and 05-00262-LBC - "Granny annexe" single storey extension adjoining existing detached garage. Refused 08/06/2005

05-00899-FUL and 05-00900-LBC - "Granny annexe" single storey extension adjoining existing detached garage (Resubmission of 05/00261/FUL and 05/00262/LBC ). Permitted 03/10/2005

08-00848-LBC - Replacement of existing windows and french doors with black mono-frame

aluminium double-glazed units. Refused 29/08/2008

150580-LBC - Retrospective Listed Building Consent for replacement windows and French doors with black mono-frame aluminium double glazed units. Refused 29/08/2015

151920-PREAPP - Replacement of 3 No. French doors and 14 No. windows (listed building

enquiry). Observations sent 17/12/2015

160312-LBC - Replacement of 3 French doors and 14 windows from aluminium (unauthorised) to timber framed, to dwelling house. Permitted 28/04/2017

161147-NMA - Details required by Condition 2 of listed building consent ref: 160312 dated 28 April 2016 re Replacement of 3 French doors and 14 windows to dwellinghouse. Conditions Discharged 14/07/2016

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

LEGAL AND PLANNING POLICY CONTEXT

Section 16(2) of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires the local planning authority to have special regard to the desirability of preserving the listed building or its setting or any features of special interest which it possesses.

National Planning Policy and Guidance

National Planning Policy Framework (NPPF)

National Planning Practice Guidance

Historic England Good Practice Guide Note Number 2: managing significance in decision-taking

Reading Borough Local Development Framework Adopted Core Strategy (2008) (Altered 2015):

CS7     Design and the Public Realm

CS33	Protection and Enhancement of the Historic Environment

APPRAISAL

Historic Character and Appearance

The proposed changes are located within the historic core of the Listed Building and minimal evidence has been provided regarding the significance of the building or the elements of the building affected by the proposals. Under paragraphs 133 and 134 of the NPPF, any harm to a Listed Building requires a justification in terms of the public benefit which outweigh the harm of the proposals. This has not been provided with the application. As such a site visit was undertaken by the Case Officer and Heritage Officer to better ascertain the significance of the elements of the building affected by the proposals

Following a site visit to the application property by the Case Officer and the Heritage Officer, despite the fact that the wall to be removed appears to be of modern construction, it is considered that, given the proportions of the spaces, the different direction of the ceiling beams in the two spaces and that this is the historic core of the building, it is likely that there has always been a wall in the location of that to be removed. Given the above, it is felt that the entire removal of this wall would cause unacceptable harm to the historic character of the property, changing the original proportions of the space.

However, it is considered that the specifics of this proposal (retaining 600mm of the wall to either side of the opening, inserting a large oak support beam across the new opening and inserting a large oak central supporting column) allows the character of the two separate spaces to be retained whilst allowing greater permeability to accommodate the preferences and demands of modern living. It is considered that this partial removal of the wall is acceptable given that the character of two distinctly different spaces is being retained and the details of the construction of the opening will minimize the impact of the proposal on the remaining existing fabric of the building. The new oak lintel beam is to have a minimum of 200mm bearing on the walls retained of the opening which will give it an appearance in keeping with other lintels within the property. The new beam will also be notched around the existing kitchen ceiling joists allowing the work to be carried out without causing damage to these potentially historic joists.

Conclusion

Given the above, it is considered that the proposed works would respect and preserve the character of this listed building and would be in keeping with Policies CS7 (Design and the Public Realm) and Policy CS33 (Protection and Enhancement of the Historic Environment) of the Reading Core Strategy 2008 (as amended).

RECOMMENDATION:

GRANT Listed Building Consent

Conditions

- The works to which this consent relate must be begun not later than the expiration of three years beginning with the date on which the consent is granted.

Reason: to accord with Section 18 of the Planning (Listed Building and Conservation Areas) Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

Approved Plans

- The works hereby approved shall not be carried out other than in accordance with the submitted application and accompanying details:

Drawing No: AD/01 rev. D – Existing and proposed details.

Received 6th October 2017

Reason: to ensure that the works carried out are fully in accordance with the approved drawings and details in order to protect the character of the listed building, in accordance with Policy CS33 and national policy contained within the NPPF and associated guidance.

- All new external and internal works and finishes, and any works of making good, shall match the existing original fabric in respect of using materials of a matching form, composition and consistency, detailed execution and finished appearance, except where indicated otherwise on the drawings hereby approved.

Reason: In order that the special architectural and historic interest of this Listed Building is safeguarded in accordance with Policy CS33.

- The development hereby permitted shall be carried out in a proper workmanlike manner appropriate to the age and character of the building and using traditional materials and techniques except where the use of modern materials and techniques have specifically been approved by the local planning authority.

Reason: To accord with Reading Borough Council Core Strategy CS33: Protection and Enhancement of the Historic Environment and for the purpose of ensuring that work is appropriate to the special architectural or historic interest of the listed building.

- Unless such work is clearly and specifically referred to on drawings or other documents  approved, no features of architectural or historic interest such as doors, linings, shutters, paneling, cornices, decorative plasterwork, floorboards,  fireplaces, lath and plaster ceilings may be altered, replaced or removed without the written approval of the local planning authority.  Any such features encountered during the work shall be drawn to the attention of the local authority.

Reason: To accord with Reading Borough Council Core Strategy CS33: Protection and Enhancement of the Historic Environment and for the purpose of ensuring that work is appropriate to the special architectural or historic interest of the listed building.