DELEGATED APPLICATION REPORT

Application No: 171124/FUL

Address: 4 Dovecote Road, Reading

Ward: Whitley

Proposal: Proposed one bedroom house

Target Date: 18/09/2017

Case officer: Ralph Chakadya

Site Description and Surrounding Area

- The application site consists of garden and garage (part of a pair) belonging to 4 Dovecote Road. Due to the odd layout, number 4 Dovecote is separated from the site by the rear garden to number 5 Dovecote Road being approximately 20m from site. The house at 4 Dovecote Road was built as part of a scheme granted permission in 1983. This followed two schemes granted permission on appeal. The dwelling is not listed or in a conservation area.

Proposal

- The proposal is for the construction of a one-bedroom house fronting Dovecote Road on the garden to number 4 Dovecote Road and incorporating part of the grass verge along Dovecote Road.

- The garage belonging to number 4 Dovecote Road would be incorporated.

- There is a tree to the front of the site that is considered of amenity value. However, the tree is not protected and at the time of site visit by officers, the applicant had undertaken works to remove main branches of the tree.

- Supporting information

- Design and Access Statement

- Unnumbered Block Plan

- Drawing No.3 Location, Proposed Floor and Elevation Plans

Planning history

- 170773FUL - 2 X 1-bed dwellings on land adjacent. Withdrawn 13/06/2017

- 162169 – Pre-application advice for the construction of a house. The proposal was considered unacceptable.

- 83/TP/539 – 8 no. one and two bedroom 2-storey houses, garages, parking spaces and associated roads and drainage. Granted September 1983

- 82/TP/469 – Construction of estate road, 15 one, two and three bedroomed houses, each with a single garage and parking space. Granted on Appeal

- 82/TP/65 – Construction of estate road, 19 one, two and three bedroomed houses, each with a single garage and parking space. Granted on Appeal

Consultations

RBC Development Control Transport –

- This application consists of the construction of a 1 bedroom detached dwelling on land belonging to 4 Dovecote Road consisting of a garden and a garage which will result in the loss of one car parking space for the existing property.  The Design and Access statement states that No.4 has its own parking area in addition to the garage although this has not been illustrated on submitted plans. Clarification is required to what and where this provision is to ensure it complies with the councils parking standards.

- The site is located within Zone 3, Secondary Core Area, of the Council’s adopted Parking Standards and Design SPD.  Typically these areas are within 400m of a Reading Buses high frequency ‘Premier Route’, which provides high quality bus routes to and from Reading town centre and other local centre facilities.

- In accordance with the adopted Parking Standards and Design SPD the new development would be required to provide parking with 1 on site space for a 1 bedroom dwelling. Plans indicate the provision of a single garage, currently associated with No. 4.  It is recommended that the internal dimensions for a single garage are 7m long x 3m wide to allow easy access to/from the vehicle and sufficient storage.  Dimensions of the garage fall short of the Councils standard. Research shows that less than half the garages are used for parking cars, and that many are used primarily as storage.  It’s therefore likely that the future occupants of the property would not use the garage for parking if it is below the Councils required standards.  However the parking area in front of the garage can accommodate one vehicle without overhanging the carriageway; therefore this would meet the Councils requirement for one parking space for the new development.

- Goggle images indicate that there is a tree in close proximity to the proposed site and therefore guidance and advice will need to be sought from the Natural Environmental Officer with regards to any impact on the tree/tree roots.

- In accordance with the Council’s Parking Standards and Design SPD, a 1 bedroom dwelling house should be provided with 1.0 secure cycle storage space. No provision for cycle storage has been submitted. Revised plans required illustrating provision.

- Details of the bin storage and collection should also be submitted. Bin storage should comply with Manual for Streets and British Standard 5906: 2005 for Waste Management in Buildings to avoid the stationing of service vehicles on the carriageway for excessive periods.

- To be able to fully determine this application clarification is required on the points raised above.

RBC Natural Environment (Trees)

- This is a very small property proposed for a very small plot of land. There will be limited amenity space on site and /or opportunities for new planting.

- There is a significant tree on the highway verge at the front of the property. The scarce mature tree cover in this locality increases the visual significance and importance of the tree at the front of the property considerably.

- I note that the tree has already been cut back to improve light for the neighbouring garden. On completion of this development the new property and small yard space to the front will be heavily shaded. Shedding of small scale debris from the tree will be a continual nuisance and there is likely to be further pressure on the tree to be cut back once the new dwelling is occupied.

- To date I have not received any information on the tree or plans which show the proximity of the tree and its canopy in relation to the new dwelling. As such, my assessment of the site is somewhat hypothetical. The dwelling will be located forward of the existing boundary wall in part and is likely to result in considerable root severance as a result. Although the canopy has been cut back, the branches are likely to come into direct contact with the property as they continue to grow. Furthermore, drainage and services will require trenching within the RPA of the tree.

- The applicant will need to provide an arboricultural report in accordance with BS:5837 2012 so that the impact of the development on the tree can be ascertained. This will include a survey and impact assessment in the first instance; an arboricultural method statement and protection plan if the application is considered potentially viable.

RBC Environmental Health

- No objection subject to conditions and informatives to control noise, dust, working hours and bonfires

Public consultations

- Twelve letters objecting to the proposal were received from consulted neighbours highlighting the following issues:

- Overdevelopment of the site

- Proposal being out of character

- Lack of parking

- Loss of trees

- Overlooking

- Loss of garden amenity space

- Inadequate outdoor garden amenity space

- Being overbearing

- Loss of light and overshadowing

- Profit motive by the developer

- Applicant is not local

- Devalue properties

- As highlighted within the history section above, pre-application advice was provided informing that the proposal to erect a dwelling on the application site was unacceptable.

Relevant Planning Policy and Guidance

- Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  However the NPPF does not change the statutory status of the development plan as the starting point for decision making (NPPF paragraph 12).

- The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework (NPPF)

Local Development Framework Core Strategy (2008 and altered 2015)

- CS1 (Sustainable Construction and Design)

- CS2 (Waste Minimisation)

- CS4 (Accessibility and the Intensity of Development)

- CS5 (Inclusive Access)

- CS7 (Design and the Public Realm)

- CS9 (Infrastructure, Services, Resources & Amenities)

- CS14 (Provision of Housing)

- CS15 (Location, Accessibility, Density and Housing Mix)

- CS24 (Car/Cycle Parking)

- CS38 (Trees, Hedges and Woodlands)

Reading Borough Local Development Framework: Sites and Detailed Policies Document (SDPD) (2012 and altered 2015)

- SD1 (Presumption in Favour of Sustainable Development)

- DM1 (Adaptation to Climate Change)

- DM3 (Infrastructure Planning)

- DM4 (Safeguarding Amenity)

- DM6 (Affordable Housing)

- DM10 (Private and Communal Outdoor Space)

- DM11 (Development of Private Residential Garden Land)

- DM12 (Access, Traffic and Highway-Related Matters)

Local supplementary planning guidance:

- The Council’s ‘Revised Sustainable Design & Construction SPD’ (July 2011)

- The Council’s ‘Parking Standards and Design’ SPD (Oct 2011)

- The Council’s ‘Revised SPD on Planning Obligations under Section 106 of the Town and Country Planning Act 1990’ (2013)

- The Council’s ‘Affordable Housing’ SPD (2013)

Appraisal

- Main Issues

- Principle of the development

- Design and appearance

- Residential amenity

- Transport issues

- Trees and landscaping

- Representations

- Community infrastructure levy

- Affordable housing contributions

Principle of development

- As outlined within the proposals section above, despite the odd layout, the application site constitutes the outdoor garden to 4 Dovecote Road. It should be noted that the cluster of 4 houses (4-7 Dovecote Road) are all provided with outdoor garden amenity space within the odd layout. The proposal would therefore constitute development of garden land. The principle of the development would need to be assessed in terms of the requirements of the National Planning Policy Framework (NPPF) paragraph 53 that seeks to ensure that policies are set out to resist inappropriate development of residential gardens.  Also, at local level, the Council’s LDF SDPD policy DM11 establishes a number of criteria against which applications for the development of private residential gardens would need to be assessed. Council’s LDF SDPD policy DM10 seeks to ensure that adequate private and communal outdoor space is retained in terms of development proposals. Core Strategy policy CS7 seeks to ensure development that enhances the character and appearance of the area.

- The introduction of a new dwelling would result in the total loss of outdoor garden amenity space to 4 Dovecote Road, a situation considered to be detrimental to the current and future occupiers of 4 Dovecote Road. Due to the small nature of the plot size and limited space around the built form of the proposed dwelling, the proposed dwelling would fail to retain adequate space to provide functional outdoor amenity space for the new dwelling contrary to policy DM10.

- The existing garages adjoining the site are single storey so as small scale ancillary buildings they appear acceptable at the rear of existing dwellings 4-5 Dovecote Road. The proposed two-storey dwelling with limited spacing around it within the plot would introduce built form very different in character to the ancillary nature of the existing garages.  In addition, the footprint of the proposed dwelling would incorporate part of the grass verge along Dovecote Road, demonstrating the inadequacy of the plot size to accommodate an independent dwelling. This green verge is part of the character of Dovecote Road and its loss would be detrimental to the visual amenities of the area. Due to the small plot size, the resultant built form would be at odds with the established urban grain on this part of Dovecote Road.  As such the proposed development would constitute overdevelopment of the site that would be detrimental to the appearance and character of the area.

- The proposed development would therefore be contrary policies DM10 that seeks to ensure adequate provision of outdoor amenity space; criterion 1 of policy DM11 that requires proposals to make a positive contribution to the character of the area in terms of the relationship of the proposal to the existing built form and spaces around buildings within the surrounding area; criterion 2 of DM11 that seeks to ensure that the application site provides a site of adequate size and dimensions to accommodate the development proposed in terms of the setting and spacing around buildings; policy CS7 that seeks to ensure development enhances the character and appearance of the area and the NPPF.

Design and appearance

- The proposed dwelling has been designed to replicate the style of dwellings on this part of Dovecote Road. However, the acceptable design of the house is not considered to overcome the concerns raised above regarding the principle of the development.  Therefore whilst the design is considered acceptable, the principle of the development on this location fails to accord with Core Strategy policy CS7 that seeks to ensure development makes a positive contribution to the appearance and character of the area.

Residential amenity

- Policy DM4 seeks to ensure that new development will not cause a significant detrimental impact to the living environment of existing or new residential properties in terms of privacy and overlooking, access to sunlight and daylight visual dominance and overbearing among other things. Due to the limited spacing, the proposed dwelling would have an overbearing and overshadowing effect onto number 5 Dovecote Road’s outdoor private garden amenity space. The first floor rear bedroom window would overlook the rear gardens of both 6 and 7 Dovecote Road. Harm to amenity and privacy would be contrary to Policy DM4 and the NPPF.

- As highlighted above, the proposal would result in the loss of outdoor private garden amenity space for 4 Dovecote Road and an uncharacteristically smaller outdoor garden to the proposed dwelling. This would be detrimental to the amenities of the current and future occupiers of the dwellings.  This is contrary to the Policy DM10, DM11 and the NPPF.

Transport Issues

- The site is located in an area with good public transport links. In accordance with the adopted SPD, the development would be required to make parking provision of 1 space for the 1-bedroom dwelling. As the application site incorporates the garage to number 4, the assumption is that the existing garage would now be used to provide parking for the additional dwelling. Whilst the Transport officer raised concerns regarding the size of the garage, the garage is existing and therefore acceptable.  However, the proposal would therefore result in the loss of a parking space to existing number 4 Dovecote Road. Whereas the submitted Design and Access statement states that No.4 has its own parking area in addition to the garage, this has not been illustrated on submitted plans. If the principle of the proposal was acceptable in planning terms, clarification could have been sought regarding where this provision is to ensure it complies with the councils parking standards. In addition, details of the bin storage and collection have not been provided.  This could be conditioned if the proposal was acceptable.

Trees and landscaping

- Whilst there are no protected trees on the site, the Council’s Natural Environment Officer has assessed the proposal and highlighted the importance of the existing tree to the front of the application site. However, as pointed out within the proposals section above, the tree has had all branches chopped with only the trunk remaining.  Provided the tree is not allowed to grow, concerns raised by the Natural Environment Officer regarding overshadowing would not arise. However, if the proposal was acceptable in other planning terms, the Council’s Natural Environment Officer had recommended the submission of an arboricultural report in accordance with BS:5837 2012 so that the impact of the development on the tree could be ascertained. As the principle of the development was considered unacceptable, these details have not been sought.

Representations

- Most of the issues raised have been addressed in the above report. Issues to do with profit motive by the developer, the applicant not being a local resident and devaluation of properties are not considered to be planning matters in this instance.

Community Infrastructure Levy

- The Council implemented its Community Infrastructure Levy (CIL) on 1st April 2015 and planning applications decided after the 1st of April 2015 may be liable to pay the levy. This proposal would be liable to pay the levy.

Affordable Housing

- The proposed development would also need to accord with policy DM6 in terms of making a financial contribution to affordable housing. Policy DM6 (Affordable Housing) of the Local Development Framework (LDF) Sites and Detailed Policies Document (SDPD) seeks to ensure that on sites of 1-4 dwellings, a financial contribution will be made that will enable the provision of affordable housing elsewhere in the Borough.

- However, there have been changes to national planning policy in terms of the 2014 Secretary of State’s Written Ministerial Statement (WMS). The changes sought to exempt developments of 10 or less dwellings from planning obligations for affordable housing and social infrastructure contributions and to introduce a new measure known as the Vacant Building Credit.

- West Berkshire District Council and Reading Borough Council applied for a Judicial Review of the Secretary of State’s Written Ministerial Statement (WMS) to Parliament in 2014 on changes to national planning policy. The High Court handed down its judgment on the case on 31st July 2015. The High Court found in favour of the challenge by the local authorities and quashed the amendments to the NPPG. The Secretary of State appealed the judgment and the Court of Appeal then quashed the decision of the High Court on11th May 2016.

- Reading Borough Council considered its options for implementing its affordable housing policy in the light of this decision. At its meeting of the Strategic Environment Planning and Transport Committee on 13th July 2016, the Council discussed the outcome of the Court of Appeal’s decision on its challenge (http://www.reading.gov.uk/media/5651/Item09-SEPT-C-Report-on-C-of-Appeal-judgement-05-16/pdf/Item09_SEPT_C_Report_on_C_of_Appeal_judgement_05_16.pdf)

- It is notable that at paragraph 99(iii) of the judgement, the Judge reports that a statement made to the Court on behalf of the Secretary of State seeking to explain the effect of the new national policy accepted that:

“In the determination of planning applications the effect of the new 	national policy is that although it would normally be inappropriate to 	require any 	affordable housing or social infrastructure contributions on 	sites below the thresholds stated, local circumstances may justify lower (or 	no) thresholds as an exception to the national policy.  It would then be a 	matter for the decision-maker to decide how much weight to give to lower 	thresholds justified by local circumstances as compared with the new 	national policy;”

- Having considered its options, the Committee agreed the following as the basis for determining planning applications where Policy DM6 of the SDPD is relevant:

‘To implement Policy DM6 as currently adopted in the SDPD but excluding 	proposals that solely involve the conversion of an existing property, where 	the conversion involves the provision of 10 or less dwelling units (i.e. not 	HMOs), or the replacement of dwellings by the same number of	replacement dwellings where there is no net increase’.

- This planning application does not relate to the conversion of an existing property and therefore would need to accord with the requirements of Policy DM6 and make appropriate financial affordable housing contribution. The formula for calculating the actual amount sought is contained within the Affordable Housing’ SPD (2013) that is publicly available to the applicant. However, as the proposal is not acceptable in other planning terms, the applicant has not entered into a legal agreement to secure the financial affordable housing provision. The proposal therefore fails to accord with the Council’s LDF SDPD Policy DM6 (Affordable Housing) and ‘Affordable Housing’ SPD (2013).

Equality Act 2010

- In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Conclusion

- The plot size of the application site is incapable of accommodating the 1-bedroom dwelling as proposed without resulting in a cramped and overdeveloped appearance that would be detrimental to the character of the area contrary to Council’s LDF Core Strategy policy CS7 (Design and the Public Realm) and SDPD policy DM11 (Development of Private Residential Garden Land) and the NPPF.

- The proposal would result in the loss of outdoor private rear garden amenity space to 4 Dovecote Road and due to the plot size the resultant outdoor private garden amenity space for the proposed dwelling would be uncharacteristically small in comparison with the prevailing garden sizes in the locality. This is contrary to the Council’s LDF SDPD Policy DM10 (Private and Communal Outdoor Space) and DM11 (Development of Private Residential Garden Land) and the NPPF.

- Due to the limited space around the proposed dwelling, the proximity of the proposed dwelling to no. 5 Dovecote Road’s outdoor private rear garden amenity space would have an overbearing and overshadowing appearance, as well as overlooking the private gardens to 6 and 7 Dovecote Road. This would be contrary to the Council’s LDF SDPD Policy DM4 (Safeguarding Amenity) and the NPPF.

- In the absence of a completed legal agreement to secure appropriate and necessary financial contributions towards Affordable Housing provision, the proposal fails to accord with the Council’s LDF SDPD Policy DM6 (Affordable Housing) and ‘Affordable Housing’ SPD (2013 ).

REFUSE