### Delegated Application Report				Target Date: 05/03/2018

Address: 1 Morecambe Avenue

Application No: 180014/CLP

Proposal: Certificate of lawfulness for proposed construction of a hip to gable and rear dormer roof extension and two front facing roof lights.

Case Officer: Heather Banks

## Introduction & Proposal

The application site is a two storey, semi-detached property on the corner of Morecambe Avenue and Conisboro Avenue. The property has an existing single storey rear extension which runs along the boundary with no.3 and has an element which projects to the side of the existing property. The property also has a large rear, side and front garden, enclosed by fencing and a relatively high hedge. The property and its adjoined neighbour face the corner of the two roads, rather than sitting perpendicular to either one or the other although the boundary line between the rear gardens runs perpendicular to Morecambe Avenue rather than the properties themselves.

The applicant proposes to construct a hip to gable and rear dormer roof extension to the property and insert two roof lights to the front elevation of the property. The dormer extension is to have a flat roof, set down 0.5m from the ridge line of the existing house, and will be set in 0.2m from the eaves of the roof. The hip to gable extension will also require the removal of one of the two existing chimneys.

Plans Submitted:

The agent supplied the following drawings:

Location Plan

Drawing No: 1357-06 – Existing Ground Floor Plan

Drawing No: 1357-07 – Existing First Floor Plan

Drawing No: 1357-08 rev.B – Existing and Proposed Elevations

Drawing No: 1357-09 – Proposed Ground Floor Plan

Drawing No: 1357-10 – Proposed First Floor Plan

Drawing No: 1357-11 – Proposed Loft Floor Plan

Drawing No: 1357-12 – Section A-A

Planning Statement – Wargrave Design Consultancy Limited

Received 3rd January 2018

Block Plan

Received 8th January 2018

Relevant Planning History:

180016/HOU – Ground Floor rear/side extension. Under Consideration

01-00221-OUT - Erection of 1 x detached dwelling and detached single garage. Refused 12/04/2001

Summary of Consultations:

No consultations were undertaken as it is not required for an application of this type.

Nevertheless two objections were received expressing concern regarding the impact of the proposals on the amenity of the neighbouring properties and the character of the property and the street scene.

Relevant Planning Policy & Guidance:

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions Class B and C and G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘the enlargement of a dwelling house consisting of an addition or alteration to its roof’ and ‘other alterations to the roof of a dwelling house’ and ‘The installation, alteration or replacement of a chimney, flue or soil and vent pipe on a dwellinghouse’.

Development is not permitted by Class B if:

- B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES.

- B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; COMPLIES.

- B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; COMPLIES. Although the hip to gable extension will extend beyond the plane of a roof slope which fronts a highway, this is not considered to form part of the principle elevation. The principle elevation is considered to be that which faces the junction of Morecambe Avenue and Conisboro Avenue.

- B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than 50 cubic metres in the case of a semi-detached house; COMPLIES, volume is approx. – 33.18 cubic metres.

- B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe DOES NOT COMPLY. HOWEVER, REMOVAL OF CHIMNEY IS PERMITTED DEVELOPMENT UNDER CLASS G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 or

- B.1 (f) the dwellinghouse is on article 2(3) land.  COMPLIES.

Development is permitted by Class B subject to the following conditions:

- B.2 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; COMPLIES. The planning statement submitted with the application states that all materials are to match the existing dwelling.

- B.2 (b) the enlargement must be constructed so that –

- other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension – (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; NOT APPLICABLE for hip to gable extension and COMPLIES for dormer extension, and

- other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; COMPLIES.

- B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and (ii) non-opening unless the parts of the windows which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. NOT APPLICABLE in this instance

Development is not permitted by Class C if:

- C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- C.1 (b) the alteration would protrude more than 0.15m beyond the plane of the slope of the original roof; COMPLIES. The planning statement submitted with the application states the roof lights will project a maximum of 150mm above the plane of the roof.

- C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; COMPLIES or

- C.1 (d) it would consist of or include (i) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment. DOES NOT COMPLY. HOWEVER, REMOVAL OF CHIMNEY IS PERMITTED DEVELOPMENT UNDER CLASS G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended)

Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of a dwellinghouse shall be:

- C.2 (a) obscure glazed; and (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. NOT APPLICABLE IN THIS INSTANCE.

Conclusion

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Classes B, C and G of Part 1 of, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), and it has been confirmed that the proposal would be lawful, on the basis of the following submitted drawings and documents:

Location Plan

Drawing No: 1357-06 – Existing Ground Floor Plan

Drawing No: 1357-07 – Existing First Floor Plan

Drawing No: 1357-08 rev.B – Existing and Proposed Elevations

Drawing No: 1357-09 – Proposed Ground Floor Plan

Drawing No: 1357-10 – Proposed First Floor Plan

Drawing No: 1357-11 – Proposed Loft Floor Plan

Drawing No: 1357-12 – Section A-A

Planning Statement – Wargrave Design Consultancy Limited

Received 3rd January 2018

Block Plan

Received 8th January 2018

Therefore the Local Planning Authority will, on this occasion, issue the applicant with a Certificate of Lawfulness for the proposed development.

Recommendation:

GRANT Certificate

Conditions

As conclusion.

Informatives

- The applicant’s attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

- During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at: http://www.communities.gov.uk/publications/planningandbuilding/partywall

- The construction (and demolition) of the development will be subject to environmental health law.

- Work on a loft or roof may affect bats.  The applicant will need to consider protected species when planning work of this type.  A survey may be needed, and if bats are using the building, a licence may be required.  To view specific information regarding bats use the following link:

https://www.gov.uk/wildlife-licences

Site Map