DELEGATED APPLICATION REPORT 				Target Date:	07/05/2018

Application No: 180445/HOU

Address: 101 St Saviours Road, Reading

Proposal: Single storey side and rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property.  The property is forward of its neighbouring property at 103 St Saviours Road.  The property has a long rear garden and there are steep levels to the rear.  The surrounding area is predominantly residential.

Proposal:

The proposal involves a single storey side and rear extension.  The proposed side extension measures 2.3m in width and includes a front canopy to the main entrance and a car port to the front side with an extension behind.  The proposed car port measures 4.7m in depth and is open sided although it has a pitched roof with tile hanging.  The proposed side extension extends a further 7.2m to the rear and wraps around the rear of the property.  The extension measures 4m in depth from the rear of the existing property and extends the full width of the existing house and to the rear of the proposed side extension.  The rear element of the extension has a mono pitched roof and measures 2.5m in height to eaves level and 3.3m in height at its highest point.

The proposal would extend close to the boundary with both 99 and 103 St Saviours Road.  99 St Saviours Road has a rear conservatory close to the boundary measuring approximately 3m in depth and 103 St Saviours Road has a single storey side and rear extension which extends along the boundary.  103 St Saviours Road is located further back than the application site and this neighbouring extension extends approximately 4m further to the rear than the proposed extension.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 10th March 2018:

Drawing No: 01 – Existing Plans & Elevations

Drawing No: 02 – Proposed Elevations

Drawing No: 03 – Proposed Ground Floor Plan

Drawing No: 04 – Proposed Plans

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection subject to informatives.

Ecology raised no objection.

Properties at 99 and 103 St Saviours Road were consulted.  One letter of observation has been received with regards to:

- Potential for damage to foundations of existing conservatory.  Officer Note: this matter would be addressed through building regulations and is a Party Wall matter between neighbours.

- Preferred if existing boundary fence was retained.  Officer Note: this is not a material planning consideration however the agent advised that the proposed extension is 150mm off the boundary and the fence will be retained.  The agent did state it will be difficult to maintain the fence between 101 and 99 St Saviours Road and it would be better to remove it between the existing wall at 99 and the wall of the proposed extension and attach it at the end.  This will be something for the neighbours to address.

- There is an existing sewage main at the rear of the properties.  Officer Note: this would be addressed through building regulations and Thames Water.

Summary of discussions with applicant/agent:

The agent was asked in an email dated 30th April 2018 whether the existing boundary fence is to be retained.  The agent responded on 1st May 2018 and their comments are noted in point 2 above.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the proposal it is not considered harmful in terms of visual dominance or overbearing effects.  A condition will be imposed to ensure the car port remains open and available for parking as it is forward of, and extends along the boundary with, 103 St Saviours Road.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

- Car port to remain open and available for parking

Informatives relating to:

- Positive and proactive

- Terms and conditions

- Highways

4.	Building regulations

5.	Construction and demolition

6.	Encroachment

7.	Do not damage the verge