Delegated Application Report

Target Date: 20/07/2017

Address: 35 Alpine Street, Reading

Application No: 170818

Case Officer: Ralph Chakadya

Site Description

- The application site comprises a mid-terrace unit on a distinct bend in the street and associated frontages. The property currently comprises 2 flats, 35 and 35A, with separate front entrances and rear gardens.

- The unused garage is beneath part of the upper flat and leads to an area of decking and rear garden. The garage appears not be in use due to its narrow frontage that is maximum 2m wide and therefore restrictive to most car types.

- The ground levels are very steep towards the rear such that the properties are two-storey at the front and four-storey to the rear with lower and upper ground floor levels.

Description of Development

- The proposal is for the conversion of an existing unused garage and part single- part two-storey rear extension to create a two-bedroom residential maisonette.

- The proposal will involve creating a lower ground floor under the existing garage with 4 m deep extensions replacing the existing decking to create lower ground floor to accommodate the kitchen/lounge area for the unit.  The lower ground floor will lead onto the existing rear garden that will provide amenity space to the new unit.

- The garage will be converted and extended (2m deep x 2.8m wide) to create the upper ground floor accommodating 2 bedrooms and a bathroom. Separate front access matching that to existing number 35A will be provided, with internal stairs accessing the lower ground floor unit.

- Following concerns raised by officers regarding the potential overbearing impact of the single storey element at lower ground floor onto existing number 35A rear garden, amended plans were submitted with a more regular shape away from the boundary.

Relevant Planning History

- 151041PREAPP - Single storey rear extension and conversion of existing maisonette into a one bedroom maisonette and a three bedroom house

- HISPLAN310010947 - Conversion of house into two flats. Granted 19 October 1973

Neighbour Responses

- One letter of support was received highlighting also the need to tidy up the existing garden by pruning trees and clearing leaves.

Consultation Responses

RBC Transport

- The existing house at No 35 alpine Street already contains 2 maisonettes.  This application proposes to extend and convert the existing garage to form a 2 bedroom maisonette therefore in accordance with the adopted Parking Standards and Design SPD; the development would be required to provide a parking provision of 1 parking space for the new dwelling.

- The site is located within the Zone 2, the primary core area but on the periphery of the central core area which lies at the heart of Reading Borough, consisting primarily of retail and commercial office developments with good transport hubs.

- The development site is located in an area designated as a Residents Parking Permit Area; Zone 10R.  Under the Borough’s current parking standards, this proposal would generate additional pressure for parking in the area.  Therefore if this application is approved there should be an assumption that any future occupants of the proposed dwelling will not be issued with a resident parking permit which should be covered by condition and an informative applied.

- No bin storage area is indicated in the submitted plans however it is assumed that it would be the same as the other units on the site.

- Cycle storage is not illustrated on plans.  In accordance with the adopted Parking SPD, the development is required to provide a minimum of 1 cycle parking space for the new dwelling which should be in a conveniently located, lockable, covered store.  Clarification is required on this point and a plan submitted illustrating the proposed provision.  This can be in the form of a lockable garden shed.  I am happy to deal with this by condition.

- There are no transport objections in respect of this proposal subject to conditions

POLICIES

Legal and Planning Policy Context

- Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

- The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Framework

Reading Borough Local Development Framework Core Strategy (2008)

- CS1 (Sustainable Construction and Design)

- CS4 (Accessibility and the Intensity of Development)

- CS5 (Inclusive Access)

- CS7 (Design and the Public Realm)

- CS14 (Provision of Housing)

- CS15 (Location, Accessibility, Density and Housing Mix)

- CS24 (Car/Cycle Parking)

Sites and Detailed Policies Document (2012)

- SD1 (Presumption in Favour of Sustainable Development)

- DM4 (Safeguarding Amenity)

- DM6 (Affordable Housing)

- DM9: House Extensions and Ancillary Accommodation

- DM12 (Access, Traffic and Highway Related Matters)

Local supplementary planning guidance:

- The Council’s ‘Revised Sustainable Design & Construction SPD’ (July 2011)

- The Council’s ‘Parking Standards and Design’ SPD (Oct 2011)

- The Council’s ‘Revised SPD on Planning Obligations under Section 106 of the Town and Country Planning Act 1990’ (2013)

- The Council’s ‘Affordable Housing’ SPD (2013)

- Supplementary Planning Guidance “A Design Guide to House Extensions” (2003)

Principle of change of use

- The principle of residential use is already established in terms of the existing two maisonettes.  In terms of the character of the area, pursuance of residential development is considered more acceptable. Subject to other planning considerations, the proposal that accords with Core Strategy policy CS14 in providing additional housing stock to the borough is considered acceptable in principle.

Character and Design

- This area is predominantly residential characterised by terraced Victorian houses. Viewed from the street scene, the only alterations would be the introduction of a front door replacing the garage door. The front door matching that at number 35 would enhance the frontage appearance to the property and would therefore not be detrimental to the street scene. To the rear, properties on this part of Alpine Street have been extended in various ways ranging from single to two-storey extensions with elevational appearance at 4 storey level due to the changing ground levels. The proposed extensions creating lower and upper ground floor levels with matching external materials would not depart from the character of the area.

Impact on neighbouring properties and future occupants

- The proposed extension at lower ground floor would project 2m beyond the existing single storey element to neighbouring number 33, a distance not considered to result in harm to number 33 in terms of being overbearing. As highlighted within the proposals section above, amended plans were submitted shifting the lower ground extension from the boundary with number 35A thereby mitigating the overbearing impact to levels considered not detrimental to the amenities of number 35A’s rear garden.

- The proposed upper ground floor extension would project 2m beyond the rear building line of neighbouring number 33, a projection that would not result in the breach of the 45-degree angle in relation to a rear window to habitable room. The extension would therefore not result in loss of light to number 33.

- The proposed room sizes are considered to provide an acceptable level of amenity to the future occupiers of the flats. The applicant has submitted the existing floor plan of the upper floor to No.35A that shows bedrooms above the proposed bedrooms to the upper ground floor. As such the stacking of the rooms is considered acceptable not to result in harm to the future occupants in terms of noise and disturbance.

- Whilst no bin storage area is indicated in the submitted plans, it is assumed that it would be the same as the other units on the site and therefore acceptable. A condition will be added requiring the submission of cycle storage details.

Transport and parking

- The location of the application site is located within Zone 2 that has good public transport. It is therefore considered sustainable in terms of the provision of means of transport.

- The development site is located in an area designated as a Residents Parking Permit Area; Zone 10R.  Under the Borough’s current parking standards, this proposal would generate additional pressure for parking in the area.  To prevent pressure on existing parking, any future occupants of the proposed dwelling will not be issued with a resident parking permit. This will be covered by condition and an informative.

Other considerations

Community Infrastructure Levy

- The Council implemented its Community Infrastructure Levy (CIL) on 1st April 2015 and this proposal would be liable to pay the levy.

Affordable housing

- The proposed development would also need to accord with policy DM6 in terms of making a financial contribution to affordable housing. Policy DM6 (Affordable Housing) of the Local Development Framework (LDF) Sites and Detailed Policies Document (SDPD) seeks to ensure that on sites of 1-4 dwellings, a financial contribution will be made that will enable the provision of affordable housing elsewhere in the Borough.

- However, there have been changes to national planning policy in terms of the 2014 Secretary of State’s Written Ministerial Statement (WMS). The changes sought to exempt developments of 10 or less dwellings from planning obligations for affordable housing and social infrastructure contributions and to introduce a new measure known as the Vacant Building Credit.

- West Berkshire District Council and Reading Borough Council applied for a Judicial Review of the Secretary of State’s Written Ministerial Statement (WMS) to Parliament in 2014 on changes to national planning policy. The High Court handed down its judgment on the case on 31st July 2015. The High Court found in favour of the challenge by the local authorities and quashed the amendments to the NPPG. The Secretary of State appealed the judgment and the Court of Appeal then quashed the decision of the High Court on11th May 2016.

- Reading Borough Council considered its options for implementing its affordable housing policy in the light of this decision. At its meeting of the Strategic Environment Planning and Transport Committee on 13th July 2016, the Council discussed the outcome of the Court of Appeal’s decision on its challenge (http://www.reading.gov.uk/media/5651/Item09-SEPT-C-Report-on-C-of-Appeal-judgement-05-6/pdf/Item09_SEPT_C_Report_on_C_of_Appeal_judgement_05_16.pdf).

- It is notable that at paragraph 99(iii) of the judgement, the Judge reports that a statement made to the Court on behalf of the Secretary of State seeking to explain the effect of the new national policy accepted that:

“In the determination of planning applications the effect of the new national 	policy is that although it would normally be inappropriate to require any 	affordable housing or social infrastructure contributions on sites below the 	thresholds stated, local circumstances may justify lower (or no) thresholds as an 	exception to the national policy.  It would then be a matter for the decision-	maker to decide how much weight to give to lower thresholds justified by local 	circumstances as compared with the new national policy;”

- Having considered its options, the Committee agreed the following as the basis for determining planning applications where Policy DM6 of the SDPD is relevant:

‘To implement Policy DM6 as currently adopted in the SDPD but excluding 		proposals that solely involve the conversion of an existing property, where 		the conversion involves the provision of 10 or less dwelling units (i.e. not 		HMOs), or the replacement of dwellings by the same number of 				replacement dwellings where there is no net increase’.

- This planning application relates mainly to the conversion of an existing garage to create another two-bedroom flat. As such the resultant flat from the conversion of the existing garage would not be liable to make contributions to affordable housing contributions in accordance with policy DM6. No affordable housing contributions would therefore be sought.

Equality Act 2010

- In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Recommendation

- Grant Permission