DELEGATED APPLICATION REPORT 				Target Date:	27/12/2017

Application No: 171891/HOU

Address: 10 Cholmeley Place, Reading

Proposal: Single storey rear extensions

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey end terrace property with a single storey rear extension measuring 6m in depth.  The property has an existing flat roof rear dormer window.  The property has a wider plot than the neighbouring properties due to vehicular access to the side.  The surrounding area is predominantly residential.

Proposal:

The proposal has been amended and now includes a single storey side extension to the side of the existing single storey rear extension and a single storey rear extension to the rear of the existing single storey rear extension.  The proposed side extension extends along the boundary with 12 Cholmeley Place and measures 6m in depth and has a mono pitch roof measuring 2.7m in height to eaves level.  The proposed rear extension measures 2.5m in depth with an eaves height of 3m.  There is a distance of approximately 3m between the side elevation of the property and the boundary with 6 Cholmeley Place.  The proposed rear extension would be 1.6m from the boundary with 12 Cholmeley Place.

The materials will consist of brick and slates to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 26th September 2017:

Drawing No: SB.10 – Site Block Plan

Drawing No: SB.11 – Proposed Layout

Drawing No: SB.12 – Proposed Elevations

Drawing No: SB.20 – Existing Layout

Drawing No: SB.21 – Existing Elevations

The agent submitted the following plan, received on 30th September 2017:

Location Plan

The agent submitted the following amended drawings, received on 18th December 2017:

Drawing No: SB.10 Rev A – Site Block Plan

Drawing No: SB.11 Rev A – Proposed Layout

Drawing No: SB.12 Rev A – Proposed Elevations

Relevant Planning History:

141859/HOU - Single storey ground floor and first floor rear extensions.  Refused 14/01/2015.

Consultation Responses:

Properties at 6 and 12 Cholmeley Place and 5 Eastgate Court were consulted.  One letter of representation has been received with regards to:

- Overdevelopment of the property.

- Overbearing on back garden of the neighbouring property.

- Existing use as a HMO causing problems with parking, vehicular and pedestrian movements and an extension will no doubt be used to accommodate more rental income.  Officer Note: the existing use of the property is not known however the application site does not fall within an Article 4 Direction area which removed permitted development rights for changes between a C3 (dwellinghouse) and a C4 (small HMO).  The property could be used as a C4 HMO without the need for planning permission however an informative regarding change of use to a sui generis HMO will be included in the decision.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 15th December 2017 that there was no objection in principle to a side and rear extension however the proposal as submitted was considered harmful to the neighbouring property at 12 Cholmeley Place due to the excessive depth and height along the boundary.  Amended plans were submitted on 18th December 2017 which overcame the concerns raised.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposed extension to the side of the existing extension exceeds the Council’s guidance of 4m in depth it would not extend any further than the existing extension and the property and garden are larger than average due to its side access and the property is considered capable of taking an extension of this length.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the orientation of the properties is such that the proposed side extension will not significantly reduce the amount of light to the neighbouring property at 12 Cholmeley Place and there is sufficient distance between the extension and 6 Cholmeley Place.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, although the residents of the neighbouring properties will notice the additional height and depth of the amended proposal it is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Approved plans

- Materials

Informatives relating to:

- Positive and proactive

- Terms and conditions

3.	Building regulations

4.	Construction and demolition

5.	Encroachment

6. 	Sui Generis HMO