DELEGATED APPLICATION REPORT 				Target Date: 01/09/17

Application No: 171130/HOU

Address: 26 Green Road

Proposal: Side and rear dormers, roof lights and alteration to fenestration (retrospective)

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site is a semi-detached property with a part-hipped roof. The property has a two storey front gable with a pitched roof and an integral garage. The property is located on the western side of Green Road. The site topography slopes away steeply to the rear. The property has undergone a loft conversion with the provision of side and rear dormers and roof lights, the subject of this application. From the rear dormer, views extend clearly to the rear of properties of Regis Park Road and beyond. The surrounding area is predominantly residential.

Proposal

The proposal is for side and rear dormers, roof lights and an alteration to the fenestration. 26 Green Road has been the subject of an enforcement investigation (E17051) into the construction of two unauthorised dormer extensions; the outcome being that enforcement action is recommended. The applicant has been made aware of officers’ concerns and rather than voluntarily remove the dormers, has submitted this retrospective planning application.

The rear dormer projects to a depth of 4.1m and extends at a maximum width of 2.9m. The dormer is asymmetrical in shape. The dormer has a maximum height of 2.8m, with no set down from the ridgeline of the original property. The dormer is set back from the original eaves by 0.3m. The rearward elevation is predominantly glazed, with inwardly opening double doors and a Juliet balcony. The cheeks of the dormer are rendered, and do not match the original property. The roof tiles and fascia’s use materials to match the original property.

The side dormer projects to a depth of 3.9m and extends at a width of 2.3m. The dormer has a pitched roof with a maximum height of 2.3m, set down from the ridgeline of the original property by 0.3m. The dormer is set back from the original eaves by 0.4m. An obscure-glazed window, non-opening to 1.7m above the floor level of the room in which it is located, is located to the sideward elevation. The face and cheeks of the dormer are rendered, and do not match the original property. The roof tiles and fascia’s use materials to match the original property. A sun pipe projects from the rearward roof slope, exceeding the roof height by 0.1m.

Roof lights are located to the forward and sideward roof slopes of the original property.

The ground floor window to the rear elevation has been replaced by a larger window and door.

Plans Considered

The applicant supplied the following, received on 6th July 2017:

Planning Statement

Site Location

Block Plan

Drawing No: 003 – Proposed Site Plan and Photographs

Drawing No: 004 – Existing Floor Plans

Drawing No: 007 – Proposed Floor Plans

Drawing No: 008 – Proposed Second Floor Plan and Section

Drawing No: 005 – Existing Elevations

Drawing No: 006 – Existing Elevation

Drawing No: 009 – Proposed Elevations

Drawing No: 010 – Proposed Elevation

Drawing No: 011 - Photographs

Relevant Planning History

Condition 8 of planning permission 83/TP/675, granted 25/01/84.

“Notwithstanding the provisions of Article 3 and Schedule 1 (Class 1) of the Town and Country Planning General Permitted Development Orders 1977 to 1981 no extensions to the houses hereby permitted, shall be built without the express permission of the Local Planning Authority.”

Reason – “Due to the restricted size of the plots, width of the site and the design of the houses, any extensions could be detrimental to the amenities of adjacent dwellings and to the appearance of the development. The Local Planning Authority therefore wish to retain control over future extensions”.

171159/VAR - Variation of condition 8 (removal of permitted development rights) of planning permission 83/TP/675 dated 25/01/84 for 8 semi-detached dwellings at 20-28 Green Road, namely to restore permitted development rights Class B to Class H of Part 1 to Schedule 2 of The Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) – to be determined

Consultation Responses

Neighbouring owners and occupiers at 24, 27, 28, 29 and 31 Green Road and 16, 17 and 18 Regis Park Road were consulted.

Four letters of representation were received with regard to the following:

- The design of the dormer window and grey frames add to the architecture of the area

- The dormer blends well with the existing roofs and overall finishes used in other houses along the street

- The proposal re-invigorates the property and adds to the immediate surroundings and overall environment of the street

- Existing trees to the western site boundary obstruct light to the garden of 22 Green Road. Two further fast growing trees will prevent even more light reaching the garden. Slow growing trees should be considered instead

- Appropriate steps should be taken to respect the privacy of 17 Regis Park Road. If trees are to be planted they should be of a decent height to block the view

- The rear dormer removes all amenity of 18 Regis Park Road in terms of privacy, overlooking, visual dominance and its overbearing, huge scale

- The proposal is detrimental to the residents of 18 Regis Park Road internally and externally

- The glazing to the rear dormer is excessive, unnecessary and out of character with the house and area

- Issues are exacerbated by the significantly higher land position of 26 Green Road and the small lengths of the gardens

- The exterior materials used are not similar to those of the existing property

- The size and scale of the rear dormer are out of scale with the house and the surrounding area

- The white render is out of character

- Evergreen trees with a minimum/maximum height as a planning condition would help

Transport raised no objection to the proposal.

Ecology raised no objection to the proposal.

Trees raised no objection to the proposal.

Summary of discussions with applicant/agent

The applicant was advised on 17th August 2017 that the application was to be recommended for refusal.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

The main issues to be considered are:

- Design and impact on the character of the surrounding area

- Impact on neighbours

a) Design and impact on the character of the surrounding area

The dormer extensions have a detrimental impact on the character of the host property and those in the surrounding area due to their size and design. The dormers are excessive in size, particularly the rear dormer, and are not sufficiently set into the roof slope or down from the ridgeline of the original roof. The dormers are not in proportion or scale with the rest of the property in terms of their overall size and window shape. Additionally, the dormer windows do not respect the pattern of existing windows by being positioned to line through vertically with the window openings below. The glazing to the rear dormer is excessive. The materials used in the exterior finishing of the dormers do not match those of the original property. As such, the development is contrary to Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document (2012) and the Supplementary Planning Guidance – A Design Guide to House Extensions (2003).

The roof lights and the alteration to the fenestration are generally acceptable.

b) Impact on neighbours

The rear dormer causes a significant detrimental impact to the living environment of the adjacent properties, particularly 17 and 18 Regis Park Road, through loss of privacy caused by overlooking. Due to the site topography, the rear dormer is visually dominant and draws the eye. As such, the development in contrary to DM4: Safeguarding Amenity of the Sites and Detailed Policies Document (2012).

It is acknowledged that the intention for the development was to create additional habitable accommodation, suitable to support a family home. The retention and extension of family housing is generally supported.  However, in this instance this is outweighed by other material planning considerations as identified above.

RECOMMENDATION

REFUSE

Informatives relating to:

- Refused plans

- Positive and proactive