### Delegated Application Report			Target Date: 21/06/2017

Address:  42 Highmoor Road

Application No: 170667/CLP

Proposal: Ground floor rear extension, rear dormer roof extension, two rooflights in front roofslope, removal of two chimneys and fenestration alterations.

Case Officer: Heather Banks

## Introduction & Proposal

The application site is a two storey, pitched roof, brick, semi-detached property with a two storey rear gable and a two storey bay window with pitched roof to the front of the property. The main entrance to the property is to the side.

The applicant proposes to construct a single storey extension to the rear of the two storey rear gable and insert a new set of glazed sliding doors and a full height window to the ground floor side elevation of the existing rear gable. It is also proposed to enlarge the existing first floor rear facing window to have a greater height and width and fit it with sliding doors and a Juliet balcony. Finally, it is also proposed to construct a loft conversion with an l-shaped dormer extension to the rear and two rooflights to the front roof slope. The existing two chimneys to the rear of the property will need to be removed for the construction of the dormer extension.

Plans Submitted:

The agent supplied the following drawings:

Drawing No: 17006-0-001 rev.P1 – OS Location Map

Drawing No: 17006-0-002 rev.P1 – Site Plan Existing

Drawing No: 17006-0-003 rev.P1 – Site Plan Proposed

Drawing No: 17006-0-010 rev.P1 – Ground Floor Plan, Existing

Drawing No: 17006-0-011 rev.P1 – First Floor Plan, Existing

Drawing No: 17006-0-012 rev.P1 – Roof/Loft Plan Existing

Drawing No: 17006-0-020 rev.P1 – Front Elevation, Existing

Drawing No: 17006-0-021 rev.P1 – Side Elevation, Existing

Drawing No: 17006-0-022 rev.P1 – Rear Elevation, Existing

Drawing No: 17006-0-110 rev.P1 – Garden/Patio, Ground floor plan, Proposed

Drawing No: 17006-0-111 rev.P1 – First Floor Plan, Proposed

Drawing No: 17006-0-112 rev.P1 – Second/Loft Floor Plan, Proposed

Drawing No: 17006-0-113 rev.P1 – Roof Plan, Proposed

Drawing No: 17006-0-120 rev.P1 – Front Elevation, Proposed

Drawing No: 17006-0-121 rev.P1 – Side Elevation, Proposed

Drawing No: 17006-0-122 rev.P1 – Rear Elevation, Proposed

Received 27th April 2017

Relevant Planning History:

None

Summary of Consultations:

No consultations were undertaken as it is not required for an application of this type.

Nevertheless, a comment has been received raising concerns that:

- Construction work has been started.

- The applicant is a commercial letting company.

- The property may be turned into flats.

Case Officer Comments:

- It has been confirmed that construction work has started at this property but is not yet complete. The applicant does not need to have received a Certificate of Lawfulness before commencing work (although it is recommended) as works which are permitted development do not need planning permission and the Certificate is just to confirm whether or not the works are indeed permitted development. Should the applicant have completed the works before applying for a Certificate, a Certificate of Existing Use (rather than Proposed) would need to have been applied for. However, as the works are underway but not complete, it is considered that the correct Certificate has been applied for.

- The nature of the company who is the applicant for this application is not an issue which can be considered under a Certificate of Lawfulness application.

- Should the applicant intend to convert the property into flats they will need to submit a full planning application. No such application has been submitted to date, and if it were it would be considered on its own merits.

Relevant Planning Policy & Guidance:

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions of Classes A, B, C and G of Part 1, of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relate to ‘the enlargement, improvement or other alteration of a dwelling house’, ‘the enlargement of a dwelling house consisting of an addition or alteration to its roof’, ‘other alterations to the roof of a dwelling house’ and ‘chimneys, flues etc. on a dwelling house’.

Development is not permitted by Class A if:

- A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); COMPLIES

- A.1 (c) the height of the dwellinghouse enlarged, improved or altered would exceed the highest part of the roof of the existing dwellinghouse; COMPLIES

- A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; COMPLIES, eaves height will be 3m

- A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principal elevation of the original dwellinghouse; (ii) fronts a highway and forms a side elevation of the original dwellinghouse; NOT APPLICABLE IN THIS INSTANCE

- A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres in the case of a semi-detached house, or (ii) exceed 4 metres in height; COMPLIES, the proposal is to be 3m in depth and is 3m in height.

- A.1 (g) - NOT APPLICABLE IN THIS INSTANCE.

- A.1 (h) - NOT APPLICABLE IN THIS INSTANCE.

- A.1 (i) the enlarged part of the dwellinghouse would be within 2m of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3m; COMPLIES, the proposed single storey rear extension measures 3m in height to eaves level.

- A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would (i) exceed 4m in height, (ii) have more than one storey, or (iii) have a width greater than half the width of the original dwellinghouse; NOT APPLICABLE IN THIS INSTANCE.

- A.1 (k) it would consist of or include – (i) the construction or provision of a veranda, balcony or raised platform, (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse.  COMPLIES. A Juliet ‘balcony’ is not considered to constitute a balcony in this instance.

Development is permitted by Class A subject to the following relevant conditions:

- A.3 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; COMPLIES. Drawing Nos. 17006-0-121 and 17006-0-122 state that the rear extension is to be finished in brick to match the existing property

- A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse shall be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed; NOT APPLICABLE IN THIS INSTANCE and

- A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, the roof pitch of the enlarged part shall, so far as practicable, be the same as the roof pitch of the original dwellinghouse. NOT APPLICABLE IN THIS INSTANCE.

It should be noted that the single storey rear extension, the new ground floor openings to the side elevation of the property and the enlarged first floor rear window fall to be considered under Class A. Most of the conditions of Class A refer to extensions rather than alterations and as such the answers above are mostly related to the proposed rear extension. The new and enlarged openings comply with Class A as the side elevation openings are at ground level and the first floor opening is rear facing meaning that condition A.3(b) does not apply and the restriction stated in this condition are not required for these openings.

Development is not permitted by Class B if:

- B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES.

- B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; COMPLIES. Although the proposed works would exceed the height of the roof of the existing rear gable at the property, they would not exceed the height of the highest part of the roof of the main house.

- B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; COMPLIES.

- B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than 50 cubic metres in the case of a semi-detached house; COMPLIES, the agent has calculated that the volume of the proposed dormer extension would be approx. 46.17m3. The Case Officer calculated the approx. volume as 44.1m3.

- B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; DOES NOT COMPLY. HOWEVER, REMOVAL OF CHIMNEY IS PERMITTED DEVELOPMENT UNDER CLASS G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015

Or

- B.1 (f) the dwellinghouse is on article 2(3) land.  COMPLIES.

Development is permitted by Class B subject to the following conditions:

- B.2 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; COMPLIES, Drawing Nos. 17006-0-121 and 17006-0-122 state that the dormer extension will be clad in slate to match the existing roof.

- B.2 (b) the enlargement must be constructed so that –

- other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension – (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; COMPLIES, and

- other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; COMPLIES.

- B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and (ii) non-opening unless the parts of the windows which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. NOT APPLICABLE IN THIS INSTANCE.

Development is not permitted by Class C if:

- C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- C.1 (b) the alteration would protrude more than 0.15m beyond the plane of the slope of the original roof; COMPLIES, AS PER AN ANNOTATION ON THE PLAN

- C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; COMPLIES or

- C.1 (d) it would consist of or include (i) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment. DOES NOT COMPLY. HOWEVER, REMOVAL OF CHIMNEY IS PERMITTED DEVELOPMENT UNDER CLASS G, Part 1, Schedule 2 of the Town and Country

Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of a dwellinghouse shall be:

- C.2 (a) obscure glazed; and (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. NOT APPLICABLE IN THIS INSTANCE.

Development is not permitted by Class G if:

- G.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use);

COMPLIES

- G.1 (b) the height of the chimney, flue or soil and vent pipe would exceed the highest part of the roof by 1 metre or more; COMPLIES

- G.1 (c) in the case of a dwellinghouse on article 2(3) land, the chimney, flue or soil and vent pipe would be installed on a wall or roof slope which—

(i) fronts a highway, and

(ii) forms either the principal elevation or a side elevation of the dwellinghouse.

NOT APPLICABLE IN THIS INSTANCE.

Conclusion

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class A, B, C and G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), and it has been confirmed that the proposal would be lawful, on the basis of the following submitted drawings received on 26th April 2017:

Drawing No: 17006-0-001 rev.P1 – OS Location Map

Drawing No: 17006-0-002 rev.P1 – Site Plan Existing

Drawing No: 17006-0-003 rev.P1 – Site Plan Proposed

Drawing No: 17006-0-010 rev.P1 – Ground Floor Plan, Existing

Drawing No: 17006-0-011 rev.P1 – First Floor Plan, Existing

Drawing No: 17006-0-012 rev.P1 – Roof/Loft Plan Existing

Drawing No: 17006-0-020 rev.P1 – Front Elevation, Existing

Drawing No: 17006-0-021 rev.P1 – Side Elevation, Existing

Drawing No: 17006-0-022 rev.P1 – Rear Elevation, Existing

Drawing No: 17006-0-110 rev.P1 – Garden/Patio, Ground floor plan, Proposed

Drawing No: 17006-0-111 rev.P1 – First Floor Plan, Proposed

Drawing No: 17006-0-112 rev.P1 – Second/Loft Floor Plan, Proposed

Drawing No: 17006-0-113 rev.P1 – Roof Plan, Proposed

Drawing No: 17006-0-120 rev.P1 – Front Elevation, Proposed

Drawing No: 17006-0-121 rev.P1 – Side Elevation, Proposed

Drawing No: 17006-0-122 rev.P1 – Rear Elevation, Proposed

Therefore the Local Planning Authority will, on this occasion, issue the applicant with a Certificate of Lawfulness for the proposed development.

Recommendation:

GRANT Certificate

Conditions

As conclusion.

Informatives

- The applicants attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

- During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at: http://www.communities.gov.uk/publications/planningandbuilding/partywall

- The construction (and demolition) of the development will be subject to environmental health law.

- Work on a loft or roof may affect bats.  The applicant will need to consider protected species when planning work of this type.  A survey may be needed, and if bats are using the building, a licence may be required.  To view specific information regarding bats use the following link:

https://www.gov.uk/wildlife-licences

Location Plan (not to scale)