DELEGATED APPLICATION REPORT

Application No: 171039/LBC					Target Date: 29/09/2017

Address: Wrenbury Court, 16-18 Eldon Road

Proposal: Remedial works to Listed Building to address water ingress and damp problems

Case officer: Heather Banks

Site Description and Surrounding Area:

The application site comprises a Grade II listed pair of properties, circa 1840s. The properties have been converted into flats and together are known as Wrenbury Court. The properties are two storey with a basement and 2 small dormer windows to both the front at rear roof slopes providing accommodation at roof level. The property is clad in Bath stone, and the eaves of the hipped roof are supported by stone eaves brackets. Set back side bays form the entrances to the property. The entrance to no.16 has a flat roof, projecting porch with balustrade supported by pillars. The first floor element is set back. The entrance to no.18 is two storey with a round headed doorway. To the rear of the property there is a two storey, brick extension, which has been rendered, to the north side and a single storey extension to the south side. There is also access to the basement properties and a parking area. The property also sits within the Eldon Square Conservation Area.

Proposal:

Listed Building Consent is sought for remedial works to address water ingress and damp problems to the front porch of no.16 and the first floor bay behind and the two storey extension to the rear of the property. The works to the front porch will include the removal of the roof hatch, the repair of the lead covering and repairs to the timber decking. The works to the set-back bay behind the porch include the clearance of moss growth and the redirection of the existing rainwater goods from the main roof towards the rear of the property rather than as existing where water is directed onto the porch. Works to the two storey rear extension include the renewal of the lead flat roof and associated rain water goods, the removal and replacement of the external render and the insertion of a ‘passivent’ into the wall of the extension.

Documents Considered:

The applicant submitted the following documents, received on 25th June 2017

Drawing No: 98/0135/LB/CM – Location Plan

Drawing No: 1778-98-39 rev B – Partial Site Plan

Outline Specification – Metcalfe Briggs Surveyors

And the following, received 26th September 2017

Drawing No: 10603/01 – Existing and Proposed Elevations

Relevant Planning History:

98-00134-FUL and 98-00135-LBC - Refurbishment of Day Hospital to form 6 no dwellings, demolition of 2 storey rear extension to be replaced with smaller single storey rear extension and associated parking area. Permitted 26/08/1998

Consultations

A site notice was erected by the case Officer on the 10th August, expiring on the 31st August. Properties at 14, 14A and 20 Eldon Road were consulted and given a period of 21 days to respond. The 21 day period expired on the 11th August. No responses were received.

The RBC Heritage Officer was consulted and visited the site. The Heritage Officer commented that the works were considered appropriate to the Listed Building. The Heritage Officer advised against removing the render to the two storey rear extension unless it was essential to solving the damp issues. The agent was notified and confirmed that the surveyors that had been consulted has stated that the removal of render was essential to preventing the damp ingress. The Heritage Officer therefore was happy to agree to the removal and replacement of the render.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application. Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

LEGAL AND PLANNING POLICY CONTEXT

Section 16(2) of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires

the local planning authority to have special regard to the desirability of preserving the listed building or its setting or any features of special interest which it possesses.

Section 72 (1) of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires the local planning authority in the exercise of its functions to pay special attention to the desirability of preserving or enhancing the character or appearance of a conservation area.

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  However the NPPF does not change the statutory status of the development plan as the starting point for decision making (NPPF paragraph 12).

In this regard, the NPPF states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Core Strategy, Sites and Detailed Policies Document and Central Area Action Plan) according to their degree of consistency with the NPPF (the closer the policies in the plan to the policies in the NPPF, the greater the weight that may be given).

National Planning Policy and Guidance

National Planning Policy Framework (NPPF)

National Planning Practice Guidance

Reading Borough Local Development Framework Adopted Core Strategy (2008) (Altered 2015):

CS7     Design and the Public Realm

CS33	Protection and Enhancement of the Historic Environment

Other guidance

Eldon Square Conservation Area Appraisal

Historic England Good Practice Guide Note Number 2: managing significance in decision-taking

APPRAISAL

Historic Character and Appearance

The proposals relate to ‘repair’ type works, and are mainly fairly minor/non-intrusive in nature. It is considered that where these repairs would affect heritage features (in particular the front porch), the appropriate method and materials are to be employed, as detailed in the submitted Outline Specification.

The removal and replacement of the render to the two storey rear extension and the insertion of the passivent, will alter the fabric of the building. However given that the extension is not original to the property, is located to the rear and the proposed works will improve the appearance of the extension and prevent further water ingress/condensation, it is considered that the proposed works are acceptable.

Whilst the use of cast iron rain water goods throughout the proposal would be preferable, the proposed UPVC pipes replace existing UPVC elements and will not be easily visible.

Given the above the proposal is considered to protect and enhance the character of the listed building and is therefore considered to be in accordance with Policies CS7 and CS33 of the Reading Borough Local Development Framework Adopted Core Strategy (2008) (Altered 2015).

Conclusion

The proposed works have been assessed and found to be acceptable as they comply with the identified policies.

RECOMMENDATION:

GRANT Listed Building Consent subject to conditions

- The works to which this consent relate shall be begun not later than the expiration of three years from the date of this consent.

Reason: in accordance with Section 18 of the Planning (Listed Building and Conservation Areas) Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

- The works hereby approved shall be carried out and completed in all respects in accordance with the submitted application and the following accompanying documents:

Drawing No: 98/0135/LB/CM – Location Plan

Outline Specification – Metcalfe Briggs Surveyors

Received 25th June 2017

Drawing No: 10603/01 – Existing and Proposed Elevations

26th September 2017

Reason:  because Listed Building Consent is granted in respect of the works as detailed in the approved documents only.

- The materials and methods to be used to carry out the proposed works hereby permitted shall match those used specified in the approved Outline Specification, received 25th June 2017

Reason: in the interests of ensuring that the new development appears in keeping with the existing building in accordance with Policy CS33 and CS7

Informatives

1.	Officers have worked positively and proactively with the applicant to determine the application within a statutory timeframe for determining such applications in conjunction with the associated application for full planning permission.

2.	For the avoidance of doubt, no other works are approved except those as authorised by this Consent.  Any other works required to the Listed Building may require a separate application for Listed Building Consent and/or an application for Permitted Works to a Listed Building.