Delegated Application Report 				          Target Date: 14/06/17

Address: 12 Falmouth Road

Application No: 170621/CLP

Proposal: Loft conversion with hip-to-gable roof alteration, rear box dormer, roof light, alteration to soil and vent pipe and alteration to fenestration

Case Officer: Tom Hughes

Introduction and Proposal

The application site is a two storey semi-detached property with a hipped roof. A garage with a flat roof adjoins the side elevation and a single storey extension with a mono-pitch roof projects to the rear. The surrounding area is predominantly residential.

The proposal is for a loft conversion with a hip-to-gable roof alteration. A box dormer would project from the rearward roof slope and a roof light would be located in the forward roof slope. An existing soil and vent pipe would be altered and there would be an alteration to the existing fenestration.

The rear box dormer would project to a maximum depth of 3.4m and would extend at a width of 5.1m. The box dormer would have a maximum height of 2.6m and would be set back from the existing eaves by 0.4m. Two windows would be located to the rearward elevation. Materials have been selected to match those of the existing property.

The cubic content of the resulting roof space would exceed the cubic content of the original roof space by 38.2 cubic metres.

The altered soil and vent pipe would exceed the height of the existing roof by approximately 0.4m.

An obscure-glazed window would be located to the side elevation at the first and second floors. The window would be non-opening to 1.7m above the floor of the room in which it is located.

Plans Submitted

The agent supplied the following, received on 19th April 2017:

Drawing No: P/1721/1 – Existing Elevations and Floor Plans, Site Location and Block Plan

Drawing No: P/1721/2 – Proposed Elevations and Floor Plans

Subsequently, the agent supplied the following, received on 12th May 2017:

Drawing No: P/1721/2A – Proposed Elevations and Floor Plans

Relevant Planning History

None.

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2017

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed loft conversion with hip-to-gable roof alteration, rear box dormer, roof light, alteration to soil and vent pipe and alteration to fenestration must be assessed to ensure that it complies with the conditions of Class B, Class C and Class G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘additions etc to the roof of a dwellinghouse’, ‘other alterations to the roof of a dwellinghouse’ and ‘chimneys, flues etc on a dwellinghouse’.

The relevant conditions of the GPDO for a loft conversion with hip-to-gable roof alteration, rear box dormer, roof light, alteration to soil and vent pipe and alteration to fenestration, which is not listed or in a conservation area, are as follows:

Loft conversion with hip-to-gable roof alteration, rear box dormer and alteration to fenestration

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, n/a or (ii) 50 cubic metres in any other case; complies, the cubic content of the resulting roof space would exceed the cubic content of the original roof space by 38.2 cubic metres

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; complies, subject to Class G or

B.1 (f) the dwellinghouse is on article 2(3) land. complies

Conditions

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

Interpretation of Class B

B.3 For the purposes of Class B, “resulting roof space” means the roof space as enlarged, taking into account any enlargement to the original roof space, whether permitted by this Class or not.

B.4 For the purposes of paragraph B.2(b)(ii), roof tiles, guttering, fascias, barge boards, and other minor roof details overhanging the external wall of the original dwellinghouse are not to be considered part of the enlargement.

Roof light

C.1 Development is not permitted by Class C if:

C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

C.1 (b) the alteration would protrude more than 0.15 metres beyond the plane of the slope of the original roof when measured from the perpendicular with the external surface of the original roof; complies

C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; complies or

C.1 (d) it would consist of or include (i) the installation, alteration, or replacement of a chimney, flue or soil and vent pipe, complies, subject to Class G or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment. complies

Conditions

C.2 Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of the dwellinghouse must be:

C.2 (a) obscure-glazed; complies and

C.2 (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

Alteration to soil and vent pipe

G.1 Development is not permitted by Class G if:

G.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P, or Q of Part 3 of this Schedule (changes of use); complies

G.1 (b) the height of the chimney, flue or soil and vent pipe would exceed the highest part of the roof by 1 metre or more; complies or

G.1 (c) in the case of a dwellinghouse on article 2(3) land, the chimney, flue or soil and vent pipe would be installed on a wall or roof slope which (i) fronts a highway, n/a and (ii) forms either the principal elevation or a side elevation of the dwellinghouse. n/a

Recommendation

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed loft conversion with hip-to-gable roof alteration, rear box dormer, roof light, alteration to soil and vent pipe and alteration to fenestration.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B, Class C and Class G, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the following drawings:

Drawing No: P/1721/1 – Existing Elevations and Floor Plans, Site Location and Block Plan (received 19th April 2017)

Drawing No: P/1721/2A – Proposed Elevations and Floor Plans (received 12th May 2017)

Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Encroachment and Party Wall Act

Construction and Demolition

Materials

Bats