DELEGATED APPLICATION REPORT - Application No.: 170727

Address: 3 St. Ronan’s Road

Proposal: Erection of a 2 bedroom house

Case officer: Alison Amoah

Site Description and Surrounding Area:

The application site is adjacent to no.3 St. Ronan’s Road a semi-detached house, and comprises part of no. 3’s land and part of the former land/ garden area of a demolished bungalow (no. 1 St. Ronan’s Road).  The other part of the former bungalow land is the access road for a cul-de-sac of four recently constructed semi-detached dwellings (1a-1d St. Ronan’s Road).  The overall proposed plot is an irregular shape.  This segment of the road is characterised by semi-detached houses with front and rear gardens.

The site slopes from north up to south, i.e. the front up to the rear.

Proposal: The proposal is for a two bed house, almost identical to that refused under 141801 and the same design as the extension (approved under 111737) to create an end of terrace.   It would be set back from the front building line of 3 St. Ronan’s Road by ca 0.8m, and would be 4.2m wide and 10.3m deep.  The design would match that of the adjoining house, but with the entrance to the side.  To the side and rear is the proposed amenity space with parking for one car to the rear of the plot, accessed from the access road which serves the 1a-1d St. Ronan’s Road.  The materials proposed are to match existing.

Plans and other information considered:

Received 8th May 2017:

Location Plan – Drawing no: PL-01

Existing Block Plan – Drawing no: 01

Existing Floor Plans – Drawing no: PL-04

Existing Elevations – Drawing no: PL-05

Received 7th June 2017:

Proposed Block Plan – Drawing no: PL-03 Rev A

Proposed Floor Plans – Drawing no: PL-06 Rev A

Proposed Elevations – Drawing no: PL-07 Rev A

3D Views – Drawing no: PL-09 Rev A

Street Scene – Drawing no: PL10 Rev A

Relevant Planning History:

07/01592/FUL (Civica Ref:071445) Demolition of existing bungalow at 1 St Ronan’s Rd and development to rear gardens of both number 1 and number 3 by erecting a singular building housing 8 flats. - Withdrawn 28/3/08

08/00879/FUL (Civica Ref: 080819) - 4 dwellings at the rear of 1-3 St Ronan’s Road – Approved 4/9/08

08/00523/FUL (Civica Ref: 080088) - Demolition of 1 St Ronan’s Road to be replaced by a 1 x 2 bedroom dwelling and the erection of 4 x 3 bedroom dwellings on land and gardens at the rear of 1 and 3 St Ronan’s Road (re-submission of planning application 07/01592/FUL) – Refused 18/06/08

11/01775/FUL (Civica Ref: 111737) - Demolition of existing adjacent bungalow to enable erection of two-storey side extension – Approved 11/9/12 – informative included regarding being ancillary to the residential use of 3 St. Ronan’s Road.

141801 – Erection of a 2 bed house – Refused 27/3/15 for the following reasons: not making a positive contribution to the character of the area; a detrimental effect on the amenity of the existing dwelling; private amenity space uncharacteristically small for the area; layout fails to demonstrate that it complies with the Local Planning Authority’s standards in respect of vehicle parking, which could affect road safety and flow of traffic; insufficient information has been submitted with the planning application to enable the highways, traffic and transportation implications of the proposed development to be fully assessed; the absence of a completed legal agreement to secure an acceptable contribution towards the provision of Affordable Housing, transport, and local open space, sport and recreation improvements.

This was subject to an appeal, which was dismissed on 6//2/16 (Ref: APP/E0345/W/15/3133645).  The appeal was dismissed on the grounds of no affordable housing provision, but on all other matters: character and appearance; amenity of existing and future residents including amenity space; parking and access, other S106 obligations, the inspector concluded that the proposed scheme at that time was acceptable.

Consultation Responses: 1a-1d, 8 & 9 St. Ronan’s Road, and 174, 176, 178, 180, and 182 Waverley Road were consulted.  One objection was received as follows:

- If it's the same builders and they take as long as it took to build the other house behind our house then I would object to this due to the length of time it took them, building on Saturday and Sundays, general lack of respect for their neighbours when building.

Natural Environment: There are no trees on or adjacent to the site that are subject to a Tree Preservation Order. The site is between an existing green corridor and an area of less than 10% tree canopy cover; 2 priority locations for tree planting as defined in Reading’s Tree Strategy. Allowance should be made for soft landscaping (such as hedging or tree planting) to be incorporated into the layout to soften the frontage. The principle of this should be agreed prior to a decision but details can be left to a condition.

Officer note:   A further plan was submitted and additional comments were as follows:

I note the inclusion of the hedge on the frontage and the two trees in the back garden. I would like to see one small tree added to the hedge in the front. The trees in the back garden will need to be a minimum of 3m high at planting in order to be visible from behind the close boarded fence. Further details can be provided on condition: soft landscape plan, implementation and maintenance details.

Transport: The site is located within Zone 3, Secondary Core Area, of the Council’s adopted Parking Standards and Design SPD.  Typically these areas are within 400m of a Reading Buses high frequency ‘Premier Route’, which provides high quality bus routes to and from Reading town centre and other local centre facilities.

1 off road parking space at the rear of the development is illustrated on plans and is reached by the existing road which is 4.1m in width for its entire length and serves other properties.  In previous applications it has been agreed that one off road parking space would be deemed acceptable for the new dwelling.

Refuse for the proposed dwelling can be presented on collection days to within close proximity to the Public Highway as it currently undertaken and therefore is deemed acceptable.

The design and access statement states that cycle storage be provided for cycles in a garden shed at the rear of the development. This is therefore deemed acceptable.

Transport does not have any objections to this proposal subject to the conditions and informatives: CMS, vehicle parking, bicycle parking, bin storage and the highways informative.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Guidance

NPPF

NPPG

Reading Borough Local Development Framework Core Strategy (2008)

CS1:	Sustainable construction and design

CS7: 	Design and the Public Realm

CS9:	Infrastructure, services, resources and amenities

CS14: 	Provision of Housing

CS15:   Location, Accessibility, Density and Housing Mix

CS20: 	Implementation of Reading Transport Strategy

CS24:	Car / cycle parking

CS38:	Trees, Hedges and Woodlands

Sites and Detailed Policies Document (2012)

SD1: 	Presumption in Favour of Sustainable Development

DM1:	Adaptation to Climate Change

DM3:	Infrastructure Planning

DM4:	Safeguarding Amenity

DM5:	Housing Mix

DM6:	Affordable Housing

DM10: 	Private and Communal Outdoor Space

DM11: 	Development of Private Residential Gardens

DM12:	Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance/Documents:

Revised Parking Standards and Design SPD (2011)

Revised SPD on Planning Obligations under Section 106 (2013)

Affordable Housing (2013)

Sustainable Design and Construction SPD (2011)

APPRAISAL

Main Considerations:

- Principle of Development

- Design/ Appearance

- Alterations to existing dwelling

- Residential amenities of new and existing residents

- Landscaping

- Transport implications

- Affordable housing and Community Infrastructure Levy

- Principle of development

This is a wholly residential area and there is an existing permission on the land for a two storey side extension (which has been implemented).  The principle of a new dwelling is therefore acceptable in principle and would accord with Policy CS14: Provision of Housing, subject to meeting other policy requirements.

b)	Design/ Appearance of Development

The surrounding area is predominantly two storey semi-detached houses.  When considered under application 141801, the view was that the proposed dwelling being smaller in width and height and overall plot width would not maintain and enhance the character and appearance of the area. However, the Inspector for the appeal did not agree.  There have been no policy changes or other material considerations, since the appeal decision, which would warrant reconsideration of this matter.  Therefore the Council accepts the proposed design and appearance.

- Alterations to existing dwelling

The proposed development would be attached to the existing dwelling at no. 3 St. Ronan’s Road and a number of side facing windows at ground and first floor would be lost.  When considered under application 141801 the view was that their loss would have a detrimental effect on the amount of daylight entering it.   However, the Inspector did not agree.  There have been no policy changes or other material considerations, since the appeal decision, which would warrant reconsideration of this matter.  Therefore the Council accepts the proposed alterations.

- Residential amenities of new and existing residents

The proposed dwelling has rear facing windows at ground and first floor, and due to the alignment of the boundary to no. 3 St. Ronan’s Road it was considered (under application 1418010 that this would lead to overlooking of the garden of no. 3 St. Ronan’s Road, contrary to Policy DM4 of the Sites and Detailed Policies Document.  The Inspector concluded that “there would remain a large part of the garden, which would not be directly overlooked”.  There have been no policy changes or other material considerations, since the appeal decision, which would warrant reconsideration of this matter.  Therefore the Council accepts the level and quality of amenity space as being in accord with Policy DM10 of the Sites and Detailed Policies Document (2012).

e) 	Landscaping

The Natural Environment officer confirmed the need for tree planting within the area and that this along with hedging and other soft landscaping should be incropated into the scheme, but was happy that this could be dealt with via details to be submitted via condition.  Relevant conditions are included.

f)	Transport implications

Transport has no objection to the proposed scheme subject to conditions and informatives, which are included.  The proposal therefore accords with policies CS20, CS23, CS24 and Policy DM12.

- Affordable Housing & Community Infrastructure Levy

Policy DM6 of the Sites and Detailed Policies Document requires a contribution to allow 10% of the housing to be provided as affordable housing elsewhere in the Borough.  The Applicant has offered the full policy compliant provision which equates to £13,625.  A section 106 legal agreement will be entered into.

The proposed scheme would be CIL liable.  The applicant has submitted a self-build exemption and the informative below makes it clear that such exemption needs to be granted prior to the commencement of the development in accordance with the Regulations.  CIL does not form part of the formal determination of the planning application.

RECOMMENDATION:

APPROVE

Conditions:

- The development hereby permitted shall begin no later than three years from the date of this full planning decision.

Reason: to prevent an accumulation of unimplemented planning permissions, 	and 	in accordance with Section 91 of the Town & Country Planning Act 1990 as 	amended by the Planning and Compulsory Purchase Act 2004.

- The development hereby permitted shall be carried out in accordance with the following approved plans received 7th June 2017:

Proposed Block Plan – Drawing no: PL-03 Rev A

Proposed Floor Plans – Drawing no: PL-06 Rev A

Proposed Elevations – Drawing no: PL-07 Rev A

Reason:  because permission is granted in respect of the development as detailed in the approved plans only.

Pre-Commencement

- No development shall take place until details of the materials to be used in the construction of the external surfaces of the development including all external fixtures, fittings and window frames have been submitted to and been approved in writing by the local planning authority.  The development shall be carried out using only the approved materials.

Reason: as no details have been submitted and in the interests of the visual appearance of the building and the area in accordance with Policy CS7.

Pre-Commencement

4)	No development shall take place until full details of both hard and soft landscaping have been submitted to and been approved in writing by the local planning authority.   The submitted details shall include:

(a) hard surfacing materials; and

(b) proposed and existing functional services above and below ground (e.g drainage, power, communications cables, pipelines etc. indicating lines, manholes etc); and

(c) planting plans; written specifications (including cultivation and other operations associated with plant and grass establishment); schedules of plants, noting species, plant sizes and proposed numbers/planting densities where appropriate.

Reason: in the interest of ensuring that the landscaping of the development enhances the visual local environment and in accordance with Policy CS7

5)	The hard and soft landscaping of the development shall be carried out, in accordance with the approved landscaping plans and documents. The soft landscaping shall take place no later than during the first planting season following the date when the development hereby permitted is ready for occupation or in accordance with a timetable agreed in writing with the Local Planning Authority.

Reason: in the interests of the character and appearance of the new development and in accordance with Policy CS7

6)	All planted materials shall be maintained for five years and any trees or plants removed, dying, being severely damaged or becoming seriously diseased within 5 years of planting shall be replaced with others of similar size and species to those originally required to be planted.

Reason: in the interests of the character and appearance of the new development and in accordance with Policy CS7

Pre-commencement

7)	No development shall take place, including any works of demolition, until a 	Construction Method Statement has been submitted to and been approved in 	writing by the local planning authority to provide for:

• space where vehicles of site operatives and visitors can be parked with details 	of how site operatives and visitors will be required to make use of the parking 	area provided

• location on site for storage of plant and materials used in constructing the 	development

• the erection and maintenance (including removal of any graffiti or fly posters) 	of security hoarding around the site

• any footpath closures or road closures needed during construction

• wheel washing facilities on site

• a scheme for recycling waste resulting from the construction works

The measures within the approved Construction Method Statement shall be 	maintained and adhered to throughout the course of the development unless 	changes are agreed in advance and in writing by the local planning authority.

Reason: in the interests of protecting the amenity of local land uses or 	neighbouring residents, the character of the area and highway safety. In 	accordance with Policy DM12.

8)	The hours of noisy construction, demolition and associated deliveries shall be restricted to the hours of 08:00hrs to 18:00hrs Mondays to Fridays, and 09:00hrs to 13:00hrs on Saturdays, and not at any time on Sundays and Bank or Statutory Holidays without prior approval from the Local Planning Authority.

Reason: In the interests of the amenities of nearby residents in accordance with  Policy DM4.

9)	The dwelling(s)/building(s) hereby permitted shall not be occupied until all vehicle parking space(s) have been provided in accordance with the approved plan. The space(s) shall be kept available for parking at all times thereafter.

Reason: To ensure that the development is provided with adequate parking facilities to meet the needs of future occupiers and reduce the likelihood of roadside parking which could be a danger to other road users to accord with Policy CS23 & CS24 and Policy DM12.

10) 	No dwelling/building hereby permitted shall be occupied until the bicycle parking 	facility for that dwelling/building has been provided in accordance with the 	approved plan. The facility shall be kept available for bicycle parking at all times 	thereafter.

Reason: To encourage travel by sustainable alternatives to driving a motorcar in 	accordance with the Local Planning Authority's approved transport policies CS23 	and CS24.

11)	No dwelling/building shall be occupied until the bin storage facility shown on the 	approved plans has been provided and the bin storage facility shall be retained 	thereafter for use by occupiers of the dwelling/building at all times.

Reason: in the interests of the visual amenity of the area and to ensure that bins 	can be easily accessed when required in accordance with Policy CS7.

Informatives:

Your attention is drawn to the terms and conditions of this permission. Any development which is carried out but which differs materially from the approved plans and details, or does not comply with any condition(s) or planning obligation(s) attached to the permission may result in the Council taking action to remedy the breach of planning control. If you are in any doubt please contact the Council.

Your attention is drawn to the requirement for submission of a Building Regulations application for the proposed development. Please contact Building Control on 0118 937 2449 for advice.

i) The attention of the applicant is drawn to Section 59 of the Highways Act 1980, which enables the Highway Authority to recover expenses due to damage caused by extraordinary traffic.

ii) Any works affecting the Highway shall be in accordance with Reading Borough’s Council’s document “Guidance Notes for Activities on the Public Highway within the Borough of Reading”. The applicant should note that compliance with this document is mandatory and licences to work on the Highway will only be issued if the requirements contained within it are met. A copy can be obtained from the Council’s website.

Complaints are commonly received about noise and dust caused by construction and demolition works, particularly working outside reasonable hours and smoke from bonfires associated with the burning of waste on site.  The developer is reminded that construction and demolition associated with the approved development will be subject to environmental health law.

A section 106 Agreement has been completed for this development. A copy of the S106 Legal Agreement is available on the Council’s Public Register of Planning

The local planning authority has worked positively and proactively with the applicant to resolve relevant matters to allow permission to be granted within the prescribed target or agreed extended timescale for the decision to be issued.