### Delegated Application Report			Target Date: 09/01/2017

Address: 342 Gosbrook Road

Application No: 161915/CLP

Proposal: Single storey rear extension

Case Officer:  Alison Amoah

## Introduction & Proposal

The application site is a two storey, brick, Victorian terraced, property on the north side of Gosbrook Road. The property has a pitched roof and a two storey rear gable with a mono-pitched roof attached to an identical gable at the neighbouring property. There is also a small single storey lean-to at the rear of the rear gable

The applicant proposes to construct a single storey mono-pitched roof extension to fill the gap between the existing two storey rear gable and the boundary with the neighbouring property at 344 Gosbrook Road. The proposed extension will have a width of 1.5m and a depth of 3m. It will have a pitched roof at 2.6m high to the eaves, rising to a height of 2.9m, with a parapet to the northern end of the extension at a height of 3.2m.  There are 3. No proposed roof lights.

The materials of the extension will match those of the existing building.

Plans Submitted:

The agent submitted the following drawings, received on 10th October 2016:

Location and Block Plans - Drawing No: 342GOS/100

Existing Plans and Elevations - Drawing No: 342GOS/010

Received on 9th November 2016:

Proposed Plans and Elevations - Drawing No: 342GOS/100  Rev P5

Relevant Planning History:

160727/HPA – Single storey rear extension. Refused 27/05/2016

161020/CLP – Single storey rear extension. Approved 27/7/16.

Note: this current proposal has a change in roof form from the previous CLP at the request of the neighbour

Summary of Consultations:

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy & Guidance:

Section 10 of the Planning and Compensation Act 1991 established a procedure that enables anyone who wishes to do so, to apply to the local planning authority to determine whether a proposed use or operation, or an existing operational development or an existing use of land, or any other matter constituting a failure to comply with any condition or limitation subject to which planning permission has been granted, is lawful, and if so, be granted a certificate to that effect.

A Lawful Development Certificate is a legal document stating the lawfulness of past, present or future development. If granted by the local planning authority, the certificate means that enforcement action cannot be carried out to the development referred to in the certificate. However, the certificate will not protect from enforcement action by the planning authority if the specified use or development is then changed 'materially' without a planning application for it.

The certificate is not a planning permission. The planning merits of the use, operation or activity in the application are not relevant. The issue of a certificate depends entirely on factual evidence about the history and planning status of the building or other land and the interpretation of any relevant planning law or judicial authority. The responsibility is on the applicant to provide evidence to support the application.

The NPPG states that “In the case of applications for proposed development, an applicant needs to describe the proposal with sufficient clarity and precision to enable a local planning authority to understand exactly what is involved.  In determining an application for a prospective development under section 192 of the Town & Country Planning Act a local planning authority needs to ask “if this proposed change of use had occurred, or if this proposed operation had commenced, on the application date, would it have been lawful for planning purposes."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘the enlargement, improvement or other alteration of a dwellinghouse’.

Development is not permitted by Class A if:

- A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); COMPLIES

- A.1 (c) the height of the dwellinghouse enlarged, improved or altered would exceed the highest part of the roof of the existing dwellinghouse; COMPLIES

- A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse;  COMPLIES

- A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principal elevation of the original dwellinghouse; (ii) fronts a highway and forms a side elevation of the original dwellinghouse; NOT APPLICABLE IN THIS INSTANCE

- A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres in the case of a semi-detached house, or (ii) exceed 4 metres in height; COMPLIES, the proposal is 3m in depth and is a maximum of 3.2m in height, with the majority of the proposed extension a maximum (to top of pitch) of 2.9m

- A.1 (g) - NOT APPLICABLE IN THIS INSTANCE.

- A.1 (h) - NOT APPLICABLE IN THIS INSTANCE.

- A.1 (i) the enlarged part of the dwellinghouse would be within 2m of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3m; COMPLIES.

- A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would (i) exceed 4m in height, (ii) have more than one storey, or (iii) have a width greater than half the width of the original dwellinghouse; NOT APPLICABLE IN THIS INSTANCE.

- A.1 (k) it would consist of or include – (i) the construction or provision of a veranda, balcony or raised platform, (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse.  NOT APPLICABLE IN THIS INSTANCE.

Development is permitted by Class A subject to the following relevant conditions:

- A.3 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; COMPLIES. The extension will be finished in facing brickwork to match the existing property

- A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse shall be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed; NOT APPLICABLE IN THIS INSTANCE and

- A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, the roof pitch of the enlarged part shall, so far as practicable, be the same as the roof pitch of the original dwellinghouse. NOT APPLICABLE IN THIS INSTANCE.

Conclusion

The proposal has been assessed against Class A of Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), and it has been confirmed that the proposal would be lawful, on the basis of the following submitted drawings:

Received on 10th October 2016:

Location and Block Plans - Drawing No: 342GOS/100

Existing Plans and Elevations - Drawing No: 342GOS/010

Received on 9th November 2016:

Proposed Plans and Elevations - Drawing No: 342GOS/100  Rev P5

Therefore the Local Planning Authority will, on this occasion, issue the applicant with a Certificate of Lawfulness for the proposed development.

Recommendation:

GRANT Certificate

Informatives relating to:

  - Building regulations

  - Encroachment

  - Construction and Demolition