DELEGATED APPLICATION REPORT 				Target Date: 06/04/17

Application No: 170205/HOU (re-submission of 162277/HOU)

Address: 26 Fontwell Drive

Proposal: Two storey front extension

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site is a two storey semi-detached property with a pitched part-catslide roof. A box dormer is located within the forward roof slope. A garage with a pitched roof adjoins the northward side elevation, projecting beyond the rear elevation of the property.

The surrounding area is predominantly residential, comprised of detached and semi-detached properties of similar design, characterised by pitched catslide roofs and front box dormers. The topography of the street is such that Fontwell Drive rises steeply, so that the application site is set above and below the respective adjacent neighbours.

Proposal

The proposal is for a two storey front extension with a dual hip-to-gable roof design and various alterations to the fenestration. The existing catslide roof and front box dormer would be removed. Above the existing front projection would extend a first floor extension of the same width and depth. This element would have a height of 6.6m, with a maximum height to eaves of 5m. To the principal elevation, this element would have a maximum height to eaves of 4m.

Aside this projection, a two storey extension would extend at a width of 3.8m at the ground floor, flush with the original northward side elevation. At the ground floor the proposal would project to a depth of 2m. From the front elevation of this projection, a porch would project to an additional depth of 0.4m at a width of 2m. The porch would have a hipped roof at a height of 3.4m with a height to eaves of 2.3m. The proposed roof would project as a canopy to an additional depth of 0.4m. At the first floor the extension would be set back by 0.8m, with the provision of a mono-pitch above the ground floor at a height of 1.6m with a height to eaves of 2.3m. This element of the proposal would have a total height of 6.6m with a maximum height to eaves of 4.5m. To the principal elevation, this element would have a maximum height to eaves of 4m.

Located within the proposed forward roof slope would be two box dormers. The northward box dormer would project to a depth of 0.4m and would extend at a width of 2.2m at a height of 0.8m. The southward box dormer would project to a depth of 0.6m and would extend at a width of 2.2m at a height of 1m.

The existing ground floor window to the principal elevation would be enlarged. A further window would be located at the ground floor to the principal elevation. A window would be located to the southward side elevation of the proposed porch. Two further windows to the southward side elevation of the original property, at the ground and first floors. The single door to the southward side elevation of the garage would be renewed. To the rear elevation of the property, all existing windows would be renewed. To the northward side elevation of the property, all existing windows would be renewed. A further window would be located at the ground floor.

Materials have been selected to match those of the existing property.

Plans Considered

The agent supplied the following, received on 9th February 2017:

Drawing No: 6527:16:1 Rev. A – Existing and Proposed Elevations, Floor Plans, Site Plans and Site Location

Relevant Planning History

162277/HOU – Part one, part two storey front extension and various alterations to the fenestration – Refused 27/01/17

Consultation Responses

Ecology raised no objection to the proposal.

Transport raised no objection to the proposal, subject to an informative regarding damage to the highway.

Properties at 15, 24 and 28 Fontwell Drive. No letters of representation were received.

Summary of discussions with applicant/agent

The agent was advised on 8th March 2017 that the proposal had not overcome the reasons for refusal of application 162277/HOU. The revised proposal would still completely overcome the form of the host property, to the detriment of its character and appearance. The proposal would not be a subservient or sympathetic addition and would not respect the existing building line. The resultant property would appear jarring and unbalanced within the street scene. The proposed development would therefore be contrary to the relevant policy and guidance.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

The main issues to be considered are:

- Design and impact on the character of the surrounding area

- Impact on neighbours

Design and impact on the character of the surrounding area

Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document (2012) states that an extension to a house should respect the character of the house in terms of scale, location, materials and design, and respect the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line. Extensions to the side and front of houses are particularly visible. It is important that they reflect the character of the street, to ensure that the extension is not visually dominant in a way that detracts from the surroundings. Extensions should reflect the building line or preferably, be set back from the building line. Policy CS7 (Design and the Public Realm) of the Core Strategy also states that all development must be of high design quality that maintains and enhances the character and appearance of the area. Development must respond positively to the local context and reinforce local character and distinctiveness. The Council’s Supplementary Planning Guidance – A Design Guide to House Extensions, states that proposals for front extensions require particular attention to detail to avoid failing the criteria relating to the protection of character of neighbouring properties and the street generally and should achieve good, compatible design. In addition, details such as materials, fenestration and roof pitch should complement the original buildings design, age and scale.

The application relates to a detached property that shares the same design as that predominant in the surrounding area. Slight amendments have been made to the proposal of application 162277/HOU. The projection of the porch has been reduced, the height to eaves has been partially reduced and two flat roof dormers have been incorporated into the forward roof slope. The amendments do not overcome the reasons of harm identified by the previous refusal. The proposal would still completely overcome the form of the host property, to the detriment of its character and appearance. The proposal would not be a subservient or sympathetic addition and would not respect the existing building line. The resultant property would appear jarring and unbalanced within the street scene. The proposed development is therefore contrary to Core Strategy Policy CS7 (Design and the Public Realm), Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

Impact on neighbours

The application site is a splayed plot on the outside of a bend in Fontwell Drive. The road rises steeply in a northerly direction, such that the ground floor level of 26 Fontwell Drive approximately equates to first floor level at 24 Fontwell Drive. It is not considered that the proposal would have a detrimental impact with regard to loss of light and overbearing effects due to the distance to the adjacent neighbouring properties.

The agent has advised that windows to the southward side elevation would be obscure-glazed. This would mitigate the concern of overlooking into private living areas and the garden of the neighbouring property at 24 Fontwell Drive. As such, the proposal would be generally in accordance with Policy DM4: Safeguarding Amenity of the Sites and Detailed Policies Document.

RECOMMENDATION

REFUSE

Informatives relating to:

- Refused plans

- Positive and proactive