DELEGATED APPLICATION REPORT 			             Target Date: 27/08/17

Application No: 171189/HPA

Address: 29 Stockton Road

Proposal: Single storey rear extension

Case officer: Tom Hughes

### Introduction

Under Class A, Part 1, of Schedule 2 to the Town and Country Planning (General Permitted Development) (England) Order 2015 (“the 2015 Order”) the following is allowed:

A single storey extension, beyond the rear wall of a house, between 3m and 6m in depth, and up to 4m in height

provided the development is not excluded by virtue of the criteria in Paragraph A.1 of the 2015 Order.

The application site is not within a Conservation Area nor have PD rights been withdrawn or restricted by way of a direction made under Article 4 or condition attached to an express planning permission.

### Proposal

The proposal falls to be considered under Paragraph A.1 (g) and accordingly Condition A4 applies.

The agent has provided information on the proposed single storey rear extension. The extension would:

- Extend beyond the rear wall of the original dwelling by 6m

- Have a maximum height of 2.8m

- Have an eaves height of 2.8m

The proposal would adjoin a part one, part two storey side and rear extension approved under application 151096/HOU. The proposed element would project to an additional depth of 2m along the boundary with 27 Stockton Road, extending at a width of 3.8m. The proposal would have a flat roof at a height of 2.8m. A window and a single door would be located to the rear elevation, with a further window located to the side elevation.

Site Description and Surrounding Area

The application site is a two storey semi-detached property with a hipped roof. A part one, part two storey side and rear extension was approved under application 151096/HOU. A hip-to-gable roof alteration and rear box dormer was approved under application 161708/CLP. The surrounding area is predominantly residential.

Plans Submitted

The agent submitted the following, received on 17th July 2017:

Site Location

Block Plan

Drawing No: MC/2001/01 – Proposed Floor Plans and Elevations

Planning History

151096/HOU – Part one, part two storey side and rear extension – Permitted 05/10/15

161708/CLP – Hip-to-gable roof alteration and rear box dormer – Permitted 21/10/16

Notification

In accordance with the relevant provision of the GPDO 2015 Order, the LPA notified on 19th July 2017 the adjoining owners and occupiers at 27 and 31 Stockton Road and 34 and 36 Camelford Road of the proposed extension providing a period of 21-days for representations to be made. The 21 day consultation period expired on 9th August 2017. No letters of representation have been received.

Policies

Legal and Planning Policy Context

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise. Material considerations include relevant policies in the National Planning Policy Framework (NPPF) – among them the ‘presumption in favour of sustainable development’.

The following local and national planning policy and guidance is relevant to this application:

Sites and Detailed Policies Document (2012)

SD1: Presumption in Favour of Sustainable Development

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Appraisal

The proposed development has been assessed under Class A, Part 1, of Schedule 2 to The Town and Country Planning (General Permitted Development) (England) Order 2015. Under A.1 (ja) – ‘Development is not permitted by Class A if any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (e) to (j)’.

The proposal would adjoin the enlargement approved under application 151096/HOU. The enlargement approved under application 151096/HOU constitutes a part one, part two storey side and rear extension. The total enlargement extends beyond a wall forming a side elevation of the original dwellinghouse. The maximum height of the total enlarged part is 6m. This is contrary to A.1 (j) (i). The total enlargement would have two storeys. This is contrary to A.1 (j) (ii). The total enlargement would have a maximum width of 7.3m. The width of the original dwellinghouse is 5.9m. The width of the total enlargement is therefore greater than half the width of the original dwellinghouse, which is contrary to A.1 (j) (iii).

As such, the proposal would exceed the limits of A.1 (j) – ‘A.1 (j) Development is not permitted by Class A if the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would (i) exceed 4 metres in height, (ii) have more than a single storey, or (iii) have a width greater than half the width of the original dwellinghouse’.

Recommendation

Refuse prior approval

The proposed development has been assessed under Class A, Part 1, of Schedule 2 to The Town and Country Planning (General Permitted Development) (England) Order 2015. The development would not meet the criteria of Paragraph A.1 (ja) - ‘Development is not permitted by Class A if any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (e) to (j)’ and the criteria of Paragraph A.1 (j) - ‘A.1 (j) Development is not permitted by Class A if the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would (i) exceed 4 metres in height, (ii) have more than a single storey, or (iii) have a width greater than half the width of the original dwellinghouse’.

Consideration was given to the following information received by the Local Planning Authority on 17th July 2017:

Site Location

Block Plan

Drawing No: MC/2001/01 – Proposed Floor Plans and Elevations