### Delegated Application Report					Target Date: 16/06/2017

Address: Caversham House, 13-17 Church Road

Application No: 170624/ADV

Proposal: 3 internally illuminated fascia signs and 1 internally illuminated projecting sign

Case Officer: Heather Banks

## Site Description and Surrounding Area:

The application site is a modern residential development with commercial units at ground floor level fronting Church Road. The building is located towards the western end of Caversham High Street. The unit under consideration for the proposed advertisements is currently vacant but is intended to be occupied by Sevenoaks Sound and Vision. The other ground floor commercial unit is occupied by Reflex spinal health. The signage for this business consists of a single non-illuminated fascia sign. The neighbouring properties are mainly in Class A1 (retail) use and the site is part of Caversham District Centre. The existing ground floor frontage of the property is not a designated Key frontage in Caversham District Centre. The property sits opposite the St Peter’s Conservation Area and the listed buildings of 14 (Valentine Cottage) and 16 Church Road.

Proposal:

The application seeks advertisement consent for three internally illuminated fascia signs and one internally illuminated projecting sign.

It is proposed that the fascia signs will be fitted into the existing timber shop front onto the angled timber fascia boards. The fascia signs will be internally illuminated with LED illumination with a depth of 0.08m and an illumination level of 500cd/m. The sign will be constructed from folded black aluminium with push through opal acrylic lettering.

The proposed projecting sign will be located to the vertical ‘pillar’ detail to the east of the shop front entrance. The sign will have a height of 0.6m a depth of 0.01m and a maximum projection from the face of the building of 0.94m. Again the sign will be constructed from folded black aluminium with push through opal acrylic lettering. The sign will also be internally illuminated with LED illumination with an illuminance level of 500cd/m.

Plans Submitted:

The agent supplied the following plans, received on 19th April 2017

Location Plan

Drawing No: 112559 sheet 1/4 version 4 – Site photos

Drawing No: 112559 sheet 2/4 version 4 – Illuminated left fascia sign

Drawing No: 112559 sheet 3/4 version 4 – Illuminated central fascia sign and double sided internally illuminated projecting sign

Drawing No: 112559 sheet 4/4 version 4 – Illuminated right fascia sign

Relevant Planning History:

160838-FUL - Change of use of part of the ground floor from A1/A2 to B1(a). Refused 11/08/2016

160208-OPA - Change of use of Ground Floor from Class B1(a) (offices) to C3 (dwelling houses) to comprise of 2 x 1 bed flats. Prior Notification under Class O, Part 3 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015. Refused 05/04/2017

152176-FUL - Change of use of part of ground floor from B1(a) to 2 x 1-bed flats. Permitted 05/07/2016

151518-OPA - Change of use of ground floor from B1(a) (offices) to C3 (dwelling houses) to comprise of 2 x 1-bed flats.  Prior Notification under Class 0, Part 3 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015. Refused 27/10/2015

150329-APPCON - Discharge of Conditions 3, 4, 10 and 20 of planning permission 130609/FUL. Conditions discharged 08/06/2015

150176-FUL - Change of use of ground floor from that approved by application 130609 as A1/A2 to a mixed use of A1/2, D1 and B1(a). Minor fenestration changes to facilitate the new uses and reorganisation of previously approved external parking and amenity space. Permitted 14/07/2015

130609-FUL - Redevelopment comprising 14 residential apartments on the first, second and third floors and 2 commercial units on the ground floor (Classes A1 & A2) associated parking and private amenity space. Permitted 28/10/2013

Consultations

A site notice was erected on 8th May 2017, expiring on 29th May 2017. One response was received. The response objected to the proposal for the following reasons:

- Too much illumination

- Too much signage

- Too much intrusion into already busy bottle neck

- Adverse impact on the 16 private dwellings which form much of the rest of Caversham House

Case Officer Response:

With regard to the impact of the illuminance level of the advertisements on the surrounding area and residential properties, officers consider policy DM22, with the supporting text referencing  guidance provided in the publication produced by the Institute of Lighting Engineers ‘Brightness of Illuminated Advertisements’. This guidance is helpful when making a decision as to whether the illuminance level is appropriate. This publication recommends that the following values be used as a guide for maximum luminance levels (these values differ based on the environment in which the host property is located):

| Illuminated Area (m2) | Illuminance level (cd/m2) Zone E1 | Illuminance level (cd/m2) Zone E2 | Illuminance level (cd/m2) Zone E3 | Illuminance level (cd/m2) Zone E4 |
| --- | --- | --- | --- | --- |
| Up to 10.00 | 100 | 600 | 800 | 1000 |
| Over 10.00 | n/a | 300 | 600 | 600 |

E1: Intrinsically dark areas (e.g. National Parks, Area of Outstanding Natural Beauty or other dark landscapes)

E2: Low district brightness areas (e.g. rural or small village locations)

E3: Medium district brightness areas (e.g. small town centres, urban locations)

E4: High district brightness areas (e.g city and town centres with high levels of night time activity)

It is considered that the application property falls within Zone E3. Given that the black aluminium of most of the signage is opaque and only the acrylic lettering will allow the illumination to pass through, it is considered that the proposed advertisements have an illuminated area of less than 10m2. It is therefore recommended that the advertisement should have an illuminance level of no greater than 800cd/m2. The illuminance level of the proposed advertisements will be lower than this, at 500cd/m2, and even if one considered the full area of the signage and not just the illuminated area, the illuminance level would still fall below the suggested 600cd/m2. Therefore it has been concluded that the illuminance would be unlikely to have a detrimental effect on the amenity of neighbouring properties. The implementation of the proposed level of illuminance, 500cd/m2 is to be secured by condition.

It is not considered that the proposal would form an over-abundance of signage. The application property was designed with blank fascia boards where the signage is to be fixed and it is therefore considered that it was always intended for signage to be located here.

Summary of discussions with applicant/agent:

None

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance and Legal Context

The following local and development plan policies and supplementary planning guidance are relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008) (Altered 2015) Policies:

CS7: Design and the Public Realm

CS20: Implementation of the Reading Transport Strategy

CS33: Protection and Enhancement of the Historic Environment

Sites and Detailed Policies Document (2012) (Altered 2015)

DM12: Access, Traffic and Highway-Related Matters

DM22: Advertisements

Legal context

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.

Regulation 3 of the Town and Country Planning (Control of Advertisements) (England) Regulations 2007 requires the Local Planning Authority to exercise its powers under these regulations in the interests of amenity and public safety taking into account the provisions of the development plan, so far as they are material; and any other relevant factors.  Regulation 3 states that factors relevant to amenity include the general characteristics of the locality, including the presence of any feature of historic, architectural, cultural, or similar interest.

LBCA 1990 duty to consider impact on conservation areas.

Factors relevant to public safety include highway safety and whether the advert would hinder security or surveillance devices, including speed cameras.

Appraisal

The main issues are considered to be:

i)	Amenity

ii)	Public safety

i) Amenity

The advertisements on nearby properties mainly seem to be non-illuminated signage consisting of awnings with painted timber fascia boards to the shopfront itself. Those properties without awnings also tend to be non-illuminated signage painted to the timber shop front. The proposed signage would therefore not be in keeping with the general character of the surrounding shopfronts. However, the application property is not a listed building or in a Conservation Area but is rather a modern building and there are many examples of internally illuminated fascia throughout Caversham High Street. Also the proposed fascia would sit be the same size as the timber fascia boarding of the existing shop fronts, working with the existing style of the property. Furthermore, the signage would be a relatively slim design which would not be considered to add an excessively bulky appearance to the shop front. It is therefore considered that although not necessarily in keeping with the character of the area, the proposed signage would not cause sufficient harm to the visual amenity of the area to warrant the refusal of the application.

ii) Public Safety

The proposed signage is located in a position that would not cause a physical obstruction to members of the public using the highway.  The illuminated elements to the fascia will not, as per information submitted by the applicant, exceed a level of luminance of 500 cd/m2, which is unlikely to cause a distraction to users of the highway. As such the proposal is considered to be acceptable in public safety terms in accordance with DM12: Access, Traffic and Highway-Related Matters of the Sites and Detailed Policies Document (2012).

# Conclusion

The proposed advertisements are acceptable for the reasons set out above.

# RECOMMENDATION

GRANT Advertisement Consent

Conditions

- This consent relates to the following drawings only, received on 19th April 2017

Location Plan

Drawing No: 112559 sheet 2/4 version 4 – Illuminated left fascia sign

Drawing No: 112559 sheet 3/4 version 4 – Illuminated central fascia sign and double sided internally illuminated projecting sign

Drawing No: 112559 sheet 4/4 version 4 – Illuminated right fascia sign

Reason: For the avoidance of doubt and to clarify which drawings are approved.

- The maximum luminance from the illuminated fascia and projecting signs should not exceed 500 cd/m2 .

Reason: To avoid glare, which could lead to danger to highway users and in the interests of visual amenity in accordance with Policy CS20 of the Reading Borough LDF Core Strategy 2008 (Altered 2015) and Policies DM12 and DM22 of the Sites and Detailed Policies Document 2012 (Altered 2015).

- The Standard Conditions:

i. No advertisement is to be displayed without the permission of the owner of the site or any other person with an interest in the site entitled to grant permission.

Reason: In accordance with Schedule 2, Regulation 2(1) of the Town and Country Planning (Control of Advertisements) (England) Regulations 2007.

ii. No advertisement shall be sited or displayed so as to:

a) endanger persons using any highway, railway, waterway, dock, harbour or aerodrome (civil or military);

b) obscure, or hinder the ready interpretation of, any traffic sign, railway signal or aid to navigation by water or air; or

c) hinder the operation of any device used for the purpose of security or surveillance or for measuring the speed of any vehicle.

Reason: in accordance with Schedule 2, Regulation 2(1) of the Town and Country Planning (Control of Advertisements) (England) Regulations 2007.

iii. Any advertisement displayed, and any site used for the display of advertisements, shall be maintained in a condition that does not impair the visual amenity of the site.

Reason: In accordance with Schedule 2, Regulation 2(1) of the Town and Country Planning (Control of Advertisements) (England) Regulations 2007.

iv. Any structure or hoarding erected or used principally for the purpose of displaying advertisements shall be maintained in a condition that does not endanger the public.

Reason: In accordance with Schedule 2, Regulation 2(1) of the Town and Country Planning (Control of Advertisements) (England) Regulations 2007.

v. Where an advertisement is required under these Regulations to be removed, the site shall be left in a condition that does not endanger the public or impair visual amenity.

Reason: In accordance with Schedule 2, Regulation 2(1) of the Town and Country Planning (Control of Advertisements) (England) Regulations 2007.

Informatives

1. Your attention is drawn to the terms and conditions of this consent. Any advertisement displayed which differs materially from the approved plans and details, or does not comply with any condition(s) attached to the consent may result in the Council taking action to remedy the breach of advertisement control. If you are in any doubt please contact the Council.

2. The decision to grant consent follows the positive and proactive consideration of the issues raised including issuing the decision within the eight week time period.