### Delegated Application Report				Target Date: 17/07/2017

Address:  29 Cardiff Road

Application No: 170819/CLP

Proposal: Single storey rear extension

Case Officer: Heather Banks

## Introduction & Proposal

The application site is a two storey, terrace, brick property on the south side of Cardiff Road. The application property has an existing long, partial width flat roof rear extension and the property backs onto the railway embankment.

The applicant proposes to construct a single storey, flat roof extension to the rear of the property. The extension is to sit adjoining the main house, the existing rear projection and the boundary with the neighbouring property 27 Cardiff Road. This is to be one obscure glazed window to the rear elevation of the proposed extension

Plans Submitted:

The agent supplied the following drawings:

Drawing No: 2017-355-203 – Location Plan

Drawing No: 2017-355-203 – Existing and Proposed Block Plans

Drawing No: 2017-355-101 – Existing

Drawing No: 2017-355-102 - Proposed

Received 22nd May 2017

Relevant Planning History:

170422/HOU – Single storey rear extension. Withdrawn 18/05/2017

Summary of Consultations:

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy & Guidance:

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions of Classes A of Part 1, of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relate to ‘the enlargement, improvement or other alteration of a dwelling house’.

Development is not permitted by Class A if:

- A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); COMPLIES

- A.1 (c) the height of the dwellinghouse enlarged, improved or altered would exceed the highest part of the roof of the existing dwellinghouse; COMPLIES

- A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; COMPLIES, eaves height will be 2.6m

- A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principal elevation of the original dwellinghouse; (ii) fronts a highway and forms a side elevation of the original dwellinghouse; NOT APPLICABLE IN THIS INSTANCE

- A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres in the case of a semi-detached house, or (ii) exceed 4 metres in height; COMPLIES, the proposal is to be 3m in depth and is 2.6m in height.

- A.1 (g) - NOT APPLICABLE IN THIS INSTANCE.

- A.1 (h) - NOT APPLICABLE IN THIS INSTANCE.

- A.1 (i) the enlarged part of the dwellinghouse would be within 2m of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3m; COMPLIES, the proposed single storey rear extension measures 2.6m in height to eaves level.

- A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would (i) exceed 4m in height, (ii) have more than one storey, or (iii) have a width greater than half the width of the original dwellinghouse; COMPLIES

- A.1 (k) it would consist of or include – (i) the construction or provision of a veranda, balcony or raised platform, (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse.

Development is permitted by Class A subject to the following relevant conditions:

- A.3 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; COMPLIES. Email received 13th July from agent states that exterior materials will match those of the existing property.

- A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse shall be – (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed; NOT APPLICABLE IN THIS INSTANCE and

- A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, the roof pitch of the enlarged part shall, so far as practicable, be the same as the roof pitch of the original dwellinghouse. NOT APPLICABLE IN THIS INSTANCE.

It should be noted that the single storey rear extension, the new ground floor openings to the side elevation of the property and the enlarged first floor rear window fall to be considered under Class A. Most of the conditions of Class A refer to extensions rather than alterations and as such the answers above are mostly related to the proposed rear extension. The new and enlarged openings comply with Class A as the side elevation openings are at ground level and the first floor opening is rear facing meaning that condition A.3(b) does not apply and the restriction stated in this condition are not required for these openings.

Conclusion

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), and it has been confirmed that the proposal would be lawful, on the basis of the following submitted drawings received on 22nd May 2017:

Drawing No: 2017-355-203 – Location Plan

Drawing No: 2017-355-203 – Existing and Proposed Block Plans

Drawing No: 2017-355-101 – Existing

Drawing No: 2017-355-102 - Proposed

Therefore the Local Planning Authority will, on this occasion, issue the applicant with a Certificate of Lawfulness for the proposed development.

Recommendation:

GRANT Certificate

Conditions

As conclusion.

Informatives

- The applicants attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

- During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at: http://www.communities.gov.uk/publications/planningandbuilding/partywall

- The construction (and demolition) of the development will be subject to environmental health law.

Location Plan (not to scale)