### Delegated Application Report				Target Date: 02/10/2017

Address: 2 Overton Drive

Application No: 171357/CLP

Proposal: Certificate of lawfulness for proposed construction of a rear dormer roof extension, two front facing roof lights and a side facing window.

Case Officer: Heather Banks

## Introduction & Proposal

The application site is a two storey, pitched roof, link detached property which forms part of the Overton Drive development which was granted permission in 2007 under appeal decision APP/E0345/A/07/2035298 (application no: 06/01424/FUL).

The applicant proposes to construct a flat roof dormer extension to the rear roof slope to the property. The dormer is to be set in 0.6m from the eaves of the roof (measured along the roof slope) and will have a maximum height set 0.6m below the ridge line of the main roof. The dormer will have a width of 4.4m. The two front facing roof lights will not project more than 150mm from the roof slope of the property. The side facing window will provide light to the stairwell providing access to the loft conversion. It will face towards 3 Overton Drive and will be fixed shut and obscure glazed.

Plans Submitted:

The agent supplied the following drawings, received 7th August 2017

Location Plan

Drawing No: FL 1633-1 – As existing

Drawing No: FL 1633-2 – As proposed

Relevant Planning History:

06/01424/FUL - Erection of 1 no. 2-bedroomed house; 4 no. 3-bedroomed houses; 4 no. 4-bedroomed houses, new car port to no. 10 All Hallows Road and replacement electricity sub-station. Non Determination. Appeal APP/E0345/A/07/2035298 allowed

Summary of Consultations:

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy & Guidance:

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions Class B and C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘the enlargement of a dwelling house consisting of an addition or alteration to its roof’ and ‘other alterations to the roof of a dwelling house’.

Development is not permitted by Class B if:

- B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES.

- B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; COMPLIES.

- B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; COMPLIES.

- B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than 40 cubic metres in the case of a terraced house; COMPLIES, volume is approx. – 23.2 cubic metres.

- B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; COMPLIES.

or

- B.1 (f) the dwellinghouse is on article 2(3) land.  COMPLIES.

Development is permitted by Class B subject to the following conditions:

- B.2 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; COMPLIES. Drawing No: FL 1633-2 – As proposed states that the dormer is to be clad in concrete plain tiles to match the colour of the existing roof tiles. The agent also confirmed via email on the 29th September that the proposed external materials will match the appearance of those of the existing property.

- B.2 (b) the enlargement must be constructed so that –

- other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension – (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; COMPLIES, and

- other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; COMPLIES.

- B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and (ii) non-opening unless the parts of the windows which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. COMPLIES

Development is not permitted by Class C if:

- C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES

- C.1 (b) the alteration would protrude more than 0.15m beyond the plane of the slope of the original roof; COMPLIES

- C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; COMPLIES or

- C.1 (d) it would consist of or include (i) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment. COMPLIES

Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of a dwellinghouse shall be:

- C.2 (a) obscure glazed; and (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. NOT APPLICABLE IN THIS INSTANCE.

Conclusion

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B and Class C of , Part 1 of, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), and it has been confirmed that the proposal would be lawful, on the basis of the following submitted drawings:

Location Plan

Drawing No: FL 1633-1 – As existing

Drawing No: FL 1633-2 – As proposed

Received 7th August 2017

Therefore the Local Planning Authority will, on this occasion, issue the applicant with a Certificate of Lawfulness for the proposed development.

Recommendation:

GRANT Certificate

Conditions

As conclusion.

Informatives

- The applicant’s attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

- During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at: http://www.communities.gov.uk/publications/planningandbuilding/partywall

- The construction (and demolition) of the development will be subject to environmental health law.

- Work on a loft or roof may affect bats.  The applicant will need to consider protected species when planning work of this type.  A survey may be needed, and if bats are using the building, a licence may be required.  To view specific information regarding bats use the following link:

https://www.gov.uk/wildlife-licences

Site Map