DELEGATED APPLICATION REPORT 				Target Date: 16/01/17

Application No: 162194/HOU

Address: 41a Coley Avenue

Proposal: Single storey rear extension

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site a two storey detached property with a large rear garden. A conservatory/store projects to a depth of 2.5m at the rear. Beyond this, a further store is located on the northern boundary with the adjacent neighbour at 39 Coley Avenue. A detached garage with a pitched roof is located on the opposite side of the boundary wall. A conservatory is located in close proximity to the property to the southern boundary, within the curtilage of 41 Coley Avenue. The surrounding area is predominantly residential.

Proposal

The proposal is for a single storey rear extension. The existing conservatory/store and store would be demolished. The proposal would project to a depth of 6m, extending across the full width of the property by 10.1m. The proposal would have a flat roof at a height of 2.8m. A parapet wall would be located above the rear elevation, at a total height of 3m. Within the flat roof would be located two roof lights, and projecting from it would be a roof lantern to a total height of 3.4m. Bi-fold doors and a window would be located on the rear elevation, with a further window on the northward side elevation. Materials have been selected to match the existing property.

Plans Considered

The agent submitted the following, received on 21st November 2016:

Drawing No: JSD-15-74/01 – Site Location and Block Plan

Drawing No: JSD-15-74/02 – Existing Floor Plan and Roof Plan

Drawing No: JSD-15-74/03 – Existing Elevations

Drawing No: JSD-15-74/04 – Proposed Floor Plan and Roof Plan

Drawing No: JSD-15-74/05 – Proposed Elevations

Relevant Planning History

None.

Consultation Responses

Properties at 39 and 41 (Flats 1-6) Coley Avenue and 36 Mansfield Road were consulted. One letter of representation was received with regard to the following:

- Materials should match the existing property Officer note: As per the application form, materials have been selected to match those of the existing property. This will be secured by condition to ensure that the proposal accords with DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document (2012)

Summary of discussions with applicant/agent

None.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building;

- reduce the amount of usable amenity space for the property to an unacceptable degree;

- require the loss of trees in the garden or put significant trees at risk;

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties;

- invade the privacy of neighbouring properties through overlooking from windows or balconies;

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design;

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing;

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking;

- Access to sunlight and daylight;

- Visual dominance and overbearing effects of a development;

- Noise and disturbance;

- Artificial lighting;

- Vibration;

- Dust and fumes;

- Smell;

- Crime and safety;

RECOMMENDATION

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

Informatives relating to:

- Terms and conditions

- Building regulations

- Demolition and construction

- Encroachment and Party Wall

- Positive and proactive