DELEGATED APPLICATION REPORT 				Target Date: 11/01/17

Application No: 162015/HOU

Address: 3 Wendover Way

Proposal: Single storey side and rear extension

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site is a two storey semi-detached property with a hipped roof. An existing conservatory projects the rear at a depth of 3.8m and height of 3m, set 0.3m off the boundary with the adjoining neighbour at 2 Wendover Way. A two storey gable projects to the rear at a depth of 0.9m at a width of 2.5m from the southeast side elevation. A detached garage is located beyond the side and rear elevations. The surrounding area is predominantly residential. The adjacent neighbour at 4 Wendover Way has a similar side extension to that which is proposed.

Proposal

The proposal is for a single storey side and rear extension. The existing conservatory would be demolished. Set back from the principal elevation by 0.2m, the side extension would extend at a width of 1.3m to a total depth of 12m. A distance of approximately 1m would be maintained to the boundary with the adjacent neighbour at 4 Wendover Way. The proposal would project past the rear elevation of the two storey rear gable by 3.6m and extend across the full width of the property by 7m. A distance of 0.3m would be maintained to the boundary with the adjoining neighbour at 2 Wendover Way. Along the side elevation, the proposal would have a part hipped mono-pitch roof at a height of 3.8m, with a height to eaves of 2.5m. To the rear of the property the proposal would have a part flat, part hipped roof with the roof height and height to eaves maintained. The flat roof would contain a roof light. Windows would be located on the principal and side elevations, with a window and a set of double doors on the rear elevation. Materials have been selected to match those of the existing property.

Plans Considered

The agent submitted the following, received on 16th November 2016:

Drawing No: PL-01 – Site Location and Existing Block Plan, Ground Floor Plan and Elevations

Drawing No: Pl-02 – Proposed Ground Floor Plan

Drawing No: PL-03 – Proposed Elevations and Proposed Block Plan

Relevant Planning History

None.

Consultation Responses

Properties at 2 and 4 Wendover Way were consulted. No letters of representation were received.

Summary of discussions with applicant/agent

None.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; No, the proposal is sufficient in its design and proportion that it doesn’t detract from the original building’s appearance, despite posing a considerable alteration to the principal elevation

- reduce the amount of usable amenity space for the property to an unacceptable degree; No

- require the loss of trees in the garden or put significant trees at risk; No

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; No, the depth of 4.5m set 0.3m from the boundary with the adjoining neighbour at 2 Wendover Way and the height of 3.8m is not considered harmful to this neighbour. Although the proposal would exceed the Council’s guidance of a 4m rear extension, it would only be to a marginal degree. Given the size and depth of the existing rear conservatory, on balance the proposal is considered to be acceptable. The proposal is not considered to reduce the amount of light to the adjacent neighbour at 4 Wendover Way

- invade the privacy of neighbouring properties through overlooking from windows or balconies; No, there are existing windows on the side elevation at the ground and first floors, and the proposal is not considered anymore harmful to the residents of neighbouring properties than the existing arrangement. The proposed window on the side elevation will be obscure-glazed, as secured by condition

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; No, the proposal would deviate from the general norm of the neighbouring properties, though the example of the similar extension at the adjacent neighbour at 4 Wendover Way eases any concerns held about inconsistent design.

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; No, materials have been selected to match those of the existing property

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; No, see comments above

- Access to sunlight and daylight; No, see comments above

- Visual dominance and overbearing effects of a development; No, although the neighbouring properties will notice the additional height and depth of the proposal, it is not considered to be visually dominant or overbearing

- Noise and disturbance; No

- Artificial lighting; No

- Vibration; No

- Dust and fumes; No

- Smell; No

- Crime and safety; No

RECOMMENDATION

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

- Windows

Informatives relating to:

- Terms and conditions

- Building regulations

- Demolition and construction

- Encroachment and Party Wall

- Positive and proactive