DELEGATED APPLICATION REPORT 				Target Date:	23/03/2017

Application No: 170104/HOU

Address: 28 Torrington Road, Whitley, Reading

Proposal: Part first floor rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey mid-terrace property within a row of four properties.  The property has a single storey rear extension.  The surrounding area is predominately residential.

Proposal:

The proposal is for a first floor rear extension measuring 2.8m in depth and 3.9m in width.  The proposed eaves height is the same as the existing property and the proposal is set down from the existing roof by 0.3m.

The proposed extension is 1.8m from the boundary with 26 Torrington Road and 1.5m from the boundary with 30 Torrington Road.

The proposal has a hipped roof with a rear window.

The existing property is constructed of brick and the proposed materials consist of tile hanging.  The agent has confirmed the tiles will be flat interlocking clay plain tiles called 20/20 Flanders which are similar in colour to the brick on the existing property.  These hanging tiles have been proposed as it is a light weight construction based on timber framing and as the proposal would sit above an existing, recently built, single storey rear extension the agent has stated it would require less work internally and would be more economical to build.

The tiles to the roof of the proposed extension would match the existing roof of the property.

Plans Considered:

The agent submitted the following drawings, received on 24th January 2017:

Drawing No: PL/338/101 – Existing Floor Plans

Drawing No: PL/338/102 – Existing Elevations

Drawing No: PL/338/103 – Proposed Floor Plans

Drawing No: PL/338/104 – Proposed Elevations

Drawing No: PL/338/105 – Location & Site Plan

Relevant Planning History:

151650/HOU - Single storey rear extension.  Permitted 12/11/2015.

152221/HPA - Rear extension measuring 4.5m in depth, with a maximum height of

3m, and 3m in height to eaves level. Notification of the construction of an extension under class A Part 1 of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015.  Prior Approval Not Required 14/01/2016.

Consultation Responses:

Ecology raised no objection.

Properties at 26 and 30 Torrington Road and 23, 25 and 27 Blagdon Road were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The agent advised that matching materials were not being proposed due to the fact that the materials were light weight and would require less work internally and would be more economical to build.  The agent submitted details of the proposed hanging tiles in an email dated 16th March 2017.  These materials will be secured by condition however officers felt that as the proposal was to the rear of the property and the colour of the tiles would be similar to the existing property the different materials would be acceptable in this instance.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36: Biodiversity and Geology

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposed materials would not match the existing property, the colour of the hanging tiles will be similar to the existing brickwork and as the proposal is at the rear of the property having different materials in this instance is not considered harmful to the appearance of the property.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the agent has applied the 45 degrees design guidance in the Council’s SPG to both neighbouring properties at 26 and 30 Torrington Road.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO, the proposal meets general policy and guidance in terms of design and appearance.

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO, the proposal extends to the rear of the property and although the materials do not match the existing property it is not considered to detract from the appearance of the neighbouring properties.

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, the depth of the proposal and the distance from the boundaries with the neighbouring properties means the proposal is not considered to impact the neighbouring properties in terms of loss of light.  The proposal has also been designed to meet the 45 degrees design guidance in the Council’s SPG to both neighbouring properties which protects light to these properties.

- Visual dominance and overbearing effects of a development; NO, although the neighbouring properties at 26 and 30 Torrington Road will notice the additional height and depth of the proposal it is a sufficient distance from these neighbouring properties and as such the development is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials – state make and colour

- Approved plans

Informatives relating to:

- Terms and conditions

- Building Regulations

- Construction and demolition

- Encroachment and Party Wall

- Positive and proactive