DELEGATED APPLICATION REPORT 			Target Date:	06/02/2017

Application No: 162291/HOU

Address: 1 Chieveley Close, Tilehurst, Reading

Proposal: Rear conservatory

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property with a single storey side extension.  The property is joined at a right angle with the neighbouring property at 60 Hildens Drive and this property extends 1.2m deeper than the rear elevation of the application site.  The surrounding area is predominantly residential.

Proposal:

The application is for a rear conservatory that measures 3.75m in depth and 3.5m in width.  The proposal would have a hipped roof measuring 2.3m in height to eaves level and 3.2m in height at its highest point.  The proposal would have a brick base (to match the existing property) with glazing above.  The proposal would extend 2.57m deeper than the side elevation of 60 Hildens Drive.

Plans Considered:

The agent submitted the following drawings, received on 9th December 2016:

Location Plan

Block Plan

Existing & Proposed Ground Floor Plan And Elevations

Relevant Planning History:

None.

Consultation Responses:

Properties at 3 Chieveley Close and 60 and 62 Hildens Drive were consulted and no comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, the neighbouring property at 60 Hildens Drive will notice the additional height and depth of the conservatory but the proposal meets general policy and guidance and is not considered significantly harmful to the residents of these properties.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

Informatives relating to:

- Terms and conditions

2.	Building regulations

3.	Construction and demolition

4.	Encroachment and Party Wall

5.	Positive and proactive