DELEGATED APPLICATION REPORT 			Target Date:	29/12/2016

Application No: 161820/HOU

Address: 12 Grasmere Avenue, Tilehurst, Reading

Proposal: Single storey rear and first floor side extensions

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property with a single storey side/rear extension and a single storey attached store to the rear elevation.  The property is at a higher level than the rear garden and due to its position on the bend along Grasmere Avenue the property has an angled boundary.  The surrounding area is predominantly residential.

Proposal:

The application involves the removal of the existing rear store and the construction of a single storey rear extension and a first floor side extension.  The proposed single storey rear extension would attach to the existing single storey side/rear extension and measures 1.6m in depth and extend the full width of the property.  The proposal would have a mono pitch roof and measures 2.5m in height to eaves level and 3.4m in height at its highest point.

The proposed first floor side extension would extend above the existing side/rear extension and measures 1.9m in width and 2.8m in depth.  The proposal would be set back from the existing front elevation of the property by 4.6m and set down from the main roof by 1.6m.  The proposal extends up to the rear elevation of the existing property and would be a minimum of 0.8m from the boundary with the neighbouring property at 14 Grasmere Avenue and a maximum of 2m due the angled boundary.  12 and 14 Grasmere Avenue are closer at the front than they are at the rear.  A first floor side window is proposed and this will be conditioned to be obscure glazed and fixed up to 1.7m in height above internal floor level.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The applicant submitted the following drawings, received on 23rd September 2016:

Drawing No: MN/001 – Location and Site Plans

Drawing No: MN/002 – Existing Ground and First Floor Plans

Drawing No: MN/003 – Existing Rear and Side Elevations

Drawing No: MN/004 – Existing and Proposed Front Elevations

Drawing No: MN/005 – Proposed Ground and First Floor Plans

Drawing No: MN/006 – Proposed Rear and Side Elevations

Drawing No: MN/007 – Proposed Roof Plan, Sections A, A-A & B-B

Structural Calculations

Relevant Planning History:

None.

Consultation Responses:

Ecology raised no objection.

Properties at 10 and 14 Grasmere Avenue and 905 Oxford Road were consulted and no comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7: Design and the Public Realm

CS36: Biodiversity and Geology

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the first floor side extension does not have a front facing window it is a sufficient distance from the front elevation of the existing property and this is not considered to detract from the appearance of the existing property.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, although the property is at a higher level than the neighbouring property at 14 Grasmere Avenue it is sufficiently set down from the main roof and small in scale and is not considered to reduce the amount of light to this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, a condition will be imposed requiring the first floor side window to be obscure glazed and fixed up to 1.7m in height above internal floor level.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, the single storey rear extension is small in scale and although the neighbouring property at 14 Grasmere Avenue will notice the additional height and depth of the proposed first floor side extension there is a sufficient gap between the proposal and the neighbouring property.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

- First floor side window obscure and fixed

Informatives relating to:

- Terms and conditions

2.	Building regulations

3.	Construction and demolition

4.	Encroachment and Party Wall

5.	Positive and proactive