DELEGATED APPLICATION REPORT 				Target Date: 23/09/2015

Application No: 151333/HOU

Address: 140 Wantage Road, Reading

Proposal: Single storey rear extensions

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property with a two storey rear gable.  The property has a large outbuilding along the rear boundary which extends across the rear boundary of the attached property at 138 Wantage Road.  The surrounding area is predominately residential properties.

Proposal:

The application is for a single storey extension that extends along the side of the two storey gable and then wraps around and extends to the rear.  The proposed side element of the extension measures 6.7m in depth and the rear element of the extension measures 1.7m in depth.  The proposal has a hipped roof and measures 2.4m in height to eaves level and 3.4m in height at its highest point.  The proposal has 3 roof lights, bi-fold doors to the rear and a side window.

The submitted drawings also show a proposed porch however, the agent has advised this can be built under permitted development and it has not been considered under this application.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 27th July 2015:

Drawing No: P1510-01 Rev B – Location Plan & Block Plan

Drawing No: P1510-02 – Existing Plan & Elevations

Drawing No: P1510-03 – Proposed Plans And Elevations

The agent submitted the following amended plan, received on 21st September 2015:

Drawing No: P1510-01 Rev C – Location Plan & Block Plan

Relevant Planning History:

150994/HPA - Rear extensions measuring 6m and 2m in depth, with a maximum height of 4m, and 3m in height to eaves level. Notification of the construction of an extension under Class A, Part 1 of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015.  Prior Approval Not Required 14/07/2015.

Consultation Responses:

Properties at 138 and 142 Wantage Road and 81 and 83 Wilson Road were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The agent was advised on 21st September 2015 that as the previous prior approval development had not been implemented the proposed infill extension could not be considered on its own, and the whole development needed to be assessed.  The agent also advised that the applicant owned 138 Wantage Road (as the rear extension encroached over the boundary) and therefore the ownership certificate as submitted was correct.  The agent was asked to add a blue line around 138 Wantage Road on the location plan.  The agent submitted an amended location plan on 21st September 2015 and confirmed the description of development as amended was acceptable.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the proposal is 0.8m from the boundary with 142 Wantage Road and the single storey nature of the extension means the proposal will not significantly reduce the amount of light to this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, the neighbouring property at 142 Wantage Road will notice the additional height and depth of the proposal but there is sufficient distance between the side of the proposed extension and the side of this neighbouring property and the proposal is therefore not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials

- Approved plans

Informatives relating to:

- Terms and conditions

- Building Regulations

- Construction and demolition

- Encroachment and Party Wall

- Positive and proactive