DELEGATED APPLICATION REPORT 			          Target Date: 09/11/16

Application No: 161731/HOU

Address: 19 Hardwick Road

Proposal: Part one, part two storey rear extension

Case officer: Tom Hughes

Site Description and Surrounding Area

The application site is a two storey semi-detached property with a pitched roof. A conservatory extends from the rear. The boundary of the plot splays at the rear towards the adjacent neighbour at 32 Warnford Road. The surrounding area is predominantly residential.

Proposal

The proposal is for a part one, part two storey rear extension. The proposal would be facilitated by the demolition of the existing conservatory. At the ground floor, the proposal would extend across the full width of the property to a depth of 3m. The ground floor element would have a mono-pitch roof to a height of 3.4m, with a height to eaves of 2.1m. A window and bi-fold doors are proposed for the rear elevation.

Above the ground floor extension, set in by 2m from the respective side boundaries with the neighbours at 21 Hardwick Road and 32 Warnford Road, a first floor rear extension is proposed. The extension would project to a depth of 3m at a width of 3.7m. A window is proposed for the rear elevation. The first floor element would have a pitched roof at a height of 6m, with a height to eaves of 4.6m. The proposal would be set down from the ridgeline of the main roof by 0.7m.

The windows on the rear elevation at the first floor of the existing property will be reduced in size to accommodate the proposal. On the existing side elevation, a window is proposed at the ground floor. A high level window is proposed at the first floor. Materials have been selected to match those of the existing property.

Plans Considered

The agent submitted the following, received on 14th September 2016:

Site Location

Drawing No: GP/1624/A/R1 – Existing and Proposed Floor Plans and Elevations, Site Location and Block Plan

Relevant Planning History

090718 – Conversion of garage to habitable room. Single storey front extension to kitchen. Pitched roof to front elevation – Permitted 10/07/09

161642/CLP – Part 2 storey, part single storey rear extension – Withdrawn 14/09/16 –advised not CLP

Consultation Responses

Ecology were consulted. They had no objections to the proposal.

Properties at 17 and 21 Hardwick Road and 32 and 34 Warnford Road were consulted. No letters of representation were received.

Summary of discussions with applicant/agent

None.

Equality Act 2010

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; No, the proposal is sufficient in its design and proportion that it doesn’t detract from the original buildings appearance, despite posing a considerable alteration to the rear elevation. The proposal would meet general policy and guidance as the first floor element would be located 2m away from the respective side boundaries

- reduce the amount of usable amenity space for the property to an unacceptable degree; No

- require the loss of trees in the garden or put significant trees at risk; No

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; No, the proposal does not encroach on a line taken at 45 degrees from the nearest habitable window at 21 Hardwick Road, which protects the light to the rooms at that property. As such, the proposal is not considered to significantly reduce the amount of light to neighbouring properties

- invade the privacy of neighbouring properties through overlooking from windows or balconies; No, although a window is proposed for the side elevation of the original property it would be at a high level. Secured by condition, this window will be non-opening up to 1.7m in height above the internal floor level of the room in which it is located. In addition, the effect of overlooking from this window will be mitigated further by the positioning of a large tree close to the boundary with the application site at the rear of 32 Warnford Road. Therefore, the proposed window on the side elevation is not considered to be so harmful to the residents of the neighbouring properties as to be unacceptable

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; No, although the proposal would deviate from the general norm of the neighbouring properties, by setting away the proposal at the first floor from the boundary of the curtilage, this eases any concerns held about excessive bulk

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; No, materials have been selected to match those of the existing property

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; No, see comments above

- Access to sunlight and daylight; No, see comments above

- Visual dominance and overbearing effects of a development; No, although the neighbouring properties will notice the additional height and depth of the proposal, it is not considered to be visually dominant or overbearing

- Noise and disturbance; No

- Artificial lighting; No

- Vibration; No

- Dust and fumes; No

- Smell; No

- Crime and safety; No

RECOMMENDATION

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

- Windows

Informatives relating to:

- Terms and conditions

- Building regulations

- Demolition and construction

- Encroachment and Party Wall

- Positive and proactive